| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0203 |
| --- | --- |

| Title: | AHG17: Parsing HRD related SEI messages independently from the SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon (Panasonic) Karsten Sühring (HHI) | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com karsten.suehring@hhi.fraunhofer.de |
| Source: | Panasonic, Fraunhofer HHI |  |  |

_____________________________

# Abstract

It was decided at the JVET meeting in March 2019 that parsing of HRD related SEI messages should be possible independently from the active SPS in order to enable parsing of these SEI messages before the active SPS is known. After including the decoding unit related HRD syntax at the last meeting, this independent parsing was broken. This contribution suggests fixing this by repeating one syntax element in the buffering period SEI and moving another from the HRD parameters syntax structure to the buffering period SEI message.

# Introduction

It was decided at the JVET meeting in March 2019 that parsing of HRD related SEI messages (buffering period and picture timing SEI messages) should be possible independently from the SPS. Such a rule enables parsing of SEI messages as soon as they are received, even if the active SPS is not yet known. The authors of the present document support this decision.

Unfortunately, by adding decoding unit related syntax elements, this rule was broken, because two syntax elements signalled in the HRD parameters (which are signalled within the SPS) are now needed for parsing of the buffering period and the picture timing SEI message: decoding_unit_hrd_params_present_flag and decoding_unit_cbp_params_in_pic_timing_sei_flag. Furthermore, decoding_unit_cpb_params_in_pic_timing_sei_flag is also needed for parsing of the decoding unit information SEI message. While the decoding unit information SEI message was strictly speaking not discussed back in March, it is nevertheless also an HRD related SEI message. Therefore, the authors of the present document believe that it should also be parsed independently from the SPS.

This contribution suggests to solve this issue by repeating the first syntax element decoding_unit_hrd_params_present_flag within the buffering period, and moving the second syntax element decoding_unit_cpb_params_in_pic_timing_sei_flag from the HRD parameters to the buffering period SEI message. Indeed, decoding_unit_cpb_params_in_pic_timing_sei_flag is not used at all within the HRD parameters syntax structure, so its signalling there does not seem to be of any use.

With the two modifications suggested above, all three SEI messages (buffering period, picture timing and decoding unit information) could be parsed independently from the SPS. However, both picture timing and decoding unit information SEI messages would still depend on the buffering period SEI message for parsing. The authors of the present document believe this not to be a critical point, since the buffering period SEI message tends to be sent first in access units that contain one, or has already been sent previously for access units that do not contain any.

# Syntax and semantics of buffering period SEI message

It is suggested to modify the syntax of the buffering period SEI message as follows:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_nal_hrd_parameters_present_flag | u(1) |
| bp_vcl_hrd_parameters_present_flag | u(1) |
| if( bp_nal_hrd_parameters_present_flag ¦ ¦ bp_ vcl_hrd_parameters_present_flag ) { |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| bp_decoding_unit_hrd_params_present_flag | u(1) |
| if( bp_decoding_unit_hrd_params_present_flag ) { |  |
| du_cpb_removal_delay_increment_length_minus1 | u(5) |
| dpb_output_delay_du_length_minus1 | u(5) |
| decoding_unit_cbp_params_in_pic_timing_sei_flag | u(1) |
| } |  |
| } |  |
| concatenation_flag | u(1) |
| cpb_removal_delay_delta_minus1 | u(v) |
| cpb_removal_delay_deltas_present_flag | u(1) |
| if( cpb_removal_delay_deltas_present_flag ) { |  |
| num_cpb_removal_delay_deltas_minus1 | ue(v) |
| for( i = 0; i <= num_cpb_removal_delay_deltas_minus1; i++ ) |  |
| cpb_removal_delay_delta[ i ] | u(v) |
| } |  |
| bp_max_sub_layers_minus1 | u(3) |
| for( i = 0; i <= bp_max_sub_layers_minus1; i++ ) { |  |
| bp_cpb_cnt_minus1[ i ] | ue(v) |
| if( bp_nal_hrd_parameters_present_flag ) |  |
| for( j = 0; j < bp_cpb_cnt_minus1[ i ] + 1; j++ ) { |  |
| nal_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| nal_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag) { |  |
| nal_initial_alt_cpb_removal_delay[ i ][ j ] | u(v) |
| nal_initial_alt_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| } |  |
| if( bp_vcl_hrd_parameters_present_flag ) |  |
| for( j = 0; j < bp_cpb_cnt_minus1[ i ] + 1; j++ ) { |  |
| vcl_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| vcl_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag ) { |  |
| vcl_initial_alt_cpb_removal_delay[ i ][ j ] | u(v) |
| vcl_initial_alt_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

It is suggested to modify the semantics of the buffering period SEI message as follows:

bp_decoding_unit_hrd_params_present_flag equal to 1 specifies that decoding unit level syntax elements are present and the HRD may operate at access unit level or decoding unit level. bp_decoding_unit_hrd_params_present_flag equal to 0 specifies that decoding unit level syntax elements are not present and the HRD operates at access unit level. When bp_decoding_unit_hrd_params_present_flag is not present, its value is inferred to be equal to 0.

It is a requirement of bitstream conformance that the value of bp_decoding_unit_hrd_params_present_flag in the buffering period SEI message shall be equal to the value of decoding_unit_hrd_params_present_flag in the SPS.

du_cpb_removal_delay_increment_length_minus1 plus 1 specifies the length, in bits, of the du_cpb_removal_delay_increment_minus1[ i ] and du_common_cpb_removal_delay_increment_minus1 syntax elements of the picture timing SEI message and the du_spt_cpb_removal_delay_increment syntax element in the decoding unit information SEI message.

dpb_output_delay_du_length_minus1 plus 1 specifies the length, in bits, of the pic_dpb_output_du_delay syntax element in the picture timing SEI message and the pic_spt_dpb_output_du_delay syntax element in the decoding unit information SEI message.

decoding_unit_cpb_params_in_pic_timing_sei_flag equal to 1 specifies that decoding unit level CPB removal delay parameters are present in the picture timing SEI message and no decoding unit information SEI message is available (in the CVS or provided through external means not specified in this Specification). decoding_unit_cpb_params_in_pic_timing_sei_flag equal to 0 specifies that decoding unit level CPB removal delay parameters are present in decoding unit information SEI messages and the picture timing SEI message does not include decoding unit level CPB removal delay parameters. When the decoding_unit_cpb_params_in_pic_timing_sei_flag syntax element is not present, it is inferred to be equal to 0.

# Syntax of picture timing SEI message

It is suggested to modify the syntax of the picture timing SEI message as follows:

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| pt_max_sub_layers_minus1 | u(3) |
| cpb_removal_delay_minus1[ pt_max_sub_layers_minus1 ] | u(v) |
| for( i = TemporalId; i < pt_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| Else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag ) |  |
| pic_dpb_output_du_delay | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag && decoding_unit_cpb_params_in_pic_timing_sei_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| du_common_cpb_removal_delay_flag | u(1) |
| if( du_common_cpb_removal_delay_flag ) |  |
| du_common_cpb_removal_delay_increment_minus1 | u(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !du_common_cpb_removal_delay_flag && i < num_decoding_units_minus1 ) |  |
| du_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

# Semantics of decoding unit information SEI message

It is suggested to modify the semantics of the decoding unit information SEI message as follows:

The decoding unit information SEI message provides CPB removal delay information for the decoding unit associated with the SEI message.

The following applies for the decoding unit information SEI message syntax and semantics:

–	The syntax elements bp_decoding_unit_hrd_params_present_flag, decoding_unit_cpb_params_in_pic_timing_sei_flag and dpb_output_delay_du_length_minus1, and the variable CpbDpbDelaysPresentFlag are found in or derived from syntax elements in the general_hrd_parameters( ) syntax structure the buffering period SEI message that is applicable to at least one of the operation points to which the decoding unit information SEI message applies.

–	If bp_nal_hrd_parameters_present_flag or bp_vcl_hrd_parameters_present_flag of the buffering period SEI message applicable for the current access unit is equal to 1, the variable CpbDpbDelaysPresentFlag is set equal to 1. Otherwise, CpbDpbDelaysPresentFlag is set equal to 0.

–	The bitstream (or a part thereof) refers to the bitstream subset (or a part thereof) associated with any of the operation points to which the decoding unit information SEI message applies.

The presence of decoding unit information SEI messages for an operation point is specified as follows:

–	If CpbDpbDelaysPresentFlag is equal to 1, bp_decoding_unit_hrd_params_present_flag is equal to 1 and decoding_unit_cpb_params_in_pic_timing_sei_flag is equal to 0, one or more decoding unit information SEI messages applicable to the operation point shall be associated with each decoding unit in the CVS.

–	Otherwise, if CpbDpbDelaysPresentFlag is equal to 1, bp_decoding_unit_hrd_params_present_flag is equal to 1 and decoding_unit_cpb_params_in_pic_timing_sei_flag is equal to 1, one or more decoding unit information SEI messages applicable to the operation point may or may not be associated with each decoding unit in the CVS.

–	Otherwise (CpbDpbDelaysPresentFlag is equal to 0 or bp_decoding_unit_hrd_params_present_flag is equal to 0), in the CVS there shall be no decoding unit that is associated with a decoding unit information SEI message applicable to the operation point.

# Editorial modification

It is suggested to change the name of the syntax element decoding_unit_hrd_params_present_flag in the general_hrd_parameters syntax structure to general_decoding_unit_hrd_params_present_flag.

# Conclusion

It is suggested to make sure that parsing of the HRD related SEI messages (buffering period, picture timing and decoding unit information SEI messages) can be performed independently from the SPS.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).