| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0209 |
| --- | --- |

| Title: | Non-CE4: On the affine AMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the affine AMVR flag coding. It’s also proposed to extend affine AMVR to 4 precisions as in normal inter AMVP mode.

# Introduction

In the current VTM, the MVD of affine AMVP mode has 3 precisions: ¼ luma-sample, 1/16 luma-sample and 1 luma-sample.  If an affine coded CU has at least one non-zero MVD component, an amvr_flag is signaled to indicate whether ¼ luma-sample MVD precision or other MVD precision is used for current CU. If amvr_flag is true, the amvr_prececision_idx is signaled to indicate whether 1/16 luma-sample MVD precision or 1 luma-sample MVD precision is used for current CU.

In JVET O meeting, the AMVR in normal inter mode is extended to have 4 MVD precisions [1]: ¼ luma-sample, ½ luma-sample, 1 luma-sample and 4 luma-sample.

# Proposed

## Simplification of amvr_flag signaling

In this contribution, it’s proposed to simply the amvr_flag signaling in affine AMVR mode by only checking the MVDs of the first CPMV. The syntax table is modified as follows:

if( ( sps_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) ¦ ¦ ( sps_affine_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 1 && ( MvdCpL0[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ) ) {

amvr_flag[ x0 ][ y0 ]

if( amvr_flag[ x0 ][ y0 ] )

amvr_precision_idx[ x0 ][ y0 ]

}

The bd-rate performance of the proposed simplification is shown in Table 1.

Table 1 Test result of simplification of amvr_flag signalling

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | -0.02% | 101% | 100% |
| Class A2 | 0.04% | 0.03% | -0.01% | 103% | 103% |
| Class B | -0.01% | 0.07% | 0.02% | 100% | 100% |
| Class C | 0.00% | 0.11% | 0.05% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | 0.01% | 101% | 102% |
| Class D | -0.01% | -0.02% | 0.05% | 105% | 112% |
| Class F | 0.00% | 0.02% | -0.04% | 101% | 101% |

## Extension of affine AMVR

In this contribution, it’s proposed to extend affine AMVR to include ½ luma-sample precision. The specification of AmvrShift is modified as follows:

Specification of AmvrShift.

| amvr_flag | amvr_precision_idx | AmvrShift |  |  |
| --- | --- | --- | --- | --- |
|  |  | inter_affine_flag = =1 | CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) | inter_affine_flag = =0 && CuPredMode[ chType ][ x0 ][ y0 ] ! = MODE_IBC |
| 0 | - | 2 (1/4 luma sample) | - | 2 (1/4 luma sample) |
| 1 | 0 | 03 (1/162 luma sample) | 4 (1 luma sample) | 3 (1/2 luma sample) |
| 1 | 1 | 4 (1 luma sample) | 6 (4 luma samples) | 4 (1 luma sample) |
| 1 | 2 | 0 (1/16 luma sample） | - | 6 (4 luma samples) |

The bd-rate performance of the proposed simplification is shown in Table 2.

Table 2 Test result of affine AMVR extension

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | 0.00% | 102% | 101% |
| Class A2 | -0.06% | -0.05% | -0.07% | 105% | 101% |
| Class B | -0.01% | 0.05% | -0.09% | 102% | 101% |
| Class C | -0.05% | 0.02% | 0.04% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.00% | -0.03% | 103% | 102% |
| Class D | -0.05% | 0.12% | 0.01% | 106% | 112% |
| Class F | -0.04% | -0.07% | -0.08% | 103% | 101% |

The proposed method is also applied on top of solution 1 in JVET-P0090 [2] to limit the worst-case code length to 32 bits for coding the syntax element abs_mvd_minus2. Then the specification of AmvrShift is modified as follows:

Specification of AmvrShift.

| amvr_flag | amvr_precision_idx | AmvrShift |  |  |
| --- | --- | --- | --- | --- |
|  |  | inter_affine_flag = =1 | CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) | inter_affine_flag = =0 && CuPredMode[ chType ][ x0 ][ y0 ] ! = MODE_IBC |
| 0 | - | 2 (1/4 luma sample) | - | 2 (1/4 luma sample) |
| 1 | 0 | 03 (1/162 luma sample) | 4 (1 luma sample) | 3 (1/2 luma sample) |
| 1 | 1 | 4 (1 luma sample) | 6 (4 luma samples) | 4 (1 luma sample) |
| 1 | 2 | 1 (1/8 luma sample） | - | 6 (4 luma samples) |

The bd-rate performance of the proposed simplification is shown in Table 3.

Table 3 Test result of affine AMVR extension on top of JVET-P0090 solution 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.06% | -0.06% | 102% | 101% |
| Class A2 | -0.06% | 0.00% | -0.03% | 105% | 101% |
| Class B | -0.04% | 0.00% | -0.03% | 102% | 101% |
| Class C | -0.04% | 0.00% | -0.11% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.01% | -0.06% | 103% | 102% |
| Class D | -0.04% | 0.00% | 0.03% | 106% | 111% |
| Class F | -0.03% | -0.05% | -0.01% | 103% | 101% |

# References

- A. Henkel, I. Zupancic, B. Bross, M. Winken, H. Schwarz, D. Marpe, T. Wiegand, “CE4: Switchable interpolation filter”, JVET-O0057, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, July 2019.

- H. Huang, M. Coban, H. Wang, V. Seregin, W.-J. Chien, M. Karczewicz, “	Non-CE4: 32 bits coding for abs_mvd_minus2”, JVET-O0090, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Oct. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).