| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0218-v1 |
| --- | --- |

| Title: | AHG8/AHG17: On Access Unit Delimiter and Picture Detection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution includes three proposals for modifications related to Access Unit Delimiter (AUD) and picture boundary detection. The proposals are:

- A new single_slice_in_pic_flag flag in SPS and modification of the requirement of including AUD NAL units in all access units, so that it does not apply if single_slice_in_pic_flag is equal to 1.

- Optionally move the syntax elements which have the same value in all the slices of the picture to the access unit delimiter.

- Specification text for AUD NAL unit regarding nuh_layer_id and TemporalID.

# Introduction

At the 15th JVET meeting in Gothenburg it was decided to require AUD NAL units to be present in all access units. The motivation for this decision was to make it easy to detect in the bitstream where one picture ends and the next picture begins. When AUD NAL units are required to always be present in all access units, it is easy to determine if the next VCL NAL unit belongs to the same picture or to the next picture.

Having to signal AUD NAL units comes with a cost in terms of coding overhead, in particular for low resolutions and in particular when each picture is coded in a single slice – where signalling overhead for High-Level Syntax is otherwise minimized. For a Coded Video Sequence (CVS) where each picture is coded in a single slice, the problem of picture boundary detection becomes very simple – each new VCL NAL unit starts a new picture. In proposal 1, a new syntax element is proposed to indicate that all pictures in the CVS are coded with only one slice. It is further proposed that when the proposed syntax element is set equal to 1, it is not required to include AUD NAL units in each access unit.

In a different scenario, when pictures are coded with a large number of slices, the signalling overhead of the slice header can become significant. At the same time several syntax elements are required to have the same value in each slice of the picture. When an AUD is present in the bitstream, it can be used to hold information that applies to all slices of the picture. In Proposal 2, a new syntax element is proposed to indicate if pic_parameter_set_id, non_reference_picture_flag, colour_plane_id (when separate_colour_plane_flag equals 1), pic_order_count_lsb and pic_output_flag (when output_flag_presnet_flag equals 1) are present in the AUD or if these syntax elements are present in the slice header(s) of the picture.

In proposal 3 specification text is proposed for AUD NAL unit regarding nuh_layer_id and TemporalID.

# Proposal 1

The syntax element single_slice_in_pic_flag is proposed to be added to SPS. When it is set equal to one, all pictures that refer to the SPS, will each be coded in a single slice. Since it is not possible to encode multiple subpictures without also encoding multiple slices, it is also proposed that subpic_present_flag_depend is only signalled if single_slice_in_pic_flag equals 0.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| single_slice_in_pic_flag | u(1) |
| if( !single_slice_in_pic_flag) |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| } |  |

single_slice_in_pic_flag equal to 1 specifies that each picture that refers to this SPS includes exactly one slice. single_brick_per_slice_flag equal to 0 specifies that pictures that refer to this SPS may include more than one slice.

In the section of "7.4.2.4.3 Order of NAL units and coded pictures and their association to layer access units and access units" it is proposed to modify the requirement on when AUD NAL units need to present as follows:

If single_slice_in_pic_flag is equal to 1 in the SPS referred to by the pictures of the CVS then all access units in the CVS consists of zero or one access unit delimiter NAL unit and one or more layer access units in increasing order of nuh_layer_id.

Otherwise (single_slice_in_pic_flag is equal to 0), all access units in the CVS consists of one access unit delimiter NAL unit and one or more layer access units in increasing order of nuh_layer_id.

Further, when each picture consists of a single slice, there is no need to signal how many bricks are included in the slice since the total number of bricks in the picture is already known from the PPS and all those bricks will be included in that single slice.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !single_slice_in_pic_flag && !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| non_reference_picture_flag | u(1) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| slice_pic_order_cnt_lsb | u(v) |

For num_bricks_in_slice_minus1, the following addition is proposed:

num_bricks_in_slice_minus1, when present, specifies the number of bricks in the slice minus 1. The value of num_bricks_in_slice_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value ofnum_bricks_in_slice_minus1 is inferred to be equal to 0. When single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0. When single_slice_in_pic_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to NumBricksInPic – 1.

# Proposal 2

A new syntax structure called pic_header() is proposed. The syntax structure includes only syntax elements that are required to have the same value in all slices of a picture.

| pic_header( ) { | Descriptor |
| --- | --- |
| ph_pic_parameter_set_id | ue(v) |
| non_reference_picture_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| ph_pic_order_cnt_lsb | u(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| } |  |

When the pic_header() syntax structure is present in the access unit delimiter, the term current picture denotes the picture in the access unit with nuh_layer_id equal to vps_layer_id[ 0 ]. When the pic_header() syntax structure is present in the slice header, the term current picture denotes the picture that the current slice is a part of.

slice_ph_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use for the slices of the current picture. The value of slice_ph_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

It is a requirement of bitstream conformance that the value of TemporalId of the current picture shall be greater than or equal to the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to slice_pic_parameter_set_id.

non_reference_picture_flag equal to 1 specifies that the current picture containing the slice is never used as a reference picture. non_reference_picture_flag equal to 0 specifies that the current picture containing the slice may or may not be used as a reference picture.

colour_plane_id specifies the colour plane associated with the current slice RBSP picture when separate_colour_plane_flag is equal to 1. The value of colour_plane_id shall be in the range of 0 to 2, inclusive. colour_plane_id values 0, 1 and 2 correspond to the Y, Cb and Cr planes, respectively.

NOTE – : There is no dependency between the decoding processes of pictures having different values of colour_plane_id.

slice_ph_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_ph_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

pic_output_flag affects the decoded picture output and removal processes as specified in Annex C. When pic_output_flag is not present, it is inferred to be equal to 1.

The syntax element aud_pic_header_flag is proposed to be added to the AUD NAL unit syntax structure. When aud_pic_header_flag is equal to 1, the pic_header() syntax structure is present in the AUD NAL unit (and not in the slice headers of that picture).

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| aud_pic_header_flag | u(1) |
| if( aud_pic_header_flag ) |  |
| pic_header() |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The following semantic is proposed for aud_pic_header-flag.

aud_pic_header_flag equal to 1 specifies that the pic_header() syntax structure is present in the AUD. aud_pic_header_flag equal to 0 specifies that the pic_header() syntax structure is present in all slice headers in the access unit. When no AUD NAL unit is present in the access unit, the value of aud_pic_header_flag is inferred to be equal to 0.

The slice header is proposed to be modified as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| if( !aud_pic_header_flag ) |  |
| pic_header() |  |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| non_reference_picture_flag | u(1) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type = = GDR_NUT ) |  |
| recovery_poc_cnt | ue(v) |
| if( nal_unit_type = = IDR_W_RADL ¦ ¦ nal_unit_type = = IDR_N_LP ¦ ¦ nal_unit_type = = CRA_NUT ¦ ¦ NalUnitType = = GDR_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| … | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Proposal 3

It is proposed to make the following additions and modifications to section “7.4.2.4.3 Order of NAL units and coded pictures and their association to layer access units and access units”:

This clause specifies the order of NAL units and coded pictures and their association to layer access units and access units for CVSs that conform to one or more of the profiles specified in Annex A and that are decoded using the decoding process specified in clauses 2 through 10.

A layer access unit consists of one coded picture, zero or more VCL NAL units and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.4.

An access unit consists of an access unit delimiter NAL unit < what should be the temporal id and nuh-layer_id of the access unit> and one or more layer access units in increasing order of nuh_layer_id.

The Access unit delimiter NAL unit shall have nuh_layer_id value equal to vps_layer_id[ 0 ].

The Access Unit Delimiter NAL unit shall have TemporalId equal to the TemporalId of the access unit containing the NAL unit.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

Each acess unit starts with an access unit delimiter NAL unit. There shall be at most one access unit delimiter NAL unit in any layer access unit.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).