| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0222 |
| --- | --- |

| Title: | AHG17: Parameters in AUD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, we propose to add two syntax elements into access unit delimiter (AUD).

- rap_type, which specifies whether the coded pictures in the access unit contain non-IRAP NAL unit only, IRAP NAL unit only, or both IRAP and non-IRAP. The value of rap_type may be used for indicating whether a random access point is present in the access unit or not.

- au_order_cnt, which specifies the value of the access unit order count. The access unit order count value may be used for identifying the access unit boundary and recognizing the loss of one or more NAL units (VCL or AUD).

Introduction

At the last Gothenburg meeting, it was agreed to obligatorily signal the access unit delimiter (AUD) to indicate the access unit boundary. In AUD, a syntax element pic_type is signalled to indicate which slice_type values are present in the slices of the coded pictures in the access unit containing the access unit delimiter NAL unit. The pic_type may be useful to identify whether the AU is independent or dependent from other AUs.

In this contribution, we propose to add two syntax elements into access unit delimiter (AUD). It is asserted that those additional syntax elements are useful for indication of random access AU and robustness of AU boundary detection, respectively.

- rap_type, which specifies whether the coded pictures in the access unit contain non-IRAP NAL unit only, IRAP NAL unit only, or both IRAP and non-IRAP. The value of rap_type may be used for indicating whether a random access point is present in the access unit or not. Without parsing and interpreting NAL unit headers in NAL unit headers of VCL NAL units in all layers, the random access point of one or more layer (or sub-regions) can be early recognized by parsing rap_type in AUD. It is asserted that signaling rap_type is useful, especially when multiple NAL unit types are mixed in an AU (for example, one is IRAP, the other is non-IRAP), or when each layer has a different IRAP period in a multilayered bitstream.

- au_order_cnt, which specifies the value of the access unit order count. The access unit order count value may be used for identifying the access unit boundary, especially when one or more NAL units (VCL or AUD) are lost. In some applications, when some portion of VCL NAL units are dropped to adjust the bitrate or lost by packet error, the loss of VCL NAL units can be recognized by parsing au_order_cnt at a decoder.

Proposed Text

Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| rap_type | u(3) |
| au_order_cnt | u(8) |
| rbsp_trailing_bits( ) |  |
| } |  |

pic_type indicates that the slice_type values for all slices of the coded pictures in the access unit containing the access unit delimiter NAL unit are members of the set listed in Table 7-3 for the given value of pic_type. The value of pic_type shall be equal to 0, 1 or 2 in bitstreams conforming to this version of this Specification. Other values of pic_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved values of pic_type.

Table 7-3 – Interpretation of pic_type

| pic_type | slice_type values that may be present in the coded picture |
| --- | --- |
| 0 | I |
| 1 | P, I |
| 2 | B, P, I |

rap_type specifies that the VCL nuh_unit_type values for all slices of the coded pictures in the access unit containing the access unit delimiter NAL unit are members of the set listed in Table 7-4 for the given value of rap_type. The value of pic_type shall be equal to 0, 1 or 2 in bitstreams conforming to this version of this Specification. Other values of pic_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved values of pic_type.

Table 7-4 – Interpretation of rap_type

| rap_type | nuh_layer_id values that may be present in the coded picture |
| --- | --- |
| 0 | TRAIL_NUT, STSA_NUT, RASL_NUT, RADL_NUT |
| 1 | IDR_W_RADL, IDR_N_LP, CRA_NUT, GDR_NUT |
| 2 | All VCL nuh_unit_type values |

au_order_cnt specifies the value of the access unit order count that identifies the associated access unit among neighboring access units. The value of au_order_cnt shall be in the range of 0 to 255, inclusive.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).