| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0224 |
| --- | --- |

| Title: | AHG8/AHG12: Signaling subpicture identifier |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, signaling a subpic ID is proposed in NAL unit header or slice header. First, a flag use_nuh_layer_id_as_subpic_id_flag is signaled in SPS. When use_nuh_layer_id_as_subpic_id_flag is equal to 1, subpicID is set equal to nuh_layer_id signaled in NAL unit header. When use_nuh_layer_id_as_subpic_id_flag is equal to 0, subpicID is set equal to subpic_id, which may be signaled in slice header. When the current CVS consists of a single layer with multiple subpictures, use_nuh_layer_id_as_subpic_id_flag may be set equal to 1.

Introduction

Signaling subpicture IDs is useful for various applications including 360, point cloud, etc, which need a sub-picture based partitioning mechanism. Relying on parsing and interpreting the subpicture ID, which may be associated with specific region in a picture by mapping syntax elements in SPS or PPS, a decoder or a system element can recognize which VCL NAL units belongs to a target area associated with the selected subpicture ID values.

The issue is where the syntax element subpicture IDs needs to be signaled, while the mapping table associating the subpicture ID with specific regions could be present in a parameter set. Two most promising places could be NAL unit header and slice header. Both places have pros and cons, respectively. The NAL unit header is easily accessible by a system element, but expensive and pre-occupied by nuh_layer_id and nal_unit_type. The fixed length area at the early position of slice header may have more space, but the accessibility is worse than NAL unit header, because every time a system element should access the slice header in VCL payload.

In this contribution, a compromised solution is proposed. A flag use_nuh_layer_id_as_subpic_id_flag is signaled in SPS. When the value of use_nuh_layer_id_as_subpic_id_flag is equal to 1, the value of the variable subpicID is set equal to the value of nuh_layer_id signaled in NAL unit header. When the value of use_nuh_layer_id_as_subpic_id_flag is equal to 0, the value of the variable subpicID is set equal to subpic_id, which may be signaled in slice header.

When subpics_present_flag is equal to 1 and sps_video_parameter_set_id is equal to 0, which means the current CVS consists of a single layer with multiple subpictures, the value of use_nuh_layer_id_as_subpic_id_flag may be equal to 1. Otherwise, use_nuh_layer_id_as_subpic_id_flag shall be equal to 0.

It is asserted that the proposed method may improve the accessibility of subpic_id, when a picture is partitioned in subpictures in a single layer.

Proposed Text

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| subpics_present_flag | u(1) |
| use_nuh_layer_id_as_subpic_id_flag | u(1) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(53) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| … |  |
| } |  |

use_nuh_layer_id_as_subpic_id_flag equal to 1 specifies the value of the variable subpicID of each VCL NAL unit with nuh_layer_id in the CVS referring to the SPS is inferred to be equal to nuh_layer_id. use_nuh_layer_id_as_subpic_id_flag equal to 0 specifies the value of the variable subpicID of each VCL NAL unit with nuh_layer_id in the CVS referring to the SPS is inferred to be equal to subpic_id which is present in slice header. It is a requirement of bitstream conformance that, when sps_video_parameter_set_id is equal to 1 or subpics_present_flag is equal to 0, the value of use_nuh_layer_id_as_subpic_id_flag shall be equal to 0.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( subpics_present_flag && !use_nuh_layer_id_as_subpic_id_flag ) |  |
| subpic_id | u(v) |
| … |  |

subpic_id specifies the subpicture ID of the slice. The length of subpic_id is (subpic_id_len_minus1 + 1) bits.

Note : It is assumed that subpic_id_len_minus1is present in SPS or somewhere.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).