| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0226-v2 |
| --- | --- |

| Title: | AHG8: Output layer set with subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, the output layer set combined with output subpictures is proposed. Each output layer set indicates output layers and their subpictures to be outputted.

The proposed syntax elements include

- a flag indicates whether the subpicture partitions are aligned across layers or not,

- subpicture IDs present in each layer,

- output subpicture IDs present in each output layer of each output layer set,

- PTL information for each output layer set combined with output subpictures.

# Introduction

In this Geneva meeting, a couple of contributions including JVET-P0225 propose signaling output layer sets and PTL information. The output layer set with PTL information provides the operation points of a multi-layered bitstream. However, that information cannot define operation points combined with subpictures. Each subpicture in each layer may or may not be present in some applications. In the picture-in-picture (PIP) use case for E-sport broadcasting/streaming (introduced by JVET-O0335), for example, an ROI region can be enlarged and encoded with a high quality as a subpicture in an enhancement layer, while all area (all subpictures) in the base layer is encoded with a low quality. In an output mode, only the subpicture in the enhancement layer can be decoded and outputted, while all area of the base layer are decoded and outputted in another output mode. In a specific output mode, the output layer set can be composed of one or more subpictures from the base layer and one or more subpictures from enhancement layers, in order to show an enhanced visual quality at an ROI region (e.g. 360 viewport dependent processing).

Figure 1. Example of the enhanced coding of a local region (subpicture) for e-sport use case.

The proposed syntax elements include

- a flag indicates whether the subpicture partitions are aligned across layers or not,

- subpicture IDs present in each layer,

- output subpicture IDs present in each output layer of each output layer set,

- PTL information for each output layer set combined with output subpictures.

It is assumed that the syntax elements mapping a subpicture ID to a specific region of a picture are presented in a parameter set or elsewhere.

When a CVS consists of a single layer with multiple subpictures, the same syntax elements may be present in SPS to indicate the subpicture output set in a single layer.

# Proposed Text

In the proposed text, the yellow parts are identical to JVET-P0225. The green parts are the proposed syntax elements and semantics for the extension of output layer sets combined with subpicture.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_max_layers_minus1 | u(6) |
| num_output_layer_sets_minus1 | ue(v) |
| num_profile_tier_levels_minus1 | ue(v) |
| for( i = 0; i <= num_profile_tier_levels_minus1; i++ ) |  |
| profile_tier_level( vps_max_sub_layers_minus1 ) |  |
| vps_subpic_present_flag | u(1) |
| if(vps_subpic_present_flag) { |  |
| vps_subpic_aligned_across_layers_flag | u(1) |
| for(i = 0; i <= vps_subpic_aligned_across_layers_flag ? 0: vps_max_layers_minus1; i++ ) { |  |
| vps_max_subpics_minus1[i] | ue(v) |
| for(j = 0; j <= vps_max_subpics_minus1[i]; j++ ) { |  |
| vps_sub_pic_id_layer[i][j] | u(6) |
| } |  |
| for( i = 0; i <= num_output_layer_sets_minus1; i++ ) { |  |
| vps_output_layers_mode[ i ] | u(2) |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) { |  |
| if( vps_output_layers_mode[ i ] == 2 ) |  |
| vps_output_layer_flag[ i ][ j ] | u(1) |
| if(output_layer_flag[ i ][ j ]) { |  |
| all_subpic_output_flag[ i ][ j ] | u(1) |
| if( !all_subpic_output_flag[ i ][ j ] ) { |  |
| num_output_subpic_layer_minus1[ i ][ j ] | ue(v) |
| for( k = 0; k <= num_output_subpic_layer_minus1[ i ][ j ]; k++ ) |  |
| output_sub_pic_id_layer[ i ][ j ][ k ] | u(6) |
| } |  |
| profile_tier_level_idx[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

num_output_layer_sets_minus1 plus 1 specifies the number of output layer set in the coded vide sequence referring to the VPS. When not present, the value of num_output_layer_sets_minus1 is inferred to be equal to 0.

num_profile_tile_levels_minus1 plus 1 specifies the number of profile/tier/level information in the coded vide sequence referring to the VPS. When not present, the value of num_profile_tile_levels_minus1 is inferred to be equal to 0.

vps_subpics_present_flag equal to 1 specifies that the value of subpics_present_flag of one or more SPSs referring to this VPS is equal to 1. vps_subpics_present_flag equal to 0 specifies that the value of subpics_present_flag of any SPS referring to this VPS is equal to 0.

vps_subpic_aligned_across_layers_flag equal to 1 specifies that that vps_max_subpics_minus1[0] and vps_sub_pic_id_layer[0][j] are present in VPS and vps_max_subpics_minus1[i] is inferred to be equal to vps_max_subpics_minus1[0], and vps_sub_pic_id_layer[i][j] is inferred to be equal to vps_sub_pic_id_layer[0][j], when i is greater than 0. vps_subpic_aligned_across_layers_flag equal to 0 specifies that vps_max_subpics_minus1[i] and vps_sub_pic_id_layer[i][j] are present, when i is in the range of 0 to vps_max_layers_minus1, inclusive.

vps_max_subpics_minus1[i] plus 1 specifies the maximum number of subpictures of the i-th layer in the CVS referring to this VPS. It is a requirement of bitstream conformance that vps_max_subpics_minus1[i] shall be equal to max_subpics_minus1 of the SPS with nuh_layer_id equal to vps_layer_id[ i ].

vps_sub_pic_id_layer[i][j] specifies the subpicture ID of the j-th subpicture of the layer with nuh_layer_id equal to vps_layer_id[ i ]..

if(vps_subpic_aligned_across_layers_flag)
		for(i=1; i< vps_max_layers_minus1; i++) {
			vps_max_subpics_minus1[i] = vps_max_subpics_minus1[0] 
			for(j = 0; j <= vps_max_subpics_minus1[i]; j++ ) {
				vps_sub_pic_id_layer[i][j] = vps_sub_pic_id_layer[0][j]
			}
		}

vps_output_layers_mode[ i ] equal to 0 specifies that only the highest layer is output in the i-th output layer set. vps_output_layer_mode[ i ] equal to 1 specifies that all layers are output in the i-th output layer set. vps_output_layer_mode[ i ] equal to 2 specifies that the layers that are output are the layers with vps_output_layer_flag[ i ][ j ] equal to 1 in the i-th output layer set. The value of vps_output_layers_mode[ i ] shall be in the range of 0 to 2, inclusive. The value 3 of vps_output_layer_mode[ i ] is reserved for future use by ITU-T | ISO/IEC.

vps_output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer of the i-th output layer set is output. vps_output_layer_flag[ i ] [ j ] equal to 0 specifies that the j-th layer of the i-th output layer set is not output.

profile_tier_level_idx[ i ][ j ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the j-th layer of the i-th output layer set.

all_subpic_output_flag[ i ][ j ] equal to 1 specifies that all subpictures of the j-th layer of the i-th output layer set are outputted. all_subpic_output_flag[ i ][ j ] equal to 0 specifies that one or more subpictures of the j-th layer of the i-th output layer set may be outputted.

num_output_subpic_layer_minus1[ i ][ j ] specifies the number of output subpictures of the j-th layer of the i-th output layer set.

output_sub_pic_id_layer[ i ][ j ][ k ] specifies the subpicture ID of the k-th subpicture of the j-th layer of the i-th output layer set.

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, it is inferred to be equal to 0.

The variable DirectDependentLayerIdx[ i ][ j ], specifying the j-th direct dependent layer of the i-th layer, and the variable NumDirecDependentLayer[ i ], specifying the number of direct dependent layer of the i-th layer are derived as follows:

NumDirectDependentLayer[ 0 ] = 0
for( i = 1; i <= vps_max_layers_minus1; i++) {
	NumDirectDependentLayer[ i ] = 0
	if( !vps_independent_layer_flag[ i ] ) {
		for( j = i-1, k = 0; j  >=  0; j− − ) 		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] )
				DirectDependentLayerIdx[ i ][ k++ ] = j
		NumDirectDependentLayer[ i ] = k
	}
}

The variable GeneralLayerIdx[ i ], specifying the layer index of the layer with nuh_layer_id equal to vps_layer_id[ i ], is derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ )		(7-3)
	GeneralLayerIdx[ vps_layer_id[ i ] ] = i

The variables AllDependentLayerIdx[ i ][ j ], specifying the j-th direct or indirect dependent layer of the i-th layer, and the variable NumAllDependentLayer[ i ], specifying the number of direct or indirect dependent layer of the i-th layer are derived as follows:

NumAllDependentLayer [ 0 ] = 0
for( i = 1; i <= vps_max_layers_minus1; i++) {
	NumAllDependentLayer[ i ] = 0
	if( !vps_independent_layer_flag[ i ] ) {
		for( j = i-1, k = 0; j >= 0; j−− ) 		(7-4)
			if( vps_direct_dependency_flag[ i ][ j ] ) {
				AllDependentLayerIdx[ i ][ k++ ] = j
				NumAllDependentLayer[ i ] += 1
				for( m = NumAllDependentLayer[ j ]; m >= 0; m−− ) {
				  matched = 0
				  for( n = NumAllDependentLayer[ i ] - 1; n >= 0; n−− )
				    if(AllDependentLayerIdx[ j ][ m ] == AllDependentLayerIdx[ i ][ n])
					  matched = 1				     if( matched == 0 ) {
			       AllDependentLayerIdx[ i ][ k++ ] = AllDependentLayerIdx[ j ][ m ]
					NumAllDependentLayer[ i ] += 1
				  }
				}
			}
	}
}

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).