| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0230-v1 |
| --- | --- |

| Title: | CE8-1.2-related: Compound palette mode with signalled merge index |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Jizheng Xu, Li Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents a method of compound palette mode with signalled merge index. Different from the compound palette mode (CPM) in CE8-1.2, a merge index is signalled for each coding unit coded with CPM. The proposed method is implemented and evaluated with VTM6. The simulations are conducted with the CE8 test conditions. Simulation results are summarized as below:

CTC QP = {22, 27, 32, 37}:

Dual tree on:

TGM1080p: AI: -0.32%, 103%, 99%; RA: -0.41%, 104%, 99%; LDB: -0.27%, 104%, 99%

Dual tree off:

TGM1080p: AI: -1.31%, 103%, 100%; RA: -0.84%, 102%, 99%; LDB: -0.21%, 103%, 100%

Low QP = {2, 7, 12, 17}:

Dual tree on:

TGM1080p: AI: -0.72%, 102%, 99%; RA: -0.71%, 102%,  98%; LDB: -0.66%, 104%, 99%

Dual tree off:

TGM1080p: AI: -3.01%, 104%, 100%; RA: -1.95%, 102%, 100%; LDB: -1.58%, 102%, 100%

# Introduction

CE8-1.2 tested a compound palette mode (CPM) wherein the samples in the CPM can be reconstructed by copying the collocated sample in the IBC prediction when the current palette index is 0. Otherwise, the current sample will be reconstructed by the palette color. Figure 1 illustrates the proposed compound palette mode.

Figure 1 An illustration of the proposed compound palette mode.

In CPM, the IBC prediction is generated by using the first available merge candidate. To achieve that, the encoder and decode have to employ a same criterion to determine whether an IBC block vector is valid or not. As a result, the decoder shall implement a mechanism to tell if a certain BV is valid or not, which may not be desirable for all decoders.

# Proposed method

To deal with decoders without a BV validity check, we propose to send an index to the decoder to indicate which merge candidate is used in CPM. The encoder should guarantee that the BV associated with the signalled merge candidate shall always be valid. Thus, a decoder does not need to handle cases of invalid BV. The proposed method is same with the CPM in CE8-1.2 except for this point.

# Simulation results

The proposed method is evaluated with VTM6. The simulation follows the CE 8 test condition [1]. The results for CTC QPs are given in Table 1-6 and the results for low QPs are give in Table 7-12. More detailed results can be found in the attached spreadsheets.

Table 1. Coding performance of the proposed scheme under AI configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.32% | -0.06% | -0.06% | 103% | 99% |
| TGM 720p | 0.08% | -0.06% | -0.05% | 101% | 98% |
| Animation | 0.00% | 0.03% | 0.03% | 102% | 99% |
| Mixed content | 0.01% | -0.03% | -0.04% | 102% | 99% |
| Camera-Captured content | 0.00% | 0.06% | -0.01% | 103% | 100% |
| Overall | -0.06% | -0.02% | -0.03% | 102% | 99% |

Table 2. Coding performance of the proposed scheme under RA configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.41% | -0.28% | -0.22% | 104% | 99% |
| TGM 720p | 0.00% | -0.22% | -0.10% | 103% | 100% |
| Animation | 0.06% | 0.06% | 0.06% | 102% | 100% |
| Mixed content | -0.06% | -0.28% | -0.28% | 102% | 100% |
| Camera-Captured content | 0.00% | 0.09% | 0.10% | 101% | 100% |
| Overall | -0.10% | -0.16% | -0.11% | 103% | 100% |

Table 3. Coding performance of the proposed scheme under LDB configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.27% | -0.43% | -0.26% | 104% | 99% |
| TGM 720p | 0.21% | 0.15% | 0.16% | 101% | 100% |
| Animation | -0.04% | 0.11% | -0.07% | 101% | 99% |
| Mixed content | -0.24% | 0.21% | -0.08% | 104% | 98% |
| Camera-Captured content | -0.02% | -0.03% | -0.12% | 104% | 99% |
| Overall | -0.07% | -0.01% | -0.07% | 103% | 99% |

Table 4. Coding performance of the proposed scheme under AI configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.31% | -1.51% | -1.67% | 103% | 100% |
| TGM 720p | -0.10% | -0.49% | -0.69% | 101% | 97% |
| Animation | 0.02% | 0.01% | 0.08% | 103% | 100% |
| Mixed content | -0.21% | -1.02% | -0.88% | 105% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | -0.39% | -0.69% | -0.74% | 103% | 99% |

Table 5. Coding performance of the proposed scheme under RA configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.84% | -0.91% | -0.68% | 102% | 99% |
| TGM 720p | -0.38% | -0.85% | -0.71% | 103% | 99% |
| Animation | -0.01% | -0.07% | 0.01% | 102% | 99% |
| Mixed content | -0.29% | -0.27% | -0.52% | 103% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.01% | 101% | 98% |
| Overall | -0.36% | -0.50% | -0.44% | 102% | 99% |

Table 6. Coding performance of the proposed scheme under LDB configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.21% | -0.36% | -0.43% | 103% | 100% |
| TGM 720p | -0.08% | -0.33% | -0.30% | 103% | 98% |
| Animation | -0.05% | 0.01% | 0.05% | 102% | 98% |
| Mixed content | -0.16% | 0.02% | -0.55% | 103% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Overall | -0.11% | -0.17% | -0.27% | 103% | 98% |

Table 7. Coding performance of the proposed scheme under AI configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.72% | -0.75% | -0.76% | 102% | 99% |
| TGM 720p | -0.11% | -0.15% | -0.12% | 101% | 99% |
| Animation | -0.04% | 0.01% | -0.06% | 100% | 98% |
| Mixed content | -0.04% | -0.04% | -0.06% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.22% | -0.23% | -0.24% | 101% | 99% |

Table 8. Coding performance of the proposed scheme under RA configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.71% | -0.96% | -0.92% | 102% | 98% |
| TGM 720p | 0.07% | -0.13% | -0.24% | 100% | 98% |
| Animation | 0.01% | -0.11% | -0.01% | 100% | 100% |
| Mixed content | -0.01% | -0.06% | -0.12% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.16% | -0.30% | -0.31% | 101% | 99% |

Table 9. Coding performance of the proposed scheme under LDB configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.66% | -0.73% | -0.77% | 104% | 99% |
| TGM 720p | -0.16% | -0.29% | -0.19% | 101% | 99% |
| Animation | -0.25% | 0.06% | 0.05% | 100% | 100% |
| Mixed content | -0.12% | -0.30% | -0.02% | 101% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | -0.27% | -0.30% | -0.23% | 102% | 99% |

Table 10. Coding performance of the proposed scheme under AI configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -3.01% | -2.99% | -3.03% | 104% | 100% |
| TGM 720p | -0.75% | -0.75% | -0.78% | 102% | 100% |
| Animation | -0.13% | -0.20% | -0.16% | 100% | 100% |
| Mixed content | -0.93% | -0.90% | -1.03% | 102% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | -1.14% | -1.14% | -1.18% | 102% | 100% |

Table 11. Coding performance of the proposed scheme under RA configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.95% | -1.95% | -1.95% | 102% | 100% |
| TGM 720p | -0.47% | -0.51% | -0.51% | 101% | 100% |
| Animation | -0.08% | -0.07% | -0.10% | 99% | 99% |
| Mixed content | -0.44% | -0.38% | -0.54% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | -0.70% | -0.70% | -0.73% | 101% | 100% |

Table 12. Coding performance of the proposed scheme under LDB configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.58% | -1.62% | -1.62% | 102% | 100% |
| TGM 720p | -0.21% | -0.23% | -0.11% | 101% | 100% |
| Animation | -0.41% | -0.01% | -0.04% | 100% | 100% |
| Mixed content | -0.34% | -0.41% | -0.42% | 99% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | -0.59% | -0.54% | -0.52% | 100% | 99% |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| … |  |
| if( ( ( ( slice_type = = I ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ sps_ibc_enabled_flag ) && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ¦ ¦ ( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && !sps_ibc_enabled_flag && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && && cu_skip_flag[ x0 ][ y0 ] = = 0 &&modeType != MODE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) { |  |
| if(cbWidth < 64 && cbHeight < 64 ) |  |
| plt_ibc_flag | ae(v) |
| if(plt_ibc_flag) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( treeType = = DUAL_TREE_LUMA ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 1 ) |  |
| else /* SINGLE_TREE */ |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 3 ) |  |
| } else { |  |
| … |  |
| } |  |
| … |  |
| } |  |

plt_ibc_flag specifies the use of both IBC prediction and palette mode in the current coding unit. plt_ibc_flag equal to 1 indicates that compound palette mode is applied in the current coding unit. plt_ibc_flag equal to 0 indicates that compound palette mode is not applied in the current coding unit. When plt_ibc_flag is not present, it is inferred to be equal to 0.

palette_escape_val_present_flag equal to 1 specifies that the current coding unit contains at least one escape coded sample. escape_val_present_flag equal to 0 specifies that there are no escape coded samples in the current coding unit. When not present, the value of palette_escape_val_present_flag is inferred to be equal to 1.

The variable MaxPaletteIndex specifies the maximum possible value for a palette index for the current coding unit. The value of MaxPaletteIndex is set equal to CurrentPaletteSize[ startComp ] − 1 + palette_escape_val_present_flag + (plt_ibc_flag ? 1 : 0) .

#### 8.4.5.3 Decoding process for palette mode

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable startComp specifies the first colour component in the palette table,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is equal to 0, nSubWidth is set to 1 and nSubHeight is set to 1.

- Otherwise, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

If plt_ibc_flag is equal to 1 the following applies:

- general_merge_flag[ xCb ][ yCb ] is set equal to TRUE.

  - The derivation process for block vector components as specified in clause 8.6.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the partitioning treeType as inputs, and the reconstructed picture before in-loop filtering predSamples as output..

- The ( nCbW x nCbH ) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is represented by recSamples[ x ][ y ] with x = 0..nCTbW − 1 and y = 0..nCbH − 1, and the value of recSamples[ x ][ y ] for each x in the range of 0 to nCbW − 1, inclusive, and each y in the range of 0 to nCbH − 1, inclusive, is derived as follows:The variables xL and yL are derived as follows:

xL = palette_transpose_flag ?  x * nSubHeight :  x * nSubWidth	(8-268)

yL = palette_transpose_flag ?  y * nSubWidth :  y * nSubHeight	(8-269)

- The variable bIsEscapeSample is derived as follows:

- If PaletteIndexMap[ xCb + xL ][ yCb + yL ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, bIsEscapeSample is set equal to 1.

- Otherwise, bIsEscapeSample is set equal to 0.

- If bIsEscapeSample is equal to 0, the following applies:

  - If plt_ibc_flag is 1, the following applies:

resamples[ x ][ y ] = PaletteIndexMap[ xCb + xL ][ yCb + yL ] == 0 ? predSamples[ x ][ y ]: CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ]  - 1]

  - Otherwise, the following applies:

recSamples[ x ][ y ] =   CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ] ]		 (8-270)

…

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to PaletteMaxPredictorSize, inclusive.

8.6.2.2 Derivation process for IBC luma block vector prediction

This process is only invoked when CuPredMode[ 0 ][ xCb ][ yCb ] is equal to MODE_IBC or plt_ibc_flag is 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

…

8.8.3.5 Derivation process of boundary filtering strength

Inputs to this process are:

- a picture sample array recPicture,

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags.

Output of this process is a two-dimensional (nCbW)x(nCbH) array bS specifying the boundary filtering strength.

…

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

…

- The variable bS[ xDi ][ yDj ] is derived as follows:

…

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode or their plt_ibc_flag are both equal to 1, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

…

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

6.4.4 Derivation process for neighbouring block availability

Inputs to this process are:

- the luma location ( xCurr, yCurr ) of the top-left sample of the current block relative to the top-left luma sample of the current picture,

- the luma location ( xNbY, yNbY ) covered by a neighbouring block relative to the top-left luma sample of the current picture,

- the variable checkPredModeY specifying whether availability depends on the prediction mode.

- the variable cIdx specifying the colour component of the current block.

Output of this process is the availability of the neighbouring block covering the location ( xNbY, yNbY ), denoted as availableN.

…

When all of the following conditions are true, availableN is set equal to FALSE.

–	checkPredModeY is equal to TRUE.

–	availableN is set equal to TRUE.

–	CuPredMode[ 0][ xNbY ][ yNbY ] is not equal to CuPredMode[ 0][ xCurr ][ yCurr ]

–  All of the following conditions are false

–   CuPredMode[ 0][ xNbY ][ yNbY ] is IBC, CuPredMode[0][ xCurr ][ yCurr ] is MODE_PLT and plt_ibc_flag[ xCurr ][ yCurr ] is 1.

–	CuPredMode[0][ xNbY ][ yNbY ] is MODE_PLT, CuPredMode[0][ xCurr ][ yCurr ] is MODE_PLT and plt_ibc_flag[ xNbY ][ yNbY ] is 1.

–	CuPredMode[ 0][ xNbY ][ yNbY ] is equal to CuPredMode[0][ xCurr ][ yCurr ].

# Conclusion

This document proposes to signal the merge index in the compound palette mode to alleviate potential burden on decoder to check BV validity while maintain the coding efficiency. We recommend adopting the proposed method to the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).