| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0232 |
| --- | --- |

| Title: | AHG12: On rectangular slice addressing information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee (XRis Corp) Dongsan Jun (Kyungnam University) | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr dsjun9643@kyungnam.ac.kr |
| Source: | XRis Corporation, Kyungnam University |  |  |

_____________________________

- Abstract

This contribution proposes to conditionally signal brick_idx_delta_sign_flag[i] based on the locations of bottom-right brick of previous slice (i-1).

Introduction

The shapes of the slices of a picture shall be such that each tile(brick) shall have its entire left boundary and entire top boundary of previously decoded tile(brick). For example, Figure 1 (a) and (b) is allowed, but Figure 1 (c) is disallowed.

When the brick of the bottom right corner of (i-1)-th slice is located in the last tile column in a picture, the delta between the brick index of bottom right corner of i-th slice (BottomRightBrickIdx[i]) and the brick index of bottom right corner of (i-1)-th slice (BottomRightBrickIdx[i-1]) has always greater than 0. In that case, brick_idx_delta_sign_flag[i] could be inferred.

.

Figure 1

Following is proposed

- It is proposed to infer brick_idx_delta_sign_flag[i], when the brick of bottom right corner of (i-1)-th slice is located in the last tile column.

Proposal

If BottomRightBrickIdx[i-1] is indicated in the last tile column, the value of BottomRightBrickIdx[i] has always greater than that of BottomRightBrickIdx[i-1]. So, brick_idx_delta_sign_flag[i] could be inferred to be equal to 1.

The proposed syntax and semantic changes are shown highlighted below compared with VVC draft6 [1]

Table 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| bottom_right_brick_idx_length_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| if (i > 0 ¦¦ isLastTileColumn[BottomRightBrickIdx[i-1]] == 0) |  |
| brick_idx_delta_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

- 6.5.1 CTB raster scanning, tile scanning, brick scanning, and subpicture scanning processes

for ( brickIdx = 0, i = 0; i < NumTilesInPic; i++ ) {
	tileX = i % ( num_tile_columns_minus1 + 1 )
	tileY = i / ( num_tile_columns_minus1 + 1 )

isLastTileColumn[brickIdx ] = 0

if( !brick_split_flag[ i ] ) {
		BrickColBd[ brickIdx ] = tileColBd[ tileX ]
		BrickRowBd[ brickIdx ] = tileRowBd[ tileY ]
		BrickWidth[ brickIdx ] = colWidth[ tileX ]
		BrickHeight[ brickIdx ] = RowHeight[ tileY ]
		if (tileX == num_tile_column_minus1) 
			isLastTileColumn[brickIdx ] = 1 	
		brickIdx++
	} else {
		if( uniform_brick_spacing_flag[ i ] ) {
			remainingHeightInCtbsY = RowHeight[ tileY ]
			j = 0
			while( remainingHeightInCtbsY > ( brick_height_minus1[ i ] + 1 ) ) {
				rowHeight2[ j++ ] = brick_height_minus1[ i ] + 1
				remainingHeightInCtbsY  −=  ( brick_height_minus1[ i ] + 1 )
			}
			rowHeight2[ j ] = remainingHeightInCtbsY
			num_brick_rows_minus2[ i ] = j − 1
		} else {
			rowHeight2[ num_brick_rows_minus2[ i ] + 1 ] = RowHeight[ tileY ]
			for( j = 0; j  <=  num_brick_rows_minus2[ i ]; j++ ) {
				rowHeight2[ j ] = brick_row_height_minus1[ i ][ j ]+ 1
				rowHeight2[ num_brick_rows_minus2[ i ] + 1 ]  −=  rowHeight2[ j ]
			}
		}
		for( rowBd2[ 0 ] = 0, j = 0; j  <=  num_brick_rows_minus2[ i ] + 1; j++ )
			rowBd2[ j + 1 ] = rowBd2[ j ] + rowHeight2[ j ]
		for( j = 0; j  <=  num_brick_rows_minus2[ i ] + 1; j++ ) {
			BrickColBd[ brickIdx ] = tileColBd[ tileX ]
			BrickRowBd[ brickIdx ] = tileRowBd[ tileY ] + rowBd2[ j ]
			BrickWidth[ brickIdx ] = colWidth[ tileX ]
			BrickHeight[ brickIdx ] = rowHeight2[ j ]
			if (tileX == num_tile_column_minus1) 
					isLastTileColumn[brickIdx ] = 1 
			brickIdx++
		}
	}
}
NumBricksInPic = brickIdx

7.4.3.4 Picture parameter set RBSP semantics

brick_idx_delta_sign_flag[ i ] equal to 1 indicates a positive sign for bottom_right_brick_idx_delta[ i ]. sign_bottom_right_brick_idx_delta[ i ] equal to 0 indicates a negative sign for bottom_right_brick_idx_delta[ i ]. When not presented, brick_idx_delta_sign_flag is inferred to be equal to 1

References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019

Patent rights declaration(s)

XRis Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kyungnam University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).