| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0236-v2 |
| --- | --- |

| Title: | Non-CE4: On a simplification for triangle merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The triangle merge mode is a method that generates predicted samples by dividing a block to two non-rectangular areas and averaging boundary samples. In the current WD6, the relationship between the triangle merge mode and the explicit weighted prediction has not been clarified. In this contribution, the explicit weighted prediction can be selected in both the triangle merge mode and the other merge tools. As an experimental result on CTC, the coding efficiency and the encoding time were not changed, and the average of encoding times became 1% faster on LB configuration. As an experimental result on fade sequences by using the explicit weighted prediction, the average gain was 0.42%.

Problem Statement

In the current Working Draft 6 [1], the relationship between the triangle merge mode weighted prediction process and the explicit weighted prediction has not been clarified. In the most of inter tools, the explicit weighted prediction can be selected by the reference picture index. Several new weighted prediction tools, for example, DMVR, BDOF and BCW are prohibited when the coefficients and offsets for the explicit weighted prediction are existed on the reference picture. However, according to ticket #367 for VTM-6.0 software [2], when triangle merge mode is selected, the explicit weighted prediction is prohibited regardless the reference picture index. As a result, the design of triangle merge mode has been different from that of the other tools. In both the triangle merge mode and the other merge tools, the explicit weighted prediction should be selected by the reference picture index.

Proposed method

On VTM-6.0 software, according to ticket #367, when the triangle merge mode is selected, the explicit weighted prediction is prohibited regardless the reference picture index as shown in Figure 1.

Figure 1	Implementation on VTM-6.0.

In this contribution, we propose the weighted prediction process for triangle merge is specified after the weighted prediction process for interpolation samples as shown in Figure 2. Therefore, in both the triangle merge mode and the other merge tools, the explicit weighted prediction can be selected by the reference picture index. The changes are the order of right-shifting, clipping and weighting. There is no additional complexity on the MC stage.

Figure 2	Proposed method

Proposal changes of specification

The working draft 6 [1] should be changed by the followings.

- 8.5.7	Decoding process for triangle inter blocks

- 8.5.7.1	General

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the chroma motion vectors mvCA and mvCB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

Outputs of this process are:

- an (cbWidth)x(cbHeight) array predSamplesL of luma prediction samples,

- an (cbWidth / SubWidthC)x(cbHeight / SubHeightC) array predSamplesCb of chroma prediction samples for the component Cb,

- an (cbWidth / SubWidthC)x(cbHeight / SubHeightC) array predSamplesCr of chroma prediction samples for the component Cr.

Let predSamplesLAL and predSamplesLBL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesLACb, predSamplesLBCb, predSamplesLACr and predSamplesLBCr be (cbWidth / SubWidthC)x(cbHeight / SubHeightC) arrays of predicted chroma sample values.

The predSamplesL, predSamplesCb and predSamplesCr are derived by the following ordered steps:

- For N being each of A and B, the following applies:

- The reference picture consisting of an ordered two-dimensional array refPicLNL of luma samples and two ordered two-dimensional arrays refPicLNCb and refPicLNCr of chroma samples is derived by invoking the process specified in clause 8.5.6.2 and 8.5.6.6 with X set equal to predListFlagN and refIdxX set equal to refIdxN as input.

- The array predSamplesLNL is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 and the weighted sample prediction process specified in clause 8.5.6.6 with the luma location ( xCb, yCb ), the luma coding block width sbWidth set equal to cbWidth, the luma coding block height sbHeight set equal to cbHeight, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvN and the reference array refPicLXL set equal to refPicLNL, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 0 as inputs.

- The array predSamplesLNCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 and the weighted sample prediction process specified in clause 8.5.6.6 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / SubWidthC, the coding block height sbHeight set equal to cbHeight / SubHeightC, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCb set equal to refPicLNCb, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 1 as inputs.

- The array predSamplesLNCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 and the weighted sample prediction process specified in clause 8.5.6.6 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / SubWidthC, the coding block height sbHeight set equal to cbHeight / SubHeightC, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCr set equal to refPicLNCr, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 2 as inputs.

- The partition direction of merge triangle mode variable triangleDir is set equal to merge_triangle_split_dir[ xCb ][ yCb ].

- The prediction samples inside the current luma coding block, predSamplesL[ xL ][ yL ] with xL = 0..cbWidth − 1 and yL = 0..cbHeight − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth, the coding block height nCbH set equal to cbHeight, the sample arrays predSamplesLAL and predSamplesLBL, and the variables triangleDir, and cIdx equal to 0 as inputs.

- The prediction samples inside the current chroma component Cb coding block, predSamplesCb[ xC ][ yC ] with xC = 0..cbWidth / SubWidthC − 1 and yC = 0..cbHeight / SubHeightC − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth / SubWidthC, the coding block height nCbH set equal to cbHeight / SubHeightC, the sample arrays predSamplesLACb and predSamplesLBCb, and the variables triangleDir, and cIdx equal to 1 as inputs.

- The prediction samples inside the current chroma component Cr coding block, predSamplesCr[ xC ][ yC ] with xC = 0..cbWidth / SubWidthC − 1 and yC = 0..cbHeight / SubHeightC − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth / SubWidthC, the coding block height nCbH set equal to cbHeight / SubHeightC, the sample arrays predSamplesLACr and predSamplesLBCr, and the variables triangleDir, and cIdx equal to 2 as inputs.

- The motion vector storing process for merge triangle mode specified in clause 8.5.7.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, the partition direction triangleDir, the luma motion vectors mvA and mvB, the reference indices refIdxA and refIdxB, and the prediction list flags predListFlagA and predListFlagB as inputs.

- 8.5.7.2	Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-841)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-842)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-843)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-844)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-845)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-846)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(8-847)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + 4offset1 ) >> 3shift1 )

Experimental result

Common Test Condition

The proposed method has been implemented on VTM-6.0 software. The simulation was based on VTM configuration JVET-O2010 [2]. The PC cluster was used for the simulation, however since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As summary of experimental results, there is no loss of the coding efficiency and the encoding time on average, and the average of encoding time became 1% faster on LB configuration.

Table 1  Summary of experimental results on RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.05% | 100% | 100% |
| Class A2 | 0.02% | 0.04% | 0.02% | 100% | 100% |
| Class B | -0.01% | 0.06% | 0.09% | 100% | 100% |
| Class C | -0.03% | -0.01% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.05% | 100% | 100% |
| Class D | -0.03% | -0.03% | 0.06% | 99% | 100% |
| Class F | 0.00% | 0.02% | -0.02% | 100% | 100% |

Table 2  Summary of experimental results on LB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.10% | 0.11% | 100% | 100% |
| Class C | 0.00% | -0.10% | 0.09% | 99% | 99% |
| Class E | 0.04% | 0.15% | 0.21% | 99% | 100% |
| Overall | 0.01% | 0.05% | 0.13% | 99% | 100% |
| Class D | -0.04% | 0.03% | 0.14% | 100% | 100% |
| Class F | 0.00% | -0.14% | 0.68% | 99% | 99% |

Fade sequences

Fade sequences are generated with a fading tool in JVET-L0201-v1 [2]. In our experiments, a linear fade is applied to the about first 2 seconds of the regular sequence set, and the experiments are conducted with them. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs. The number of fade sequence frames is decided by following Table 1.

Table 3	The number of fade sequence frames against frame rate

| Frame Rate | The number of fade sequence frames |
| --- | --- |
| 20 | 33 |
| 30 | 65 |
| 50 | 97 |
| 60 | 129 |

The proposed method has been implemented on VTM-6.0 software, however the define statement JVET_O0244_DELTA_POC is set to 0. The simulation was based on VTM configuration JVET-L1010 [3] with above black and white fade sequences. Regarding WP parameters on VTM-6.0, we set the following parameters.

--WeightedPredMethod=0 --WeightedPredB=1 --WeightedPredP=1

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

Table 4  Summary of experimental results for fade sequences

|  |  |  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| RA10 | Black fade sequences | WP | -6.60% | -10.37% | -9.97% | 127% | 87% |
|  |  | Proposed | -7.15% | -11.21% | -10.77% | 124% | 91% |
|  | White fade sequences | WP | -8.40% | -12.25% | -12.23% | 125% | 86% |
|  |  | Proposed | -9.08% | -12.88% | -12.99% | 122% | 90% |
| LB10 | Black fade sequences | WP | -28.11% | -36.10% | -35.27% | 89% | 88% |
|  |  | Proposed | -28.25% | -36.24% | -35.45% | 89% | 86% |
|  | White fade sequences | WP | -29.67% | -36.56% | -36.18% | 83% | 86% |
|  |  | Proposed | -29.79% | -36.92% | -36.25% | 82% | 84% |

Table 5  Summary of experimental results (compared to WP)

|  |  | Over VTM-6.0_WPM0+WPB+WPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |  |
| Proposed (RA) | Black fade sequences | -0.60% | -0.92% | -0.88% | 98% | 104% |
|  | White fade sequences | -0.74% | -0.71% | -0.87% | 98% | 104% |
| Proposed (LB) | Black fade sequences | -0.17% | -0.23% | -0.22% | 100% | 99% |
|  | White fade sequences | -0.17% | -0.54% | -0.05% | 100% | 98% |
|  |  | -0.42% | -0.60% | -0.51% | 99% | 101% |

Conclusion

In this contribution, we proposed that the weighted prediction process for triangle merge is specified after the weighted prediction process for interpolation samples. As a result, the specification of triangle merge mode is clarified without major changes. Finally, we would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 6), Joint Video Experts Team Document, JVET-O2001-vE, Gothenburg, July 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-O2010, Gothenburg, July 2019.

[3]	P. Bordes, E. François and F. L. Léannec, AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences, Joint Video Exploration Team Document, JVET-L0201, Macao, Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).