| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0243-v1 |
| --- | --- |

| Title: | AHG17: Unequal bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan, Tim Hellman, Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The current VVC draft text allows the bit depth of the luma samples and the chroma samples to be coded independently, enabling more combinations than if the bit depths were constrained to be identical.  This contribution claims that the cost of this flexibility, specifically the additional burden to test and verify these combinations, exceeds the practical benefit seen in product use so far as well as anticipated usage in the future.  It is proposed to code a single bit depth to be used for both luma and chroma. This would significantly reduce the number of combinations while still maintaining some flexibility at the encoder to adjust bit depths, if it deems necessary.

# Background

The current draft text [1] specifies the bit depth of the samples of the luma array and the chroma array independently in the SPS using the bit_depth_luma_minus8 and bit_depth_chroma_minus8 syntax elements:

bit_depth_luma_minus8 specifies the bit depth of the samples of the luma array BitDepthY and the value of the luma quantization parameter range offset QpBdOffsetY as follows:

BitDepthY      = 8 + bit_depth_luma_minus8		(7-8)

QpBdOffsetY = 6 * bit_depth_luma_minus8		(7-9)

bit_depth_luma_minus8 shall be in the range of 0 to 8, inclusive.

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and the value of the chroma quantization parameter range offset QpBdOffsetC as follows:

BitDepthC      = 8 + bit_depth_chroma_minus8		(7-10)

QpBdOffsetC = 6 * bit_depth_chroma_minus8		(7-11)

bit_depth_chroma_minus8 shall be in the range of 0 to 8, inclusive.

# Discussion

The ability to separately specify luma and chroma bit depths results in N2 possible combinations, relative to the N combinations possible if the same bit depth was used for luma and chroma.

Table 1 and Table 2 show the combinations for 10-bit and 12-bit profiles where the shaded boxes indicate all the possible combinations and the ‘X’ represent the combinations where the luma and chroma bit depths are the same.  These tables illustrate how constraining the luma and chroma bit depths to be identical reduces the possible combinations from 9 to 3 for the 10-bit profile and from 25 to 5 for the 12-bit profile.

Table 1: Combinations for a 10-bit profile

|  | Luma Bit Depth |  |  |  |
| --- | --- | --- | --- | --- |
| Chroma Bit Depth |  | 8 | 9 | 10 |
|  | 8 | X |  |  |
|  | 9 |  | X |  |
|  | 10 |  |  | X |

Table 2: Combinations for a 12-bit profile

|  | Luma Bit Depth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Chroma Bit Depth |  | 8 | 9 | 10 | 11 | 12 |
|  | 8 | X |  |  |  |  |
|  | 9 |  | X |  |  |  |
|  | 10 |  |  | X |  |  |
|  | 11 |  |  |  | X |  |
|  | 12 |  |  |  |  | X |

The authors believe the original intent was to enable implementation flexibility at the encoder, but the experience and impression from HEVC Main 10 Profile product development is that different bit depths were only used in test bit streams.  This contribution contends that constraining the combinations to require the luma and chroma to be the same bit depth would still provide sufficient flexibility at the encoder, and eliminate the need to test and verify all the other combination that seem to not be used in practice.

# Proposal

This proposal recommends coding a single bit depth to be used for both luma and chroma.  Specifically, replace the bit_depth_luma_minus8 and bit_depth_chroma_minus8 syntax elements in the SPS with a single  bit_depth_minus8 syntax element.  Detailed specification changes are an attachment to this contribution.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, document JVET-O2001, JVET,

Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).