| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0245-v1 |
| --- | --- |

| Title: | AHG12/AHG17: Subpicture Properties |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan, Tim Hellman, Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The design principle behind subpictures is that each subpicture is independently coded but in a similar manner so that multiple subpictures can be aggregated into a picture which can be decoded as a single picture.  This contribution asserts it is important to carefully define the constraints and properties of any picture with subpictures to ensure maximum interoperability without introducing significant complexity beyond a profile without subpicture support.  A set of “subpicture rules” are proposed to be maintained as the subpicture design evolves until it is finalized.

# Proposal

- All of the subpictures in the aggregate picture must use a common DPB with common DPB management.  It is claimed that this is a critical property to maintain as there is significant complexity with having to track and switch between multiple DPB configurations.

- As a result of each subpicture sharing the same DPB, the following properties would be shared across each subpicture:

  - Same frame rate

  - Same reference picture lists

  - DPB can only be flushed at the same picture

  - Same temporal layer

  - Same reference picture resampling (RPR) scale factor

  - Same collocated reference picture

  - Delta-POCs to all reference pictures must be the same

- Tile rows and columns at the picture level apply across the entire picture.  Whatever tile structure is used by each sub-picture must align with the tile row/column grid at the picture level.

- The SPS/PPS/APS parameters must be the same for all the subpictures in a picture.  In a picture without subpictures, these parameters can be effectively loaded once to decode the whole picture.  It is claimed that if these parameters are allowed to change, then there is significant complexity with having to track and switch between different SPS/PPS/APS parameters based on the subpicture. Without such a requirement, every parameter in the standard could effectively change in every slice. Supporting such a case would be a significant burden on decoder implementations.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, document JVET-O2001, JVET,

Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).