| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0246 |
| --- | --- |

| Title: | AHG17/Non-CE5: on loop filter processing for subpicture treated as a picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Seethal Paluri Junghak Nam Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to modify the signalling of the in-loop filtering across a subpicture boundary. The existing subpicture syntax uses two flags i.e., subpic_treated_as_pic_flag and loop_filter_across_subpic_enabled_flag to control if each subpicture within the bitstream is treated as a picture and if the loop filtering is applied across its boundaries. However, it is asserted that in order to facilitate extractability, loop filter should be disabled when the subpicture is treated as a picture. Therefore, this contribution proposes to signal the loop_filter_across_subpic_enabled_flag only when subpicture is not treated as a picture or to restrict the flag should be 0 when subpicture is treated as a picture by conformance. Furthermore, it is also asserted that modifying pixels across subpicture that has subpic_treated_as_pic_flag is equal to 1 should be disallowed.

- Background

Subpictures are introduced into the Working Draft (WD 6) of the Versatile Video Coding (VVC) specification text. Subpictures which are independently decodable require normative changes to the low-level decoding process, i.e., treating subpicture boundaries as picture boundaries and enabling/disabling the loop filter across the subpicture boundaries [1]. These two aspects are controlled independently for each subpicture using two flags in the Sequence Parameter Set (SPS), i.e., subpic_treated_as_pic_flag and loop_filter_across_subpic_enabled_flag. When subpic_treated_as_pic_flag is enabled, the subpicutre boundaries are treated as picture boundaries restricting prediction across the boundaries. Similarly, if the loop_filter_across_subpic_enabled_flag is enabled, then the loop filter across subpicture boundaries is enabled. It should be noted that the deblocking filter is applied to the edge that coincides with the boundary of a subpicture picture for which loop_filter_across_subpc_enabled_flag is equal to 1 even though the across sub-picture is treated as a picture. Additionally, Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) processes are not applied when the sample being referenced is from a different subpicture and whose loop_filter_across_subpicture_enabled_flag is equal to 0.

Figure 1 Shows the deblocking across subpicture boundaries.

Figure 1 above shows two subpictures, with their corresponding flags. Here, for the subpicture on the left side, the loop filter is enabled despite the subpic_treated_as_pic_flag equal to 1. It is asserted that this hampers, main design goal of extractability.

Figure 2 shows an example where a subpicture is extracted.

Figure 2 above demonstrates the problem statement for motion compensation when current sub-picture is treated as a picture. In such a case, a restriction is placed on the interpolation and the temporal motion prediction outside the subpicture of reference frame when subpic_treated_as_pic_flag is 1. But when enabling the de-blocking filter on edge of a subpicture boundary, the samples which are inside the subpicture are modified. This implies that the reference subpicture should be referred to outside the subpicture even though subpicture is treated as a picture.

Proposal

This contribution proposes to signal the loop_filter_across_subpic_enabled_flag only for those subpictures that are not treated as a picture. The proposed modification will enable subpictures to be decoded independently. The proposed method could be represented by bitstream conformance.

Table 1 shows the excerpt from the SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| if( !subpic_treated_as_pic_flag[ i ] ) |  |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

Where the semantics is described as:

sps_decoding_parameter_set_id, when greater than 0, specifies the value of dps_decoding_parameter_set_id for the DPS referred to by the SPS. When sps_decoding_parameter_set_id is equal to 0, the SPS does not refer to a DPS and no DPS is referred to when decoding each CVS referring to the SPS. The value of sps_decoding_parameter_set_id shall be the same in all SPSs that are referred to by coded pictures in a bitstream.

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS. When sps_video_parameter_set_id is equal to 0, the SPS does not refer to a VPS and no VPS is referred to when decoding each CVS referring to the SPS.

subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the SPS RBSP syntax. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the SPS RBSP syntax.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs.

max_subpics_minus1 plus 1 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / 4) ) bits.

The variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * 4 + 3 ) /
		( subpic_grid_col_width_minus1 * 4 + 4 )	(7-5)

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / 4) ) bits.

The variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * 4 + 3 ) /
		( subpic_grid_row_height_minus1 * 4 + 4 )	(7-6)

subpic_grid_idx[ i ][ j ] specifies the subpicture index of the grid position (i, j). The length of the syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits.

The variables SubPicTop[ subpic_grid_idx[ i ][ j ] ], SubPicLeft[ subpic_grid_idx[ i ][ j ] ], SubPicWidth[ subpic_grid_idx [ i ][ j ] ], SubPicHeight[ subpic_grid_idx[ i ][ j ] ], and NumSubPics are derived as follows:

NumSubPics = 0
for( i = 0; i. < NumSubPicGridRows; i++ ) {
	for( j = 0; j < NumSubPicGridCols; j++ ) {
		if ( i = = 0)
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = 0
		else if( subpic_grid_idx[ i ][ j ]  !=  subpic_grid_idx[ i – 1 ][ j ] ) {
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = i
			SubPicHeight[ subpic_grid_idx[ i − 1][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ]
		}
		if ( j = = 0)
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = 0	(7-7)
		else if (subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i ][ j − 1 ] ) {
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = j
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ]
		}
		if ( i = = NumSubPicGridRows − 1)
			SubPicHeight[ subpic_grid_idx[ i ][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] + 1			if (j = = NumSubPicGridRows − 1)
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] + 1
		if( subpic_grid_idx[ i ][ j ] > NumSubPics)
			NumSubPics = subpic_grid_idx[ i ][ j ]
	}
}

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is derived as follows:

- If subpic_treated_as_pic_flag[ i ] is equal to 1, loop_filter_across_subpic_enabled_flag[ i ] is set equal to 0.

- Otherwise loop_filter_across_subpic_enabled_flag[ i ] is set equal to 1.

Alternatively the this suggestion could be replace conformance bitstrea conformance as below

It is a requirement of bitstream conformance that the value of loop_filter_across_subpic_enabled_flag[ i ] of the subpicture[i] shall be 0 when the subpic_treated_as_pic_flag[ i ] is equal to 1.

8.8.3.6.7	Filtering process for a luma sample using long filters

….

When nDp is greater than 0 and one or more of the following conditions are true, nDp is set equal to 0:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

- subpic_treated_as_pic_flag [ SubPicIdx of the coding unit that includes the coding block containing the sample p0 ] is equal to 0,

When nDq is greater than 0 and one or more of the following conditions are true, nDq is set equal to 0:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample q	0 is equal to 1.

8.8.6.3.7 Filtering process for a luma sample using long filters

...

When one or more of the following conditions are true, the filtered sample value, p i′ is substituted by the corresponding input sample value p i with i = 0..maxFilterLengthP − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- subpic_treated_as_pic_flag [ SubPicIdx of the coding unit that includes the coding block containing the sample p0 ] is equal to 0,

When one or more of the following conditions are true, the filtered sample value, q i′ is substituted by the corresponding input sample value q j with j = 0..maxFilterLengthQ − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

8.8.6.3.9 Filtering process for a chroma sample

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

- the variable maxFilterLength,

- the chroma sample values pi and qi with i = 0..maxFilterLengthCbCr,

- the chroma locations of pi and qi, ( xPi, yPi ) and ( xQi, yQi ) with i = 0..maxFilterLengthCbCr − 1,

- a variable tC.

Outputs of this process are the filtered sample values pi′ and qi′ with i = 0..maxFilterLengthCbCr − 1.

The filtered sample values pi′ and qi′ with i = 0..maxFilterLengthCbCr − 1 are derived as follows:

- If maxFilterLengthCbCr is equal to 3, the following strong filtering applies:

p0′ = Clip3( p0 − tC, p0 + tC, ( p3 + p2 + p1 + 2 * p0 + q0 + q1 + q2  + 4 )  >>  3 )	(8-1198)

p1′ = Clip3( p1 − tC, p1 + tC, ( 2 * p3 + p2 + 2 * p1 + p0 + q0 + q1  + 4 )  >>  3 )	(8-1199)

p2′ = Clip3( p2 − tC, p2 + tC, ( 3 * p3 + 2 * p2 + p1 + p0 + q0 + 4 )  >>  3 )	(8-1200)

q0′ = Clip3( q0 − tC, q0 + tC, ( p2 + p1 + p0 + 2 * q0 + q1 + q2 + q3  + 4 )  >>  3 )	(8-1201)

q1′ = Clip3( q1 − tC, q1 + tC, ( p1 + p0 + q0 + 2 * q1 + q2 + 2 * q3  + 4 )  >>  3 )	(8-1202)

q2′= Clip3( q2 − tC, q2 + tC, ( p0 + q0 + q1 + 2 * q2 + 3 * q3 + 4 )  >>  3 )	(8-1203)

- Otherwise, the following weak filtering applies:

= Clip3( −tC, tC, ( ( ( ( q0 − p0 )  <<  2 ) + p1 − q1 + 4 )  >>  3 ) )	(8-1204)

p0′ = Clip1C( p0 +  )		(8-1205)

q0′ = Clip1C( q0 −  )		(8-1206)

When one or more of the following conditions are true, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthCbCr − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- subpic_treated_as_pic_flag [ SubPicIdx of the coding unit that includes the coding block containing the sample p0 ] is equal to 0,

When one or more of the following conditions are true, the filtered sample value, qi′ is substituted by the corresponding input sample value qi with i = 0..maxFilterLengthCbCr − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

Reference

- Y.Wang, Hendry, J. Chien, S. Hong, J. Boyce, L. Xu, R. Skupin, Y. Sanchez, K. Suehring and M. Hannuksela , “AHG12: Text for subpicture agreements integration,” document JVET-O1143, 15th JVET meeting: Gothernburg, SE, 3–12 July 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).