| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET- P0251-v2 |
| --- | --- |

| Title: | CE5-related: Simplified CCALF with 6 filter coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +86-571-28163645 {yin.zhao, haitao.yang} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution simplifies the CCALF filter of CE5-2.1 from 14 filter coefficients to 6. It is reported that the 6-coefficient CCALF (C6-1) results in 0.11% / -6.80% / -5.34% BD-rates in AI configuration, and 0.17% / -9.29% / -8.70% BD-rates in RA configuration.

In version 2, a flag is coded in APS to indicate whether C6-1 or C6-2 filter is used for each of Cb and Cr. It is reported that this 2-type 6-coefficient CCALF (C6-3) results in 0.11% / -7.02% / -5.58% BD-rates in AI configuration, and 0.18% / -9.96% / -9.48% BD-rates in RA configuration.

Proposed Method

The CE5-2.1 uses a 14-coefficient CCALF filter for chroma, as shown in Fig. 1(a). This proposal tested two 6-coefficient CCALF filters, as shown in Fig. 1(b) and Fig. 1(c), which has similar complexity to the chroma ALF in VVC (also 6-coefficicnet). In Fig. 1, the filtered chroma sample in YUV4:2:0 is located at C(0,0) position.

- C14                       (b)  C6-1                     (c) C6-2

Fig. 1. The 14-coefficient CCALF in CE5-2.1 and the proposed 6-coefficient CCALF

The multiplication per sample for ALF part of VTM6, VTM + C14 CCALF, VTM + C6 CCALF is counted in Table 1.

Table 1 Multiplication per sample analysis

|  | Luma ALF | Chroma ALF | CC ALF | Total Multi per sample | Percentage over VTM |
| --- | --- | --- | --- | --- | --- |
| VTM | 12 | 6 / 2 = 3 | 0 | 15 | 100% |
| VTM + C14 CCALF | 12 | 6 / 2 = 3 | 14 / 2 = 7 | 22 | 147% |
| VTM + C6 CCALF | 12 | 6 / 2 = 3 | 6 / 2 = 3 | 18 | 120% |

Equation for deriving the delta value D for the current chroma sample is as follows:

D = ( i=05L(i) * Coeff(i) + 64 ) >> 7

where L(i), i=0,..5, denotes the 6 corresponding luma samples.

It is observed that the C6-1 filter performs better than C6-2 filter in some sequences, while C6-2 filter performs better in others (e.g., much better in class F). Therefore, a combined test is conducted in which the choice of C6-1 filter or C6-2 filter is signaled by a flag in APS (right after the CCALF coefficients). Cb and Cr can have different choice of filter type, since they use different filter coefficients. The filter type is decided in RDO process of CCALF coefficients. For simplicity, this 2-type 6-tap CCALF filter is denoted C6-3.

Experiment results

The proposed method were implemented on top of CE5-2.1 software, and tested under AI, RA and LDB configuration of the Common Test Conditions (CTC). A summary of the coding performance (as well as comparison with the CE5-2.1 and six tap filter in P0173) is provided in Table 2. Detailed results can be found in Table 3 to Table 4.

Table 2 Coding performance summary of C14 CCALF and C6 CCLAF.

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| C14(CE5-2.1) | AI | 0.13% | -7.58% | -6.01% | 103% | 103% |
| Six Tap in P0173 | AI | 0.10% | -6.60% | -4.83% | 98% | 99% |
| C6-1 | AI | 0.11% | -6.80% | -5.34% | 101% | 104% |
| C6-2 | AI | 0.10% | -6.75% | -5.17% | 101% | 104% |
| C6-3 | AI | 0.11% | -7.02% | -5.58% | 102% | 103% |
| C14(CE5-2.1) | RA | 0.16% | -10.69% | -10.07% | 101% | 100% |
| Six Tap in P0173 | RA | 0.16% | -9.00% | -8.22% | 101% | 103% |
| C6-1 | RA | 0.17% | -9.29% | -8.70% | 101% | 102% |
| C6-2 | RA | 0.16% | -9.46% | -8.99% | 101% | 103% |
| C6-3 | RA | 0.18% | -9.96% | -9.48% | 101% | 102% |
| C14(CE5-2.1) | LB | 0.17% | -15.62% | -11.75% | 101% | 99% |
| Six Tap in P0173 | LB | 0.13% | -11.98% | -8.24% | 96% | 99% |
| C6-1 | LB |  |  |  |  |  |
| C6-2 | LB |  |  |  |  |  |
| C6-3 | LB |  |  |  |  |  |

Note: CE5-2.1 and “Six Tap in P0173” results are copied from CE5-2.1 and JVET-P0173 excel sheets, for comparing the BD-rates.

Table 3 Coding performance of C6-1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -3.22% | -3.70% | 103% | 105% |
| Class A2 | 0.14% | -8.00% | -8.37% | 101% | 105% |
| Class B | 0.09% | -9.82% | -7.51% | 101% | 104% |
| Class C | 0.08% | -3.10% | -3.07% | 101% | 102% |
| Class E | 0.13% | -9.10% | -3.36% | 101% | 103% |
| Overall | 0.11% | -6.80% | -5.34% | 101% | 104% |
| Class D | 0.10% | -2.32% | -2.93% | 101% | 102% |
| Class F | 0.08% | -2.62% | -2.19% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.21% | -4.82% | 101% | 103% |
| Class A2 | 0.25% | -9.72% | -13.60% | 100% | 104% |
| Class B | 0.20% | -15.69% | -12.23% | 101% | 102% |
| Class C | 0.10% | -4.00% | -3.53% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -9.29% | -8.70% | 101% | 102% |
| Class D | 0.09% | -4.09% | -4.46% | 101% | 101% |
| Class F | 0.10% | -2.31% | -1.98% | 101% | 102% |

Table 4 Coding performance of C6-2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -3.08% | -3.32% | 103% | 104% |
| Class A2 | 0.13% | -7.54% | -8.31% | 101% | 105% |
| Class B | 0.08% | -10.00% | -7.45% | 101% | 104% |
| Class C | 0.08% | -3.11% | -3.08% | 101% | 102% |
| Class E | 0.13% | -9.07% | -2.87% | 101% | 104% |
| Overall | 0.10% | -6.75% | -5.17% | 101% | 104% |
| Class D | 0.07% | -2.34% | -2.61% | 101% | 103% |
| Class F | 0.14% | -4.89% | -5.20% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.81% | -4.46% | 101% | 102% |
| Class A2 | 0.24% | -9.73% | -14.52% | 100% | 104% |
| Class B | 0.21% | -16.51% | -12.87% | 101% | 102% |
| Class C | 0.07% | -3.95% | -3.40% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -9.46% | -8.99% | 101% | 103% |
| Class D | 0.11% | -4.15% | -3.79% | 101% | 101% |
| Class F | 0.16% | -3.54% | -3.52% | 101% | 103% |

Table 5 Coding performance of C6-3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -3.27% | -3.78% | 103% | 103% |
| Class A2 | 0.14% | -8.16% | -8.54% | 102% | 103% |
| Class B | 0.08% | -10.26% | -8.08% | 102% | 103% |
| Class C | 0.08% | -3.32% | -3.26% | 101% | 102% |
| Class E | 0.13% | -9.15% | -3.37% | 101% | 103% |
| Overall | 0.11% | -7.02% | -5.58% | 102% | 103% |
| Class D | 0.10% | -2.61% | -3.15% | 101% | 103% |
| Class F | 0.14% | -4.98% | -5.27% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.28% | -5.07% | 101% | 102% |
| Class A2 | 0.25% | -10.23% | -14.57% | 101% | 102% |
| Class B | 0.22% | -17.07% | -13.52% | 101% | 101% |
| Class C | 0.10% | -4.38% | -3.94% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | -9.96% | -9.48% | 101% | 102% |
| Class D | 0.11% | -4.70% | -4.59% | 101% | 101% |
| Class F | 0.16% | -3.73% | -3.64% | 101% | 102% |

Conclusions

It is suggested to adopt the simplified CCALF into VVC.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).