| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET- P0251-v3 |
| --- | --- |

| Title: | CE5-related: Simplified CCALF with 6 filter coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +86-571-28163645 {yin.zhao, haitao.yang} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution simplifies the CCALF filter of CE5-2.1 from 14 filter coefficients to 6. It is reported that the 6-coefficient CCALF (C6-1) results in 0.11% / -6.80% / -5.34% BD-rates in AI configuration, and 0.17% / -9.29% / -8.70% BD-rates in RA configuration.

In version 2, a flag is coded in APS to indicate whether C6-1 or C6-2 filter is used for each of Cb and Cr. It is reported that this 2-type 6-coefficient CCALF (C6-3) results in 0.11% / -7.02% / -5.58% BD-rates in AI configuration, and 0.18% / -9.96% / -9.48% BD-rates in RA configuration.

In version 3, the spec change over CE5-2.1 is provided.

Proposed Method

The CE5-2.1 uses a 14-coefficient CCALF filter for chroma, as shown in Fig. 1(a). This proposal tested two 6-coefficient CCALF filters, as shown in Fig. 1(b) and Fig. 1(c), which has similar complexity to the chroma ALF in VVC (also 6-coefficicnet). In Fig. 1, the filtered chroma sample in YUV4:2:0 is located at C(0,0) position.

- C14                       (b)  C6-1                     (c) C6-2

Fig. 1. The 14-coefficient CCALF in CE5-2.1 and the proposed 6-coefficient CCALF

The multiplication per sample for ALF part of VTM6, VTM + C14 CCALF, VTM + C6 CCALF is counted in Table 1.

Table 1 Multiplication per sample analysis

|  | Luma ALF | Chroma ALF | CC ALF | Total Multi per sample | Percentage over VTM |
| --- | --- | --- | --- | --- | --- |
| VTM | 12 | 6 / 2 = 3 | 0 | 15 | 100% |
| VTM + C14 CCALF | 12 | 6 / 2 = 3 | 14 / 2 = 7 | 22 | 147% |
| VTM + C6 CCALF | 12 | 6 / 2 = 3 | 6 / 2 = 3 | 18 | 120% |

Equation for deriving the delta value D for the current chroma sample is as follows:

D = ( i=05L(i) * Coeff(i) + 64 ) >> 7

where L(i), i=0,..5, denotes the 6 corresponding luma samples.

It is observed that the C6-1 filter performs better than C6-2 filter in some sequences, while C6-2 filter performs better in others (e.g., much better in class F). Therefore, a combined test is conducted in which the choice of C6-1 filter or C6-2 filter is signaled by a flag in APS (right after the CCALF coefficients). Cb and Cr can have different choice of filter type, since they use different filter coefficients. The filter type is decided in RDO process of CCALF coefficients. For simplicity, this 2-type 6-tap CCALF filter is denoted C6-3.

Experiment results

The proposed method were implemented on top of CE5-2.1 software, and tested under AI, RA and LDB configuration of the Common Test Conditions (CTC). A summary of the coding performance (as well as comparison with the CE5-2.1 and six tap filter in P0173) is provided in Table 2. Detailed results can be found in Table 3 to Table 4.

Table 2 Coding performance summary of C14 CCALF and C6 CCLAF.

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| C14(CE5-2.1) | AI | 0.13% | -7.58% | -6.01% | 103% | 103% |
| Six Tap in P0173 |  | 0.10% | -6.60% | -4.83% | 98% | 99% |
| Eight Tap in P0173 |  | 0.11% | -7.13% | -5.42% | 102% | 100% |
| C6-1 |  | 0.11% | -6.80% | -5.34% | 101% | 104% |
| C6-2 |  | 0.10% | -6.75% | -5.17% | 101% | 104% |
| C6-3 |  | 0.11% | -7.02% | -5.58% | 102% | 103% |
| C14(CE5-2.1) | RA | 0.16% | -10.69% | -10.07% | 101% | 100% |
| Six Tap in P0173 |  | 0.16% | -9.00% | -8.22% | 101% | 103% |
| Eight Tap in P0173 |  | 0.16% | -9.75% | -9.40% | 100% | 103% |
| C6-1 |  | 0.17% | -9.29% | -8.70% | 101% | 102% |
| C6-2 |  | 0.16% | -9.46% | -8.99% | 101% | 103% |
| C6-3 |  | 0.18% | -9.96% | -9.48% | 101% | 102% |
| C14(CE5-2.1) | LB | 0.17% | -15.62% | -11.75% | 101% | 99% |
| Six Tap in P0173 |  | 0.13% | -11.98% | -8.24% | 96% | 99% |
| Eight Tap in P0173 |  | 0.16% | -13.90% | -10.05% | 98% | 103% |
| C6-1 |  | 0.17% | -14.40% | -10.34% | 101% | 102% |
| C6-2 |  | 0.15% | -13.50% | -9.80% | 101% | 102% |
| C6-3 |  | 0.18% | -15.19% | -11.34% | 101% | 102% |

Note: CE5-2.1 and “Six Tap in P0173” results are copied from CE5-2.1 and JVET-P0173 excel sheets, for comparing the BD-rates.

The C6-1 has similar performance as C6-2 in AI and RA, and slightly better performance in LB. The C6-1 slightly outperforms the six tap in P0173, especially in LB (around 2% better in chroma).

The C6-3 has close performance as the 8-tap filter in P0173 in AI and RA, and has 1% chroma gain over the 8-tap filter. Compared with the 18 tap filter in CE5-2.1, the C6-3 filter shows almost no luma loss, and around 0.5% chroma loss on AI, RA and LB.

Table 3 Coding performance of C6-1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -3.22% | -3.70% | 103% | 105% |
| Class A2 | 0.14% | -8.00% | -8.37% | 101% | 105% |
| Class B | 0.09% | -9.82% | -7.51% | 101% | 104% |
| Class C | 0.08% | -3.10% | -3.07% | 101% | 102% |
| Class E | 0.13% | -9.10% | -3.36% | 101% | 103% |
| Overall | 0.11% | -6.80% | -5.34% | 101% | 104% |
| Class D | 0.10% | -2.32% | -2.93% | 101% | 102% |
| Class F | 0.08% | -2.62% | -2.19% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.21% | -4.82% | 101% | 103% |
| Class A2 | 0.25% | -9.72% | -13.60% | 100% | 104% |
| Class B | 0.20% | -15.69% | -12.23% | 101% | 102% |
| Class C | 0.10% | -4.00% | -3.53% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -9.29% | -8.70% | 101% | 102% |
| Class D | 0.09% | -4.09% | -4.46% | 101% | 101% |
| Class F | 0.10% | -2.31% | -1.98% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | -19.14% | -16.70% | 100% | 103% |
| Class C | 0.14% | -6.94% | -5.48% | 100% | 101% |
| Class E | 0.15% | -16.43% | -6.24% | 101% | 102% |
| Overall | 0.17% | -14.40% | -10.34% | 101% | 102% |
| Class D | 0.18% | -7.67% | -5.90% | 100% | 100% |
| Class F | 0.16% | -5.04% | -3.77% | 101% | 103% |

Table 4 Coding performance of C6-2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -3.08% | -3.32% | 103% | 104% |
| Class A2 | 0.13% | -7.54% | -8.31% | 101% | 105% |
| Class B | 0.08% | -10.00% | -7.45% | 101% | 104% |
| Class C | 0.08% | -3.11% | -3.08% | 101% | 102% |
| Class E | 0.13% | -9.07% | -2.87% | 101% | 104% |
| Overall | 0.10% | -6.75% | -5.17% | 101% | 104% |
| Class D | 0.07% | -2.34% | -2.61% | 101% | 103% |
| Class F | 0.14% | -4.89% | -5.20% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.81% | -4.46% | 101% | 102% |
| Class A2 | 0.24% | -9.73% | -14.52% | 100% | 104% |
| Class B | 0.21% | -16.51% | -12.87% | 101% | 102% |
| Class C | 0.07% | -3.95% | -3.40% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -9.46% | -8.99% | 101% | 103% |
| Class D | 0.11% | -4.15% | -3.79% | 101% | 101% |
| Class F | 0.16% | -3.54% | -3.52% | 101% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.19% | -18.85% | -17.03% | 101% | 103% |
| Class C | 0.13% | -5.43% | -4.34% | 100% | 101% |
| Class E | 0.10% | -15.35% | -5.04% | 101% | 103% |
| Overall | 0.15% | -13.50% | -9.80% | 101% | 102% |
| Class D | 0.13% | -6.66% | -4.27% | 100% | 101% |
| Class F | 0.19% | -6.36% | -5.17% | 101% | 103% |

Table 5 Coding performance of C6-3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -3.27% | -3.78% | 103% | 103% |
| Class A2 | 0.14% | -8.16% | -8.54% | 102% | 103% |
| Class B | 0.08% | -10.26% | -8.08% | 102% | 103% |
| Class C | 0.08% | -3.32% | -3.26% | 101% | 102% |
| Class E | 0.13% | -9.15% | -3.37% | 101% | 103% |
| Overall | 0.11% | -7.02% | -5.58% | 102% | 103% |
| Class D | 0.10% | -2.61% | -3.15% | 101% | 103% |
| Class F | 0.14% | -4.98% | -5.27% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.28% | -5.07% | 101% | 102% |
| Class A2 | 0.25% | -10.23% | -14.57% | 101% | 102% |
| Class B | 0.22% | -17.07% | -13.52% | 101% | 101% |
| Class C | 0.10% | -4.38% | -3.94% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | -9.96% | -9.48% | 101% | 102% |
| Class D | 0.11% | -4.70% | -4.59% | 101% | 101% |
| Class F | 0.16% | -3.73% | -3.64% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | -20.43% | -18.29% | 101% | 102% |
| Class C | 0.16% | -7.60% | -6.02% | 101% | 101% |
| Class E | 0.19% | -16.58% | -6.84% | 103% | 103% |
| Overall | 0.18% | -15.19% | -11.34% | 101% | 102% |
| Class D | 0.20% | -7.47% | -6.17% | 101% | 101% |
| Class F | 0.22% | -7.38% | -6.27% | 101% | 102% |

Conclusions

It is suggested to adopt the simplified CCALF into VVC.

Draft Text change over CE5-4.1

[For clarity, this section is not prepared in trace mode]

Deleted text are marked in strikethrough. Added text are highlighted in yellow.

For Test 1 (C6-1)

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cross_component_cb_filter_signal_flag | u(1) |
| alf_cross_component_cr_filter_signal_flag | u(1) |
| … |  |
| if ( alf_cross_component_cb_filter_signal_flag ) |  |
| for ( j = 0; j < 146; j++ ) |  |
| alf_cross_component_cb_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cb_coeff_abs[ j ] ) |  |
| alf_cross_component_cb_coeff_sign[ j ] | u(1) |
| if ( alf_cross_component_cr_ filter_signal_flag ) |  |
| for ( j = 0; j < 146; j++ ) |  |
| alf_cross_component_cr_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cr_coeff_abs[ j ] ) |  |
| alf_cross_component_cr_coeff_sign[ j ] | u(1) |
| } |  |

8.8.5 Adaptive loop filter process

8.8.5.1 General

  - For every luma location ( rx , ry ), where rx = 0..pic_width_in_luma_samples / CcAlfWidthCbL − 1 and ry = 0..pic_height_in_luma_samples / CcAlfHeightCbL − 1, the following applies:

  - When ChromaArrayType is not equal to 0 and alf_cross_component_cb_flag[ rx ][ ry ] is equal to 1, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCb, chroma block of samples location ( xC, yC ) set equal to ( rx * CcAlfWidthCbL / SubWidthC, ry * CcAlfHeightCbL / SubHeightC ), ccAlfWidth set equal to CcAlfWidthCbL / SubWidthC, ccAlfHeight set equal to CcAlfHeightCbL / SubHeightC, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCb[ j ], with j = 0..135, as inputs, and the output is the modified filtered picture ccAlfPictureCb.

  - For every luma location ( rx , ry ), where rx = 0..pic_width_in_luma_samples / CcAlfWidthCrL − 1 and ry = 0..pic_height_in_luma_samples / CcAlfHeightCrL − 1, the following applies:

  - When ChromaArrayType is not equal to 0 and alf_cross_component_cr_flag[ rx ][ ry ] is equal to 1, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCr, chroma block of samples location ( xC, yC ) set equal to ( rx * CcAlfWidthCrL / SubWidthC, ry* CcAlfHeightCrL / SubHeightC ), ccAlfWidth set equal to CcAlfWidthCrL / SubWidthC, ccAlfHeight set equal to CcAlfHeightCrL / SubHeightC, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCr[ j ], with j = 0..135, as inputs, and the output is the modified filtered picture ccAlfPictureCr.

- x.x.x.x Cross component filtering process for block of chroma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the luma adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPictureC,

- a chroma location ( xC, yC ) specifying the top left sample of the current block of chroma samples relative to the top left sample of the current picture,

- a width ccAlfWidth of block of chroma samples

- a height ccAlfHeight of block of chroma samples

- cross component filter coefficients CcAlfCoeff[j], with j = 0..135

…

  - The variable sum is derived as follows:

sum = 	f[ 0 ] * recPictureL[ hx, vy + yM2 ] +
	f[ 1 ] * recPictureL[ hx+xM1, vy + yM1 ] +
	f[ 2 ] * recPictureL[ hx, vy + yM1 ] +
	f[ 3 ] * recPictureL[ hx+xP1, vy + yM1 ] +
	f[ 4 ] * recPictureL[ hx+xM2, vy ] +
	f[ 5 ] * recPictureL[ hx+xM1, vy ] +
	f[ 6 ] * recPictureL[ hx, vy ] +
	f[ 7 ] * recPictureL[ hx+xP1, vy ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy ] +		(8-1289)
	f[ 4 ] * recPictureL[ hx+xM2, vy+yP1 ] +
	f[ 8 ] * recPictureL[ hx+xM1, vy+yP1 ] +
	f[ 9 ] * recPictureL[ hx, vy+yP1 ] +
	f[ 10 ] * recPictureL[ hx+xP1, vy+yP1 ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy+yP1 ] +
	f[ 11 ] * recPictureL[ hx+xM1, vy + yP2 ] +
	f[ 12 ] * recPictureL[ hx, vy + yP2 ] +
	f[ 13 ] * recPictureL[ hx+xP1, vy + yP2 ] +
	f[ 0 ] * recPictureL[ hx, vy + yP3 ] +

sum = 	f[ 0 ] * recPictureL[ hx, vy ] +
f[ 1 ] * recPictureL[ hx, vy + yP1 ] +
f[ 2 ] * recPictureL[ hx + xM2, vy ] +
f[ 3 ] * recPictureL[ hx + xP2, vy + yP1 ] +
f[ 4 ] * recPictureL[ hx, vy+yM2 ] +
f[ 5 ] * recPictureL[ hx, vy+yP3 ]

sum = curr + ( sum + 64 ) >> 7 )		(8-1290)

For Test 2 (C6-2)

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cross_component_cb_filter_signal_flag | u(1) |
| alf_cross_component_cr_filter_signal_flag | u(1) |
| … |  |
| if ( alf_cross_component_cb_filter_signal_flag ) |  |
| for ( j = 0; j < 146; j++ ) |  |
| alf_cross_component_cb_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cb_coeff_abs[ j ] ) |  |
| alf_cross_component_cb_coeff_sign[ j ] | u(1) |
| if ( alf_cross_component_cr_ filter_signal_flag ) |  |
| for ( j = 0; j < 146; j++ ) |  |
| alf_cross_component_cr_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cr_coeff_abs[ j ] ) |  |
| alf_cross_component_cr_coeff_sign[ j ] | u(1) |
| } |  |

8.8.5 Adaptive loop filter process

8.8.5.1 General

  - For every luma location ( rx , ry ), where rx = 0..pic_width_in_luma_samples / CcAlfWidthCbL − 1 and ry = 0..pic_height_in_luma_samples / CcAlfHeightCbL − 1, the following applies:

  - When ChromaArrayType is not equal to 0 and alf_cross_component_cb_flag[ rx ][ ry ] is equal to 1, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCb, chroma block of samples location ( xC, yC ) set equal to ( rx * CcAlfWidthCbL / SubWidthC, ry * CcAlfHeightCbL / SubHeightC ), ccAlfWidth set equal to CcAlfWidthCbL / SubWidthC, ccAlfHeight set equal to CcAlfHeightCbL / SubHeightC, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCb[ j ], with j = 0..135, as inputs, and the output is the modified filtered picture ccAlfPictureCb.

  - For every luma location ( rx , ry ), where rx = 0..pic_width_in_luma_samples / CcAlfWidthCrL − 1 and ry = 0..pic_height_in_luma_samples / CcAlfHeightCrL − 1, the following applies:

  - When ChromaArrayType is not equal to 0 and alf_cross_component_cr_flag[ rx ][ ry ] is equal to 1, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCr, chroma block of samples location ( xC, yC ) set equal to ( rx * CcAlfWidthCrL / SubWidthC, ry* CcAlfHeightCrL / SubHeightC ), ccAlfWidth set equal to CcAlfWidthCrL / SubWidthC, ccAlfHeight set equal to CcAlfHeightCrL / SubHeightC, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCr[ j ], with j = 0..135, as inputs, and the output is the modified filtered picture ccAlfPictureCr.

- x.x.x.x Cross component filtering process for block of chroma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the luma adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPictureC,

- a chroma location ( xC, yC ) specifying the top left sample of the current block of chroma samples relative to the top left sample of the current picture,

- a width ccAlfWidth of block of chroma samples

- a height ccAlfHeight of block of chroma samples

- cross component filter coefficients CcAlfCoeff[j], with j = 0..135

…

  - The variable sum is derived as follows:

sum = 	f[ 0 ] * recPictureL[ hx, vy + yM2 ] +
	f[ 1 ] * recPictureL[ hx+xM1, vy + yM1 ] +
	f[ 2 ] * recPictureL[ hx, vy + yM1 ] +
	f[ 3 ] * recPictureL[ hx+xP1, vy + yM1 ] +
	f[ 4 ] * recPictureL[ hx+xM2, vy ] +
	f[ 5 ] * recPictureL[ hx+xM1, vy ] +
	f[ 6 ] * recPictureL[ hx, vy ] +
	f[ 7 ] * recPictureL[ hx+xP1, vy ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy ] +		(8-1289)
	f[ 4 ] * recPictureL[ hx+xM2, vy+yP1 ] +
	f[ 8 ] * recPictureL[ hx+xM1, vy+yP1 ] +
	f[ 9 ] * recPictureL[ hx, vy+yP1 ] +
	f[ 10 ] * recPictureL[ hx+xP1, vy+yP1 ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy+yP1 ] +
	f[ 11 ] * recPictureL[ hx+xM1, vy + yP2 ] +
	f[ 12 ] * recPictureL[ hx, vy + yP2 ] +
	f[ 13 ] * recPictureL[ hx+xP1, vy + yP2 ] +
	f[ 0 ] * recPictureL[ hx, vy + yP3 ] +

sum = 	f[ 0 ] * recPictureL[ hx, vy ] +
f[ 1 ] * recPictureL[ hx, vy + yP1 ] +
f[ 2 ] * recPictureL[ hx + xM1, vy ] +
f[ 3 ] * recPictureL[ hx + xP1, vy + yP1 ] +
f[ 4 ] * recPictureL[ hx, vy+yM1 ] +
f[ 5 ] * recPictureL[ hx, vy+yP2 ]

sum = curr + ( sum + 64 ) >> 7 )		(8-1290)

For Test 3 (C6-3)

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cross_component_cb_filter_signal_flag | u(1) |
| alf_cross_component_cr_filter_signal_flag | u(1) |
| … |  |
| if ( alf_cross_component_cb_filter_signal_flag ) |  |
| for ( j = 0; j < 146; j++ ) |  |
| alf_cross_component_cb_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cb_coeff_abs[ j ] ) |  |
| alf_cross_component_cb_coeff_sign[ j ] | u(1) |
| alf_cross_component_cb_type_flag | u(1) |
| if ( alf_cross_component_cr_ filter_signal_flag ) |  |
| for ( j = 0; j < 146; j++ ) |  |
| alf_cross_component_cr_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cr_coeff_abs[ j ] ) |  |
| alf_cross_component_cr_coeff_sign[ j ] | u(1) |
| alf_cross_component_cr_type_flag | u(1) |
| } |  |

alf_cross_component_cb_type_flag equal to 1 specifies that type-1 cross component ALF filter is applied for Cb. alf_cross_component_cb_type_flag equal to 0 specifies that type-0 cross component ALF filter is applied for Cb.

alf_cross_component_cr_type_flag equal to 1 specifies that type-1 cross component ALF filter is applied for Cr. alf_cross_component_cr_type_flag equal to 0 specifies that type-0 cross component ALF filter is applied for Cr.

8.8.5 Adaptive loop filter process

8.8.5.1 General

  - For every luma location ( rx , ry ), where rx = 0..pic_width_in_luma_samples / CcAlfWidthCbL − 1 and ry = 0..pic_height_in_luma_samples / CcAlfHeightCbL − 1, the following applies:

  - When ChromaArrayType is not equal to 0 and alf_cross_component_cb_flag[ rx ][ ry ] is equal to 1, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCb, chroma block of samples location ( xC, yC ) set equal to ( rx * CcAlfWidthCbL / SubWidthC, ry * CcAlfHeightCbL / SubHeightC ), ccAlfWidth set equal to CcAlfWidthCbL / SubWidthC, ccAlfHeight set equal to CcAlfHeightCbL / SubHeightC, alf_cross_component_cb_type_flag, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCb[ j ], with j = 0..135, as inputs, and the output is the modified filtered picture ccAlfPictureCb.

  - For every luma location ( rx , ry ), where rx = 0..pic_width_in_luma_samples / CcAlfWidthCrL − 1 and ry = 0..pic_height_in_luma_samples / CcAlfHeightCrL − 1, the following applies:

  - When ChromaArrayType is not equal to 0 and alf_cross_component_cr_flag[ rx ][ ry ] is equal to 1, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCr, chroma block of samples location ( xC, yC ) set equal to ( rx * CcAlfWidthCrL / SubWidthC, ry* CcAlfHeightCrL / SubHeightC ), ccAlfWidth set equal to CcAlfWidthCrL / SubWidthC, ccAlfHeight set equal to CcAlfHeightCrL / SubHeightC, alf_cross_component_cr_type_flag, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCr[ j ], with j = 0..135, as inputs, and the output is the modified filtered picture ccAlfPictureCr.

- x.x.x.x Cross component filtering process for block of chroma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the luma adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPictureC,

- a chroma location ( xC, yC ) specifying the top left sample of the current block of chroma samples relative to the top left sample of the current picture,

- a width ccAlfWidth of block of chroma samples

- a height ccAlfHeight of block of chroma samples

- a variable ccAlfType specifying the location of luma samples used in the filtering process

- cross component filter coefficients CcAlfCoeff[j], with j = 0..135

…

  - The variable sum is derived as follows:

sum = 	f[ 0 ] * recPictureL[ hx, vy + yM2 ] +
	f[ 1 ] * recPictureL[ hx+xM1, vy + yM1 ] +
	f[ 2 ] * recPictureL[ hx, vy + yM1 ] +
	f[ 3 ] * recPictureL[ hx+xP1, vy + yM1 ] +
	f[ 4 ] * recPictureL[ hx+xM2, vy ] +
	f[ 5 ] * recPictureL[ hx+xM1, vy ] +
	f[ 6 ] * recPictureL[ hx, vy ] +
	f[ 7 ] * recPictureL[ hx+xP1, vy ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy ] +		(8-1289)
	f[ 4 ] * recPictureL[ hx+xM2, vy+yP1 ] +
	f[ 8 ] * recPictureL[ hx+xM1, vy+yP1 ] +
	f[ 9 ] * recPictureL[ hx, vy+yP1 ] +
	f[ 10 ] * recPictureL[ hx+xP1, vy+yP1 ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy+yP1 ] +
	f[ 11 ] * recPictureL[ hx+xM1, vy + yP2 ] +
	f[ 12 ] * recPictureL[ hx, vy + yP2 ] +
	f[ 13 ] * recPictureL[ hx+xP1, vy + yP2 ] +
	f[ 0 ] * recPictureL[ hx, vy + yP3 ] +

sum = 	f[ 0 ] * recPictureL[ hx, vy ] +
f[ 1 ] * recPictureL[ hx, vy + yP1 ] +
f[ 2 ] * ( ccAlfType ? recPictureL[ hx + xM1, vy ] : recPictureL[ hx + xM2, vy ] ) +
f[ 3 ] * ( ccAlfType ? recPictureL[ hx + xP1, vy + yP1 ] : recPictureL[ hx + xP2, vy + yP1 ] ) +
f[ 4 ] * ( ccAlfType ? recPictureL[ hx, vy + yM1 ] : recPictureL[ hx, vy + yM2 ] ) +
f[ 5 ] * ( ccAlfType ? recPictureL[ hx, vy + yP2 ] : recPictureL[ hx, vy + yP3 ] )

sum = curr + ( sum + 64 ) >> 7 )		(8-1290)

Acknoledgement

We would like to thank MediaTek and Canon for the cross-check.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).