| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0252-v1 |
| --- | --- |

| Title: | AHG12: Loop filter control flag for tile boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma Virginie Drugeon 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution proposes to introduce loop filter control flag for tile boundary. The current specification supports the control flag for brick, slice, and subpicture boundary respectively. They are sufficient for the partition patterns where a tile is smaller than or equal to a slice. But, in the case where a tile is larger than a slice, there is no way to disable loop filter at tile boundary while enabling it at slice boundary. This proposal can solve this issue.

Introduction

VVC has three loop filter control flags for the boundaries as shown in Table1 [1]. They can switch loop filter on/of across brick, slice, and subpicture boundary respectively. But there is no control flag for tile boundary in the current specification.

Table1: loop filter control flags for the boundaries

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| if( subpics_present_flag ) { |  |
| for( i = 0; i <= NumSubPics; i++ ) |  |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| …. |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| if( !single_tile_in_pic_flag ) { |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| …. |  |
| } |  |

Figure1 shows the example of partitioning patterns discussed at past meetings. Pattern1 and Pattern2 are variations where a tile is equal to a brick, and a slice is always larger than or equal to a tile. For these patterns, loop_filter_across_bricks_enabled_flag and loop_filter_across_slices_enabled_flag has enough functionality for boundary control. On the other hand, Pattern3 is a variation where a tile is larger than a brick, and also a tile can be larger than a slice. This contribution focuses on this pattern.

Figure1: examples of partitioning patterns

Problem

Figure2 shows an example of Pattern2 variation. A picture is divided into 4 tiles, and each tile is divided to 3 or 4 slices (==bricks). This variation was commonly used in hardware encoders with multi-core processing in the HEVC generation. One core handles one tile, and each core works independently. In this use case, some architectures want to disable loop filter across tile boundaries but enable for slice boundaries inside a tile. This setting can avoid the communication process between multiple cores and simplify multi-core control. However, the current specification does not support this setting because there is no individual control flag for tile boundaries.

Figure2: an example that is not supported by current specification

Proposal

This contribution proposes to introduce loop_filter_across_tiles_enabled_flag. It can be signaled only when both loop_filter_across_slices_enabled_flag and brick_splitting_present_flag are true. This flag controls loop filter on/off for tile boundaries that are also slice boundaries. But this flag does not affect tile boundaries inside a slice. The followings are excerpts of remarkable part of the draft specification text.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !single_tile_in_pic_flag ) { |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) { |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| if( loop_filter_across_slices_enabled_flag && brick_splitting_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| } |  |
| …. |  |
| } |  |

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries that are also slice boundaries in pictures referring to the PPS. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries that are also slice boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. This flag does not affect tile boundaries inside a slice. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to (loop_filter_across_bricks_enabled_flag & & loop_filter_across_slices_enabled_flag).

Conclusion

This contribution proposes to introduce loop filter control flag for tile boundary. The current specification supports the control flag for brick, slice, and subpicture boundary respectively. They are sufficient for the partition patterns where a tile is smaller than or equal to a slice. But, in the case where a tile is larger than a slice, there is no way to disable loop filter at tile boundary while enabling it at slice boundary. This proposal can solve this issue, so we propose to adopt it into VVC.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).