| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0253-v1 |
| --- | --- |

| Title: | Non-CE4: Simplify the division process of DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Yusuke Kato Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com kato.yusuke003@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

The fractional pel parametric MV refinement of DMVR has a block level division processing. Since the output value of this division is limited to be in the range of -8 to 8, the normal integer precision division is specified. However, it requires a lot of number of comparison with several loops for the hardware implementation. This contribution proposes to introduce the division method using size 16 LUT into this process instead of the integer precision division. Simulation results reportedly show that the proposed method can simplify division process with 0.01% BD-rate loss for RA on VTM-6.0.

Introduction

VVC has several block level division processing in some tools [1]. One is the parameter calculation of CCLM, but this division is achieved by size 16 LUT method. Another one is the fractional pel parametric MV refinement of DMVR as shown in Figure1. Since the output value of this division is limited to be in the range of -8 to 8, the normal integer precision division is specified. However, it requires a lot of number of comparison with several loops for the hardware implementation.

dMvX=sad[0][1]-sad[2][1]sad[0][1]+sad[2][1]-sad[1][1]×2

dMvY=sad[1][0]-sad[1][2]sad[1][0]+sad[1][2]-sad[1][1]×2

Figure1: Fractional pel parametric MV refinement of DMVR

Proposal

This contribution proposes to introduce the division method using size 16 LUT into the parametric MV refinement of DMVR instead of the integer precision division. This method and LUT are the same as that of CCLM. The followings are VTM-6.0 methods and proposed methods of dMvX derivation process of DMVR. It is common to dMvY derivation. It can be much simpler than VTM-6.0 method.

VTM6.0 method

num = ( sad[0][1] − sad[2][1] ) << 4;

denom = (( sad[0][1] + sad[2][1] ) − ( sad[1][1] << 1 )) << 3;

if ( denom == 0 ) dMvX = 0;

else {

if ( sad[0][1] = = sad[1][1] )    dMvX = −8;

else if ( sad[2][1] = = sad[1][1] )	dMvX = 8;

else {

signNum = 0

if ( num < 0 ) {

num = −num

signNum = 1

}

quotient = 0

counter = 3

while ( counter > 0 ) {

counter = counter − 1

quotient = quotient << 1

if ( num >= denom ) {

num = num − denom

quotient = quotient + 1

}

denom = ( denom >> 1 )

}

if ( signNum = = 1 )  dMvX = −quotient

else               dMvX = quotient

}

}

Proposed method

num = ( sad[0][1] − sad[2][1] ) << 4;

denom = (( sad[0][1] + sad[2][1] ) − ( sad[1][1] << 1 )) << 3;

if ( denom == 0 ) dMvX = 0;

else {

x = Floor( Log2( denom ) );

normDiff = ( ( denom << 4 ) >> x ) & 15;

x += ( normDiff  !=  0 ) ? 4 : 3;

dMvX = ( num * (  divSigTable[ normDiff ] | 8 )) >> x;

}

divSigTable[ ]  = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }

Table1 shows the comparison of the number of arithmetic operations required for this division processing. Since DMVR has a division for each dMvX and dMvY, two times operations in this table are required for each CU (or 16x16 subblock).

Table1: Comparison of the number of arithmetic operations

|  | VTM-6.0 | Proposal |
| --- | --- | --- |
| Add, Sub | 9 | 3 |
| Mult | 0 | 1 |
| Shift | 6 | 3 |
| Comp | 11 | 1 |
| Sign | 2 | 0 |
| Floor(Log2) | 0 | 1 |
| LUT size | - | 3bit x 16 |

Simulation results

Table2 shows the simulation results of the proposed method. The simulation is conducted using VTM-6.0 with the test condition described in JVET-N1010[2]. These results show that the coding performance of the proposed method has 0.01% BD-rate loss for RA. It shows that the proposed method can unify and simplify division processing with negligible coding loss.

Table2: Simulation results of the proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Class B | 0.02% | 0.05% | 0.01% | 100% | 101% |
| Class C | 0.00% | -0.03% | -0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.01% | 100% | 100% |
| Class D | 0.08% | 0.11% | 0.14% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.02% | 100% | 100% |

Conclusion

The fractional pel parametric MV refinement of DMVR has a block level division processing. Since the output value of this division is limited to be in the range of -8 to 8, the normal integer precision division is specified. However, it requires a lot of number of comparison with several loops for the hardware implementation. This contribution proposes to introduce the division method using size 16 LUT into this process instead of the integer precision division. Simulation results reportedly show that the proposed method can simplify division process with 0.01% BD-rate loss for RA on VTM-6.0. We propose to adopt it into VVC.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[2]	F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).