| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0254-v1 |
| --- | --- |

| Title: | Issue of simplified luma mapping of LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

JVET-O0272 was adopted at the previous meeting as the encoder constraints for luma mapping of LMCS. But it has two problems; (1) Multiple pivots are assigned to one segment when the value of pivot is a multiple of 32. (2) Unintended mapping table is generated when the input is other than 10bit depth. This proposal solves these issues with the small modification for the constraints. And the simulation results reportedly show that the proposed method has no BD-rate change on VTM-6.0 under the CTC.

Introduction

JVET-O0272 [1] was adopted at the previous meeting as the encoder constraints for luma mapping of LMCS. Figure1 shows the concept of JVET-O0272. InputPivot indicating original domain and LmcsPivot indicating reshaped domain are associated with each other, and each has 16 elements. The axis of LmcsPivot is divided into the segments having the width of 32. In the encoder process of pivot derivation, if multiple pivots are assigned to one segment, the higher pivot is adjusted to the next segment. As the result of this adjustment, it is guaranteed that a single segment does not have multiple pivots, and the decoder can detect the corresponding pivot directly using “lumasample >> 5” instead of comparing the value with each pivot. This encoder constraint is specified in VVC draft [2] as follows;

It is a requirement of bitstream conformance that, for i = lmcs_min_bin_idx..LmcsMaxBinIdx, when the value of LmcsPivot[ i ] is not a multiple of 32, the value of ( LmcsPivot[ i ] >> 5 ) shall not be equal to the value of ( LmcsPivot[ i + 1 ] >> 5 ).

Figure1: Adjustment process of LMCS pivot (JVET-O0272).

Problems

From VVC draft and VTM-6.0 implementation, we have defined two problems.

(Problem1)

The adjustment is not performed when the value of previous pivot is a multiple of 32 as shown in Figure2. As the results, LmcsPivot[2] and LmcsPivot[3] in this example are assigned to the same segment, and the decoder cannot decide directly which pivot to be selected. This issue occurs due to the sentence of “when the value of LmcsPivot[ i ] is not a multiple of 32,” in the specification.

Figure2: Example of Problem1

(Problem2)

The width of a segment is fixed to be 32. Therefore, if the input image has a different BitDepth, the number of segments will be changed. Figure3 shows the examples when BitDepth is equal to 8, 10, and 12 respectively. In the case of 10bit, the number of segments is 32 and the number of pivots is 16, it will be good tradeoff between the accuracy of the slope and the memory size. On the other hand, in the case of 8bit, the number of segments is 8 and the number of pivots is 16, it means that it is not possible to express the slope less than 2. In the case of 12bit, the number of segments is 128 and the number of pivots is 16, the accuracy of the slope will be higher than necessary while the memory size will get larger.

Figure3: Examples of Problem2

Proposal

This contribution proposes to change the constraint in VVC draft as following. It contains two items.

It is a requirement of bitstream conformance that, for i = lmcs_min_bin_idx..LmcsMaxBinIdx, when the value of LmcsPivot[ i ] is not a multiple of 32, the value of ( LmcsPivot[ i ] >> (BitDepthY - 5) ) shall not be equal to the value of ( LmcsPivot[ i + 1 ] >> (BitDepthY - 5) ).

(Item1)

Remove the condition of “when the value of LmcsPivot[ i ] is not a multiple of 32,”. This modification can guarantee that a segment has always only one pivot.

(Item2)

Fix the number of segments to 32 using BitDepth. It can solve the unintended behavior when the input is other than 10bit depth.

Simulation results

Table1 shows the simulation results of the proposed method. The simulation is conducted using VTM-6.0 with the test condition described in JVET-N1010 [3]. These results show that the coding performance of the proposed method is completely same as VTM-6.0. It shows that the proposed method can solve the issues mentioned above without any coding loss.

Table1: Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Low delay Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

Additional issue

In VTM-6.0, LMCS does not work when InternalBitDepth is set to the value other than 10bit.

LMCS mapping table is constructed by following three steps in the encoder;

(1) analyze the input image and calculate the parameter at preAnalyzerLMCS()

(2) construct initial mapping table using the parameter at constructReshaperLMCS()

(3) adjust LMCS pivot mentioned in this proposal at adjustLmcsPivot()

However, the 1st step always returns false when InternalBitDepth is set to the value other than 10bit. Therefore, we couldn’t provide the test results of our proposal with 8bit or 12bit InternalBitDepth.

Conclusion

JVET-O0272 was adopted at the previous meeting as the encoder constraints for luma mapping of LMCS. But it has two problems; (1) Multiple pivots are assigned to one segment when the value of pivot is a multiple of 32. (2) Unintended mapping table is generated when the input is other than 10bit depth. This proposal solves these issues with the small modification for the constraints. And the simulation results reportedly show that the proposed method has no BD-rate change on VTM-6.0 under the CTC. We propose to adopt it into VVC.

References

[1] Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE2-related: Simplified inverse luma mapping”, JVET-O0272-v2, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[3]	F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).