| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0258-v3 |
| --- | --- |

| Title: | AHG18: Disabling Dependent Quantization for Lossless Blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Yung-Hsuan Chao, Hilmi E. Egilmez, Muhammed Coban, and Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Proposal

In VTM-6.0, trans quant bypass (QB) mode provides an early lossless alternative in VVC. Even though quantization and transform stages are bypassed in QB mode, residual coding still uses dependent quantization (DQ) and Q-state transitions in regular (non-TS) residual coding. We propose to disable DQ in QB mode such that Q-state is always 0 for lossless blocks.

Note that the changes are in residual coding, thus slice level DQ can be 1 such that in a mixed lossy and lossless scenario, DQ will be used for blocks that are lossy coded, and DQ will not be used for lossless blocks.

# Results

We run our anchor on the VTM-6.0-lossless branch [1]. The following settings are used to run both the anchor and our proposal: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. Moreover, QP value was also set as 0.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  |
| Class A1 | 2.2 | 2.2 | -0.13% | 2.3 | 0.0 | -300.00% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.00% | 1.9 | 0.0 | -300.00% |  |  |  |
| Class B | 2.1 | 2.1 | -0.17% | 2.3 | 2.3 | -0.28% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -0.24% | 2.6 | 2.6 | -0.44% | 2.6 | 2.6 | -33.79% |
| Class D | 1.9 | 2.0 | -0.26% | 2.8 | 2.8 | -0.50% | 2.7 | 2.8 | -0.50% |
| Class E | 2.9 | 2.9 | -0.19% |  |  |  | 3.3 | 0.0 | -300.00% |
| Class F | 5.5 | 5.5 | -0.42% | 39.4 | 39.7 | -0.60% | 60.7 | 79.5 | -34.01% |
| TGM | 12.3 | 12.3 | -0.54% | 112.6 | 113.4 | -0.71% | 142.4 | 143.5 | -0.75% |
| Overall | 2.2 | 2.2 | -0.15% | 2.3 | 1.5 | -40.21% | 2.6 | 0.6 | -75.11% |
| Enc Time[%] | 99% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 99% |  |  | #NUM! |  |  | #NUM! |  |  |

(pending full results)

# Spec Changes

The following reflects our changes to VTM-6.0 syntax, where changes are highlighted with yellow.

#### Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| QState = 0 |  |  |
| … |  |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |  |
| if( dep_quant_enabled_flag && !cu_transquant_bypass_flag ) |  |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |  |
| … |  |  |
| if(AbsLevel[ xC ][ yC ] > 0 ) |  |  |
| firstSigScanPosSb = n |  |  |
| if( dep_quant_enabled_flag && !cu_transquant_bypass_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| … |  |
| coeff_sign_flag[ n ] |  |
| } |  |
| if( dep_quant_enabled_flag && !cu_transquant_bypass_flag ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| … |  |

# Conclusions

This contribution proposes to disable DQ related coefficient coding for lossless blocks. DQ can still be enabled at the slice level so it may be used for lossy blocks in a mixed lossy and lossless setting. This approach shows bit savings on top of [1] in a fully lossless setting.

It is recommended to adopt the proposed method in VVC.

# References

- AHG18 VTM-6.0-lossless, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, June 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).