| 2Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0258-v6 |
| --- | --- |

| Title: | AHG18: Disabling Dependent Quantization for Lossless Blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Alican Nalci, Yung-Hsuan Chao, Hilmi E. Egilmez, and Muhammed Coban 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Proposal

In VTM-6.0, trans quant bypass (QB) mode provides an early lossless alternative in VVC. Even though quantization and transform stages are bypassed in QB mode, residual coding still uses dependent quantization (DQ) and Q-state transitions in regular (non-TS) residual coding. We propose to disable DQ in QB mode such that Q-state is always 0 for lossless blocks.

Note that the changes are in residual coding, thus slice level DQ can still be 1 such that in a mixed lossy and lossless scenario, DQ will be used for blocks that are lossy coded, and DQ will not be used for lossless blocks.

Moreover, we present additional results for the proposals in JVET-P0082 after disabling DQ. These results are as follows:

- Variant #1: This proposal (JVET-P0258) combined with JVET-P0082 Proposal #1. In which BDPCM is enabled for both transform skip (TS) residual coding and regular transform (TR) coding for luma.

- Variant #2: This proposal combined with JVET-P0082 Proposal #3. In which BDPCM is enabled only for TS residual coding. TS and TR residual coding can still be used without BDPCM as in VTM-6.0.

- Variant #3: This proposal combined with JVET-P0082 Proposal #4. In this case BDPCM is enabled for both transform skip (TS) residual coding and regular transform (TR) residual coding for luma. Additionally, BDPCM is enabled for TR residual coding for chroma.

- Variant #4: This proposal combined with JVET-P0082 Proposal #5. In this case BDPCM is enabled for both transform skip (TS) residual coding and regular transform (TR) residual coding for luma. Additionally, BDPCM is enabled for both TR and TS residual coding for chroma.

# Results

We run our anchor on the VTM-6.0-lossless branch [1]. The following settings are used to run both the anchor and our proposal: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. Moreover, QP value was also set as 0. For Additional results, relevant configurations are listed in JVET-P0082.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  |
| Class A1 | 2.2 | 2.2 | -0.13% | 2.3 | 2.3 | -0.15% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | -0.07% |  |  |  |
| Class B | 2.1 | 2.1 | -0.17% | 2.3 | 2.3 | -0.28% | 2.3 | 2.3 | -0.30% |
| Class C | 2.1 | 2.1 | -0.24% | 2.6 | 2.6 | -0.44% | 2.6 | 2.6 | -0.46% |
| Class D | 1.9 | 2.0 | -0.26% | 2.8 | 2.8 | -0.50% | 2.7 | 2.8 | -0.50% |
| Class E | 2.9 | 2.9 | -0.19% |  |  |  | 3.3 | 3.3 | -0.46% |
| Class F | 5.5 | 5.5 | -0.42% | 39.4 | 39.7 | -0.60% | 60.7 | 61.1 | -0.65% |
| TGM | 12.3 | 12.3 | -0.54% | 112.6 | 113.4 | -0.71% | 142.4 | 143.5 | -0.75% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -0.15% | 2.3 | 2.3 | -0.25% | 2.6 | 2.7 | -0.39% |
| Overall | 4.0 | 4.0 | -0.25% | 25.1 | 25.2 | -0.41% | 35.6 | 35.9 | -0.51% |
| Enc Time[%] | 99% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  | 96% |  |  |

Table 1. Results after disabling DQ for lossless.

Additional Results combined with JVET-P0082 Proposal #1 (Variant 1):

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Variant 1 |  | AHG18 LL | Variant 1 |  | AHG18 LL | Variant 1 |  |
| Class A1 | 2.2 | 2.3 | -0.96% | 2.3 | 2.3 | -0.48% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.19% | 1.9 | 1.9 | -0.33% |  |  |  |
| Class B | 2.1 | 2.1 | -1.50% | 2.3 | 2.3 | -0.53% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -1.67% | 2.6 | 2.6 | -0.76% | 2.6 | 2.6 | -0.82% |
| Class D | 1.9 | 2.0 | -2.88% | 2.8 | 2.8 | -1.00% | 2.7 | 2.8 | -0.95% |
| Class E | 2.9 | 3.0 | -2.17% |  |  |  | 3.3 | 3.4 | -1.44% |
| Class F | 5.5 | 5.8 | -4.13% | 39.4 | 41.5 | -3.20% | 60.7 | 84.2 | -37.47% |
| TGM | 12.3 | 12.8 | -3.97% | 112.6 | 116.4 | -3.23% | 142.4 | 123.4 | -37.01% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -1.51% | 2.3 | 2.3 | -0.54% | 2.6 | 1.7 | -42.30% |
| Overall | 4.0 | 4.2 | -2.37% | 25.1 | 26.0 | -1.40% | 35.6 | 27.3 | -30.62% |
| Enc Time[%] | 112% |  |  | 108% |  |  | #NUM! |  |  |
| Dec Time[%] | 91% |  |  | 98% |  |  | #NUM! |  |  |

Table 2. Additional results after disabling DQ for lossless and enabling BDPCM for both TR and TS residual coding.

(pending LDB results)

Additional Results combined with JVET-P0082 Proposal #3 (Variant 2):

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Variant 2 |  | AHG18 LL | Variant 2 |  | AHG18 LL | Variant 2 |  |
| Class A1 | 2.2 | 2.2 | -0.42% | 2.3 | 2.3 | -0.28% |  |  |  |
| Class A2 | 1.7 | 1.8 | -0.30% | 1.9 | 1.9 | -0.20% |  |  |  |
| Class B | 2.1 | 2.1 | -0.41% | 2.3 | 2.3 | -0.35% | 2.3 | 2.3 | -150.23% |
| Class C | 2.1 | 2.1 | -0.81% | 2.6 | 2.6 | -0.62% | 2.6 | 2.6 | -0.67% |
| Class D | 1.9 | 2.0 | -1.73% | 2.8 | 2.8 | -0.81% | 2.7 | 2.8 | -0.82% |
| Class E | 2.9 | 3.0 | -1.17% |  |  |  | 3.3 | 3.4 | -0.88% |
| Class F | 5.5 | 5.7 | -3.42% | 39.4 | 41.3 | -2.94% | 60.7 | 84.3 | -37.44% |
| TGM | 12.3 | 12.7 | -3.58% | 112.6 | 116.2 | -3.09% | 142.4 | 147.1 | -3.11% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -0.61% | 2.3 | 2.3 | -0.38% | 2.6 | 2.1 | -25.48% |
| Overall | 4.0 | 4.1 | -1.53% | 25.1 | 25.9 | -1.22% | 35.6 | 36.6 | -18.08% |
| Enc Time[%] | 104% |  |  | 105% |  |  | #NUM! |  |  |
| Dec Time[%] | 93% |  |  | 96% |  |  | #NUM! |  |  |

Table 3. Additional results after disabling DQ for lossless and enabling BDPCM for TS residual coding.

(pending LDB results)

Additional Results combined with JVET-P0082 Proposal #4 (Variant 3):

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Variant 3 |  | AHG18 LL | Variant 3 |  | AHG18 LL | Variant 3 |  |
| Class A1 | 2.2 | 2.3 | -1.77% | 2.3 | 2.3 | -0.82% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.65% | 1.9 | 1.9 | -0.37% |  |  |  |
| Class B | 2.1 | 2.1 | -2.13% | 2.3 | 2.3 | -0.66% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -2.43% | 2.6 | 2.6 | -0.92% | 2.6 | 2.6 | -0.94% |
| Class D | 1.9 | 2.0 | -4.02% | 2.8 | 2.8 | -1.22% | 2.7 | 2.8 | -1.12% |
| Class E | 2.9 | 3.1 | -3.94% |  |  |  | 3.3 | 3.4 | -1.59% |
| Class F | 5.5 | 5.9 | -5.68% | 39.4 | 42.0 | -3.96% | 60.7 | 84.9 | -38.07% |
| TGM | 12.3 | 13.3 | -7.04% | 112.6 | 117.6 | -4.17% | 142.4 | 148.1 | -3.83% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -2.36% | 2.3 | 2.3 | -0.70% | 2.6 | 1.7 | -42.38% |
| Overall | 4.0 | 4.2 | -3.65% | 25.1 | 26.2 | -1.78% | 35.6 | 36.6 | -26.77% |
| Enc Time[%] | 114% |  |  | 109% |  |  | #NUM! |  |  |
| Dec Time[%] | 86% |  |  | 98% |  |  | #NUM! |  |  |

Table 4. Additional results after disabling DQ for lossless and enabling BDPCM for both TR and TS residual coding. We also enable chroma BDPCM with TR residual coding.

(pending LDB results)

Additional Results combined with JVET-P0082 Proposal #5 (Variant 4):

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Variant 4 |  | AHG18 LL | Variant 4 |  | AHG18 LL | Variant 4 |  |
| Class A1 | 2.2 | 2.3 | -1.74% | 2.3 | 2.3 | -0.80% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.64% | 1.9 | 1.9 | -0.36% |  |  |  |
| Class B | 2.1 | 2.1 | -2.14% | 2.3 | 2.3 | -0.66% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -2.45% | 2.6 | 2.6 | -0.95% | 2.6 | 2.6 | -0.97% |
| Class D | 1.9 | 2.0 | -4.00% | 2.8 | 2.8 | -1.21% | 2.7 | 2.8 | -1.10% |
| Class E | 2.9 | 3.1 | -3.96% |  |  |  | 3.3 | 3.4 | -1.58% |
| Class F | 5.5 | 5.9 | -5.70% | 39.4 | 42.1 | -4.06% | 60.7 | 85.2 | -38.19% |
| TGM | 12.3 | 13.4 | -7.91% | 112.6 | 118.0 | -4.54% | 142.4 | 125.6 | -105.18% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -2.36% | 2.3 | 2.3 | -0.71% | 2.6 | 1.7 | -42.38% |
| Overall | 4.0 | 4.3 | -3.76% | 25.1 | 26.3 | -1.85% | 35.6 | 22.4 | -34.91% |
| Enc Time[%] | 125% |  |  | 117% |  |  | #NUM! |  |  |
| Dec Time[%] | 87% |  |  | 98% |  |  | #NUM! |  |  |

Table 5. Additional results after disabling DQ for lossless and enabling BDPCM for both TR and TS residual coding. We also enable chroma BDPCM with both TR and TS residual coding.

(pending LDB results)

# Spec Changes

The following reflects our changes to VTM-6.0 syntax, where changes are highlighted with yellow.

#### Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| QState = 0 |  |  |
| … |  |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |  |
| if( dep_quant_enabled_flag && !cu_transquant_bypass_flag ) |  |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |  |
| … |  |  |
| if(AbsLevel[ xC ][ yC ] > 0 ) |  |  |
| firstSigScanPosSb = n |  |  |
| if( dep_quant_enabled_flag && !cu_transquant_bypass_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| … |  |
| coeff_sign_flag[ n ] |  |
| } |  |
| if( dep_quant_enabled_flag && !cu_transquant_bypass_flag ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| … |  |

# Conclusions

This contribution proposes to disable DQ related coefficient coding for lossless blocks. DQ can still be enabled at the slice level so it may be used for lossy blocks in a mixed lossy and lossless setting. This approach shows bit savings in a lossless setting.

Furthermore, disabling DQ appears to work well on top of proposal JVET-P0082 in which BDPCM is enabled for lossless mode.

It is recommended to adopt the proposed method and one of the variants discussed here in VVC.

# References

- AHG18 VTM-6.0-lossless, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, June 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).