| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0266 |
| --- | --- |

| Title: | Non-CE6: LFNST signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@canon.com.au jonathan.gan@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

VVC uses a CU-level signalling of LFNST, sent after the residual when last positions of the TBs indicate all significant coefficients are subject to secondary transformation. When chroma separate tree (CST) is disabled in I slices, and in P or B slices, this results in the signalling applying both to luma and chroma TBs of a CU. This contribution proposes separate luma and chroma control of LFNST when CST is not in use. Coding performance results of this are AI: x.xx%, x.xx%, x.xx%, in Y, Cb, and Cr components, respectively.

# Introduction

VVC WD6 [1] includes the LFNST tool, with signalling via lfnst_idx at the residual footer, i.e., after coding of the residual for a CU.  For an intra slice with CST enabled, the luma and chroma coding trees have independent LFNST indices signalled.  However, when CST is disabled, each CU spans the three colour channels and a single LFNST index is signalled for all TBs of the CU. Moreover, conditions to disable LFNST in block(s) of one channel type result in disablement in block(s) of the other channel type.

The selected LFNST kernel for a given CB is a function of the LFNST index, the intra prediction mode of the CB, and the size of the CB (with luma and chroma differing for non-4:4:4 chroma formats).

# Proposal

Given that intra prediction mode may differ for luma and chroma CBs in a CU resulting from a single coding tree, this contribution proposes to also allow the LFNST index to differ for luma and chroma CBs by signalling an additional LFNST index for intra-predicted chroma CBs when single coding trees are in use.

The first signalled LFNST index is used just for the luma CB. Constraints on signalling enablement are confined to the respective channel type.

# Coding performance

Table 1 shows coding performance results for this proposal, with measurements made under the JVET CTC [2] with CST disabled.

Table 1. Coding performance measurement

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (no CST) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.16% | -1.14% | -1.19% | 308% | 106% |
| Class C | 0.31% | -0.27% | -0.63% | 313% | 109% |
| Class E | 0.28% | -0.35% | -0.58% | 302% | 107% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.31% | -0.26% | -0.61% | 318% | 117% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Gothenburg, SE, July, 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March, 2019.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).