| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0271-v1 |
| --- | --- |

| Title: | Non-CE6: Context modeling for LFNST index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a simplified context modeling for LFNST index coding by removing treeType -dependent context assignments to reduce the number of context variables; The first bin of LFNST index is context-coded with single context variable and the second bin is bypassed.

It is reported that proposal without retrained cabac initial value for LFNST index reduces 1 context variable (2 to 1) for LFNST index coding with average BD-rate differences of 0.00% and 0.01% for AI and RA, respectively.

It is also reported that proposal with retrained cabac initial value for LFNST index provides average BD-rate differences of 0.00% and 0.00% for AI and RA, respectively

Introduction

There is a mismatch on the context modeling for LFNST index coding between VVC WD [1] and VTM SW [2], which was reported as ticket#492; In VVC WD, 1st bin of LFNST index is context-coded with two context variables based on treeType and MTS index ( tu_mts_idx[ x0 ][ y0 ]  = =  0  &&  treeType != SINGLE_TREE ), and 2nd bin is bypassed as shown in Table 1-1. However, in VTM SW [2], the first part of the condition (tu_mts_idx[ x0 ][ y0 ]  = =  0) is omitted because it is always true as per adoption of JVET-O0368 (LFNST is only used if the MTS index is equal to 0).

Table 1-1: Context assignments for LFNST index

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| VVC WD: lfnst_idx[ ][ ] | ( tu_mts_idx[ x0 ][ y0 ] = = 0 && treeType != SINGLE_TREE ) ? 1 : 0 | bypass | na | na | na | na |
| VTM SW: lfnst_idx[ ][ ] | ( treeType != SINGLE_TREE ) ? 1 : 0 | bypass | na | na | na | na |

In addition, following cabac intial values are used for LFNST index.

const CtxSet ContextSetCfg::LFNSTIdx = ContextSetCfg::addCtxSet

({

{  45,  37, },

{  38,  45, },

{ CNU,  45, },

{   8,   8, },

});

Proposal

To reduce the number of context variables, it is proposed to further remove treeType-dependent context assignments. Thus, proposal reduces 1 context variable (2 to 1) and makes context derivation cleaner and simpler.

Table 2-1 summarizes proposed context assignments for LNFST index.

Table 2-1: Context assignments for LFNST index

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| Proposal: lfnst_idx[ ][ ] | 0 | bypass | na | na | na | na |

Simulation Results

Proposal has been implemented on top of VTM-6.1 and following simulations were conducted under common test condition [3]:

- Test1:	VTM-6.1 vs Proposal

- Test2: VTM-6.1 vs Proposal with retrained cabac initial value*

*: retrained cabac initial values for LFNST idx are obtained using a method described in JVET-N1021[4].

For Test1, following cabac initial value for LFNST index is used:

const CtxSet ContextSetCfg::LFNSTIdx = ContextSetCfg::addCtxSet

({

{  45,  37, },

{  38,  45, },

{ CNU,  45, },

{   8,   8, },

});

For Test2, following retrained cabac initial value for LFNST index is used:

const CtxSet ContextSetCfg::LFNSTIdx = ContextSetCfg::addCtxSet

({

{  38, },

{  38, },

{  45, },

{   8, },

});

Table 3-1 to 3-2 summarizes coding performance of Test1 and Test2, respectively.

Table 3-1: Results of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.07% | -0.10% | 101% | 100% |
| Class A2 | 0.03% | 0.09% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.07% | -0.13% | 101% | 100% |
| Class C | 0.01% | 0.08% | -0.12% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | -0.09% | 101% | 100% |
| Class D | -0.02% | -0.03% | 0.00% | 101% | 100% |
| Class F | 0.01% | 0.00% | -0.03% | 100% | 99% |

Table 3-2: Results of Test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.00% | 101% | 100% |
| Class A2 | 0.02% | 0.04% | -0.05% | 101% | 98% |
| Class B | -0.01% | 0.09% | -0.05% | 101% | 100% |
| Class C | 0.00% | 0.10% | -0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | -0.03% | 101% | 100% |
| Class D | -0.05% | 0.04% | -0.05% | 101% | 101% |
| Class F | -0.02% | 0.00% | 0.01% | 100% | 100% |

Conclusion

It is recommended to adopt proposed context modeling for LFNST index coding into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “	Versatile Video Coding (Draft 6), ” JVET-O2001, July 2019

[2] VTM-6.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.1

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

[4] F. Bossen, J. Dong, H. Kirchhoffer, “Description of Core Experiment 1 (CE1): CABAC Initialization,” JVET-N1021, Apr. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Spec Changes

Proposed changes relative to JVET-O2001v14 are highlighted in yellow.

| Table 9-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| coding_unit( ) | … |  |  |  |  |  |
|  | lfnst_idx[ ][ ] | Table 9-45 | 0..1 | 1 2..3 | 2 4..5 |  |

Table 9-45 – Specification of initValue and shiftIdx for ctxIdx of lfnst_idx

| Initialization variable | ctxIdx of lfnst_idx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | EP | EP | EP | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 | 0 | 0 | 0 |

| Table 9-82 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … |  |  |  |  |  |  |
| lfnst_idx[ ][ ] | 0 ( tu_mts_idx[ x0 ][ y0 ] = = 0 && treeType != SINGLE_TREE ) ? 1 : 0 | bypass | na | na | na | na |