| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0272-v1 |
| --- | --- |

| Title: | On Scaling for Transform Skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

VVC has two scaling processes in inverse transform skip as “tsShift” and “bdShift”, which are jointly conducted as “iTransformShift (= bdShift – tsShift = maxLog2DynamicRange – bitDepth – (log2(nTbW) + log2(nTbH))/2)” in SW. However, considering concept of transform skip, there should be no scaling process in transform skip. This contribution proposes to remove scaling process in transform skip by combining scaling (tsShift, bdShift in 8.7.2) for transform skip into scaling (bdShift in 8.7.3) in inverse quantization process.

It is reported that:

For ClassA1/A2/B/C/D/E sequences, average BD-rate differences are 0.00%, 0.00% and 0.00% for AI, RA and LB, respectively.

For ClassF sequences, average BD-rate differences are 0.00%, 0.00% and 0.00% for AI, RA and LB, respectively.

For SCC sequences, average BD-rate differences are 0.00%, 0.00% and -0.04% for AI, RA and LB, respectively.

Introduction

Dequantization and Inverse Transform Skip process in VVC WD is summarized as below, and a block diagram is illustrated in Figure 1-1.

A) Dequantization of quantized coefficient TransCoeffLevel and clipping (8.7.3)

d[x][y] = clip3(minCoef, maxCoef, (invQScale* TransCoeffLevel[x][y] + invQOffset )>> invQShift ),

where

- invQScale = m * levelScale[ rectNonTsFlag ] [ qP’ % 6 ]) << (qP’ / 6),

- qP’ = qP + (dep_quant_enabled_flag ? 1:0)

- invQShift = bitDepth + (( rectNonTsFlag ? 1 : 0 ) + ( Log2( nTbW) + Log2 (nTbH))/2 – 5 + dep_quant_enabled_flag,

- invQOffset = (1<<invQShift)>>1

- m = 16,

- levelScale = { { 40,45,51,57,64,72 }, { 57,64,72,80,90,102 }}

- rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 
              transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

B) Derivation of residual samples r, i.e., Inverse transform skip of the input scaled transform coefficients d ( (8-945) in 8.7.2))

r[x][y] = d[x][y] << tsShift,

where tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )

C) Derivation of intermediate residual samples res

res[x][y] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift,         - (*)

where bdShift = Max( 20 − bitDepth, 0 )

Figure 1-1: Block diagram of Dequantization and Inverse Transform in VVC

As can be seen, current VVC has two scaling process in inverse transform skip as “tsShift” and “bdShift”, which are jointly conducted as “iTransformShift (= bdShift – tsShift = maxLog2DynamicRange – bitDepth – (log2(nTbW) + log2(nTbH))/2)” in SW.

However, considering concept of transform skip, there should be no scaling process in transform skip; The above scaling processes seem not to fit into the concept of transform skip.

Looking at scaling in Dequantization, there are two same parameters “bitDepth” and “(Log2(nTbW) + log2(nTbH))/2” between “invQShift” and “bdShift - tsShift”. Scaling factor “tsShift” in B) and “bdShift” in C) can be combined into the scaling factor “invQShift” in A); Modified invQShift’ becomes “invQShift – trShift + bdShift”. Thus, scaling (tsShift and bdShift) in inverse transform skip can be removed.

Proposal

It is proposed to remove scaling process in inverse transform skip by combining scaling factor “tsShift” in B) and “bdShift” in C) into the scaling factor “invQShift” in A). Modified Dequantization and Inverse Transform Skip process is as below, and a block diagram is illustrated in Figure 2-1.

A’) Dequantization of quantized coefficient TransCoeffLevel and clipping (8.7.3)

d[x][y] = clip3(minCoef, maxCoef, (invQScale* TransCoeffLevel[x][y] + invQOffset )>> invQShift ),

where

- invQScale = m * levelScale[ rectNonTsFlag ] [ qP’ % 6 ]) << (qP’ / 6),

- qP’ = qP + (dep_quant_enabled_flag ? 1:0)

- invQShift = (transform_skip_flag && cIdx==0 ? 10 : bitDepth + ( rectNonTsFlag ? 1 : 0 ) + ( Log2( nTbW) + Log2 (nTbH))/2 – 5 ) + dep_quant_enabled_flag,

- invQOffset = (1<<invQShift)>>1

- m = 16,

- levelScale = { { 40,45,51,57,64,72 }, { 57,64,72,80,90,102 }}

- rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 
              transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

B’) Derivation of residual samples r, i.e., Inverse transform skip of the input scaled transform coefficients d ( (8-945) in 8.7.2))

r[x][y] = d[x][y] << tsShift,

where tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )

C’) Derivation of intermediate residual samples res

res[x][y] =d[x][y] ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift,

where bdShift = Max( 20 − bitDepth, 0 )

Figure 2-1: Block diagram of Dequantization and Inverse Transform in Proposal

Simulation Results

Proposal has been implemented on top of VTM-6.1 and following simulations were conducted under common test condition [3]:

- Test1:	VTM-6.1 vs Proposal under common QP condition with SCC sequences

Table 3-1 summarizes coding performance of Test1.

Table 3-1: Results of Test1 (common QP condition)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class SCC | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.02% | 100% | 101% |
| Class SCC | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | -0.03% | -0.05% | 100% | 100% |
| Class SCC | -0.04% | -0.07% | -0.07% | 101% | 101% |

Conclusion

This contribution proposes to remove scaling process in transform skip by combining scaling for transform skip (tsShift, bdShift in 8.7.2) into scaling (bdShift in 8.7.3) in inverse quantization process. Simulation results show that proposed method has negligible impacts to coding performance. It is recommended to adopt it into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “	Versatile Video Coding (Draft 6), ” JVET-O2001, July 2019

[2] VTM-6.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.1

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Proposed Spec Changes

Changes relative to JVET-O2001v14 is as below:

Proposed changes are highlighted in yellow. Removals are striked through in red.

- 8.7.2  Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-942)

bdShift = Max( 20 − bitDepth, 0 )		(8-943)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )		(8-944)

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * slice_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to codedCIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - If transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or cIdx is not equal to 0, following applies.

    - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-945)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

    - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

res [ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-946)

Othwerwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0), following applies:

    - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

res [ x ][ y ] = d[ x ][ y ]	(8-94X)

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If cIdx is equal to codedCIdx, the following applies:

resSamples[ x ][ y ] = res[ x ][ y ]	(8-947)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

resSamples[ x ][ y ] = cSign * res[ x ][ y ]	(8-948)

- Otherwise, the following applies:

resSamples[ x ][ y ] = ( cSign * res[ x ][ y ] ) >> 1	(8-949)

- 8.7.3 Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0 and transform_skip_flag[ xTbY ][ yTbY ] is equal to 0, the following applies:

qP = Qp′Y		(8-950)

- Otherwise, if cIdx is equal to 0 (and transform_skip_flag[ xTbY ][ yTbY ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, Qp′Y )		(8-951)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr		(8-952)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-953)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(8-954)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-955)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = (transform_skip_flag[ xTbY ][ yTbY ]==1 && cIdx==0 ? 10 : 
bitDepth + ( ( rectNonTsFlag  ?  1  :  0 ) + 		( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 ) + dep_quant_enabled_flag	(8-956)

bdOffset = ( 1 << bdShift ) >> 1		(8-957)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 80, 90, 102 } } with j = 0..1, k = 0..5.

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is  derived as follows:

–	If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ] is equal to 1.

–	Otherwise, the following applies:

m[ x ][ y ] = ScalingFactor[ Log2( nTbW ) ][ Log2( nTbH ) ][ matrixId ][ x ][ y ], 
			with matrixId as specified in Table 7-5		(8-958)

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-959)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(8-960)

- When BdpcmFlag[ xTbY ][ yYbY ] is equal to 1, dz[ x ][ y ] is modified as follows:

–	If BdpcmDir[ xTbY ][ yYbY ] is equal to 0 and x is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x − 1 ][ y ] + dz[ x ][ y ])	(8-961)

–	Otherwise, if BdpcmDir[ xTbY ][ yYbY ] is equal to 1 and y is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x ][ y − 1 ] + dz[ x ][ y ])	(8-962)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = ( dz[ x ][ y ] * ls[ x ][ y ] +bdOffset )  >>  bdShift	(8-963)

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-964)