| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0274 |
| --- | --- |

| Title: | Non-CE1: Enabling TMVP in RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsui-Shan Chang Yu-Chen Sun Ling Zhu Jian Lou | Tel: Email: | +86-186-6702-0363 tsuishan.cts@alibaba-inc.com yuchen.s@alibaba-inc.com qianling.zl@alibaba-inc.com j.lou@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd |  |  |

_____________________________

- Abstract

In this proposal, TMVP is enabled for AMVP and SbTMVP by performing MV scaling when the current and collocated pictures have different sizes. The simulation results are summarized as follows:

- Y/Cb/Cr BD rate w/ PSNR1 and scaling ration 2.0x: -0.16%/-0.07%/-0.23%

- Y/Cb/Cr BD rate w/ PSNR2 and scaling ration 2.0x: -0.13%/-0.07%/-0.24%

- Y/Cb/Cr BD rate w/ PSNR1 and scaling ration 1.5x: -0.08%/-0.06%/0.26%

- Y/Cb/Cr BD rate w/ PSNR2 and scaling ration 1.5x: -0.07%/-0.08%/0.27%

Introduction

At the 15th JVET meeting in Gothenburg, reference picture resampling (RPR) (JVET-O1164) is adopted to support adaptive resolution change (ARC) in VVC. The adopted items of JVET-O1164 are quoted as follows:

- max picture size signaling in SPS,

- actual picture size and conformance window signalling moved to PPS,

- scaling ratio is defined based on the output picture size after cropping,

- resampling is done on a block basis using existed interpolation filters,

- TMVP is used only when current and collocated pictures have the same size,

- DMVR and BDOF are used only when current and reference pictures, selected in a CU for inter prediction, have the same size.

In the current design, TMVP is disabled when the resolutions of the current picture and the collocated picture are different. That means temporal MVs cannot be used in merge mode, AMVP, and SbTMVP in this case. In this proposal, we propose to additionally enable TMVP for AMVP and SbTMVP when current and collocated pictures have different sizes. To achieve this, MV scaling is applied according to the scaling ratio between the current picture and the collocated picture.

Proposal

In this proposal, TMVP is enabled for AMVP and SbTMVP even when the resolutions of the current picture and the collocated picture are different. When the current and collocated pictures have different sizes, MV scaling is applied as follows:

mvLXCol[ 0 ] = Clip3( -131072, 131072, ( hori_scale_fp * mvLXCol[ 0 ] + ( 1 << 13 ) – ( hori_scale_fp  * mvLXCol[ 0 ] >= 0 ) ) >> 14 )

mvLXCol[ 1 ] = Clip3( -131072, 131072, ( vert_scale_fp * mvLXCol[ 1 ] + ( 1 << 13 ) – ( vert_scale_fp * mvLXCol[ 1 ] >= 0 ) ) >> 14 )

where hori_scale_fp and vert_scale_fp represent the scaling factors, and mvLXCol denotes the temporal motion vector predictor.

Simulation result

The proposed method was implemented on top of VTM-6.1, and the simulation was conducted under the test condition of CE1, which is described in JVET-O2021.

Table 1. Results with PSNR1 and scaling ratio 2.0x

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | 0.14% | -0.01% | 100% | 103% |
| Class C | -0.20% | -0.29% | -0.15% | 99% | 96% |
| Class E | -0.14% | -0.12% | -0.68% | 100% | 94% |
| Overall | -0.16% | -0.07% | -0.23% | 100% | 99% |
| Class D | -0.08% | -0.27% | -0.02% | 101% | 99% |
| Class F | -0.38% | -0.25% | -0.45% | 100% | 102% |

Table 2. Results with PSNR2 and scaling ratio 2.0x

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | 0.08% | -0.02% | 100% | 103% |
| Class C | -0.11% | -0.26% | -0.09% | 99% | 96% |
| Class E | -0.12% | -0.07% | -0.80% | 100% | 94% |
| Overall | -0.13% | -0.07% | -0.24% | 100% | 99% |
| Class D | -0.06% | -0.17% | 0.05% | 101% | 99% |
| Class F | -0.36% | -0.24% | -0.13% | 100% | 102% |

Table 3. Results with PSNR1 and scaling ratio 1.5x

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.09% | 0.23% | 101% | 102% |
| Class C | -0.06% | -0.29% | -0.10% | 99% | 98% |
| Class E | -0.05% | 0.27% | 0.79% | 100% | 99% |
| Overall | -0.08% | -0.06% | 0.26% | 100% | 100% |
| Class D | -0.16% | 0.36% | 0.30% | 101% | 102% |
| Class F | 0.10% | 0.07% | 0.16% | 98% | 99% |

Table 4. Results with PSNR2 and scaling ratio 1.5x

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.07% | 0.28% | 101% | 102% |
| Class C | -0.10% | -0.36% | -0.10% | 99% | 98% |
| Class E | 0.03% | 0.26% | 0.76% | 100% | 99% |
| Overall | -0.07% | -0.08% | 0.27% | 100% | 100% |
| Class D | -0.16% | 0.57% | 0.42% | 101% | 102% |
| Class F | 0.10% | 0.23% | 0.12% | 98% | 99% |

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).