| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0276-v1 |
| --- | --- |

| Title: | Non-CE6:On Supporting 64x64 Chroma Transform Unit with Composite VPDU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | WentingCai, Jianqing Zhu, Jie Yao, KimihikoKazui 601,Yongda International Tower, 2277Longyang Road Shanghai, 201204, China | Email: | caiwenting@cn.fujitsu.com zhujianqing@cn.fujitsu.com yaojie@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution proposes to support the maximum transform unit size of chroma components in 4:2:0 format up to 64x64 by introducing composite VPDU concept. For CTUs coded in dual tree in I slices, a flag is used to indicate whether a 64x64 chroma block is split into 4 32x32 blocks or not. If a 64x64 chroma block is not split, i.e. chroma CU size is 64x64, its TU size is also 64x64. CCLM is disabled for 64x64 chroma CU. For inter CU, when inter CU size in luma samples is larger than 64x64 (i.e. 128x128, 128x64 or 64x128 ), the size of transform unit of chroma components is up to 64x64. Otherwise, the maximum transform unit size of chroma is 32x32. By sharing transform core and internal buffer between luma TU and chroma TU, there is no extra hardware cost on decoder pipeline except scheduling mechanic (controller) of pipeline. The simulation results show -0.01%(Y), -0.20%(Cb), and -0.29%(Cr)BD-rates for AI configuration and -0.00%(Y), -0.17%(Cb), and -0.22%(Cr)BD-rates for RA configuration respectively on top of VTM-6.0.

Introduction

The concept of VPDU was introduced in JVET-K0556 and embodied in implicit quald-tree split syntax where 64x64 luma samples and 32x32 chroma samples are coded interleaved in dual trees for intra slices. The adopted proposal JVET-K0230 mentioned the motivation of VPDU. It is to keep buffer size small in hardware decoder. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time. The VPDU size is roughly proportional to the buffer size in most pipeline stages. In most hardware decoders, the VPDU size can be set to maximum transform block (TB) size. In VTM-6.0, the maximum luma TB size is 64x64 and the maximum chroma TB size is 32x32.

This proposal presents a composite VPDU to make maximum chroma TB size be 64x64 without increase of buffer size in hardware decoder.

Proposed method

For CTUs coded in dual tree in I slices, when cbSize in luma is 128x128, use a flag to indicate whether a 64x64 chroma block is split into 4 32x32 blocks or not in dual_tree_implicit_qt_split(). This flag will not affect luma implicit qt split. If a 64x64 chroma block is not split, i.e. chroma CU size is 64x64, its TU size is also 64x64. The partitioning and data unit (DU) order in intra CTU is as Figure 1 and Figure 2.

CCLM is disabled for 64x64 chroma CU.

Figure 1 Partitioning of 128x128 luma /64x64 chroma DU in I slices

Figure 2 Processing order of for 128x128 intra DU

In P and B slices, when inter CU size in luma samples is larger or equal to 128x128, 128x64 or 64x128, the size of transform unit of chroma components is up to 64x64. Otherwise, the maximum transform unit size of chroma is 32x32. The TU partitioning and its order in inter CTU is as Figure 3 and Figure 4.

Figure 3 transform partition of 128x128/64x128/128x64 inter CU

Figure 4 TU processing order for 128x128/64x128/128x64 inter CU

Composite VPDU Structure

There was concern on breaking the 64x64 VPDU concept in case of 4:2:0. By analysis, if pipeline is designed appropriately, there is no break on 64x64 VPDU concept and no buffer size increase.

Take the example of pipelined video decoder architecture in Figure 5.

The classic pipeline processing timing is as Figure 6 when VPDU contains 64x64 Luma samples and 32x32 Chroma samples. Tctu is time to process a CTU.T0 is time to process a VPDU. Ci includes Cbi and Cri (i=0-3). The latency of each pipeline stage is T0. The internal buffer for each pipeline stage need cover a 64x64 luma block and 2 32x32 chroma blocks.

Figure 5 An example of pipelined video decoder architecture

Figure 6 classic pipeline processing timing of video decoder

If introducing 64x64 chroma TU, the pipeline timing will be break as Figure 7.

Figure 7 decoder pipeline break happens when introducing 64x64 chroma TU

However, if we adjust the latency of each pipeline stage as Figure 8, the break issue is solved. As in Figure 8, a composite VPDU structure is used. The latency of each pipeline stage is adjusted to T1 rather than T0. T1 is the time to process a 64x64 DU. T0 is 1.5 times T1.

VPDU_Type_0 contains 64x64 Luma samples and 32x32 Chroma samples. VPDU_Type_1 contains 64x64 samples no matter Luma or Chroma samples. The internal buffer for each pipeline stage still need cover a 64x64 luma block and 2 32x32 chroma blocks. For VPDU_Type_1, 64x64 luma block and chroma block share internal buffer. So, there is no increase on buffer size. 64x64 transform core is also shared between luma and chroma blocks.

Figure 8 pipeline processing timing of video decoder for composite VPDU

VPDU type 2 is corresponding to 128x64 inter CU in above and 64x128 inter CU in left part and the other part is normal case (chroma TU less or equal to 32x32).

VPDU type 3 is corresponding to 128x64 inter CU in bottom and 64x128 inter CU in right part and the other part is normal case (chroma TU less or equal to 32x32).

Experimental results

The proposed scheme is implemented on top of VTM-6.0 software [2].The simulations were carried out under the JVET common test conditions[3].

The test results for JVET common test conditions are tabulated in Table 1 and Table 2.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.32% | -0.32% | 106% | 99% |
| Class A2 | -0.01% | -0.20% | -0.19% | 104% | 98% |
| Class B | 0.00% | -0.19% | -0.27% | 102% | 99% |
| Class C | 0.01% | -0.07% | 0.00% | 106% | 99% |
| Class E | -0.05% | -0.29% | -0.75% | 113% | 99% |
| Overall | -0.01% | -0.20% | -0.29% | 106% | 99% |
| Class D | 0.01% | 0.03% | 0.04% | 112% | 98% |
| Class F | -0.03% | -0.25% | -0.30% | 107% | 98% |

Table 1.Experimental results for All-Intra (AI)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.47% | -0.59% | 103% | 101% |
| Class A2 | 0.02% | -0.13% | -0.10% | 104% | 98% |
| Class B | -0.01% | -0.18% | -0.20% | 101% | 101% |
| Class C | 0.01% | 0.04% | -0.08% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.17% | -0.22% | 102% | 100% |
| Class D | -0.01% | -0.02% | -0.18% | 92% | 100% |
| Class F | -0.02% | -0.41% | 0.08% | 102% | 101% |

Table 2.Experimental results for random-access (RA)

Conclusions

In this contribution, the maximum transform unit size of chroma for I slices and P, B slices is extended to 64x64 which is the same as that of luma. By sharing transform core and internal buffer between luma and chroma TU, there is no extra hardware cost on decoder pipeline except scheduling mechanic of pipeline. Appling this technique brings benefit on coding efficiency, especially for higher resolution sequences.

References

- J. Chen, Y. Ye and S. H. Kim“Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6),”JVET-O2002, Gothenburg, SE, July. 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- JVET-K0230 CE1-related: Separate tree partitioning at 64x64-luma/32x32-chroma unit level [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

- JVET-K0556 CE1-related: Constraint for binary and ternary partitions [C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Patent rights declaration(s)

FUJITSU LIMITEDmay have current or pendingpatent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).