| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0282 |
| --- | --- |

| Title: | Non-CE4: Block size alignment between PROF and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the previous meeting, prediction refinement with optical flow (PROF) for affine mode was adopted. Both PROF and bi-directional optical flow (BDOF) is applied to refine subblock based prediction. In VVC, block size constraint between PROF and BDOF is different. Therefore, in this contribution, block size aligning methods for affine mode using PROF are proposed for to align with BDOF.

Method I: Block size restriction on PROF

Method II: Block size restriction on AFFINE MERGE

Method III: Block size restriction on AFFINE INTER and AFFINE MERGE

The experimental results for Method I reportedly show 0.01% (xxx%) BD-rate changes with 100% (xxx %) and 101% (xxx %) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. The experimental results for Method II reportedly show 0.02% (xxx %) BD-rate changes with 100% (xxx %) and 101% (xxx %) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. Finally, the experimental results for Method III reportedly show -0.04% (xxx %) BD-rate changes with 103% (xxx %) and 101% (xxx %) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. Also, additional test on Method III is performed. In additional test, encoder optimization for 8xN, Nx8 blocks is applied by control the lambda in comparing the cost. The experimental results for Method III & encoder optimization reportedly show -0.xx% (-0.xx%) BD-rate changes with xxx% (xxx%) and xxx% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations

Introduction

BDOF process is performed when all of the following conditions are satisfied as described in VVC Draft [2]. Especially, BDOF process is restricted when the block width is less than 8 or block height is less than 8 or block size is 8x8 to reduce the number of multiplication in worst case.

- If all of the following conditions are true, bdofFlag is set equal to TRUE. - sps_bdof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0. - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1. - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0. - MotionModelIdc[ xCb ][ yCb ] is equal to 0. - merge_subblock_flag[ xCb ][ yCb ] is equal to 0. - sym_mvd_flag[ xCb ][ yCb ] is equal to 0. - ciip_flag[ xCb ][ yCb ] is equal to 0. - BcwIdx[ xCb ][ yCb ] is equal to 0. - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0. - cbWidth is greater than or equal to 8. - cbHeight is greater than or equal to 8. - cbHeight * cbWidth is greater than or equal to 128. - For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively. - cIdx is equal to 0.

Proposed method

In this contribution, block size constraint methods for affine mode using PROF are proposed to align with BDOF. The proposed methods are described in Table 1.

Method I: Block size restriction on PROF

The block size restriction on PROF process is applied. In here, size restriction is same with BDOF and block size for AFFINE INTER/MERGE mode is not modified.

Method II: Block size restriction on AFFINE MERGE

The block size restriction on AFFINE MERGE is applied. By using the same constraint with BDOF, both AFFINE MERGE and PROF is not performed when the block size is 8x8.

Method III: Block size restriction on AFFINE INTER and AFFINE MERGE

The block size restriction for AFFINE INTER and AFFINE MERGE is aligned. By Method 3, both AFFINE INTER and AFFINE MERGE have same condition. It means that 8xN and Nx8 (N != 8) blocks for AFFINE INTER is added and 8x8 blocks for AFFINE MERGE is excepted. Automatically, PROF is performed at same size with BDOF.

Table 1 Proposed Method I, II and III

|  |  | AFFINE INTER | AFFINE MERGE | PROF | BDOF |
| --- | --- | --- | --- | --- | --- |
| VTM6.0 |  | w>=16 && h>=16 | w>=8 && h>=8 | - | w>=8 && h>=8 && w*h>=128 |
| Proposal | Method I | w>=16 && h>=16 | w>=8 && h>=8 | w*h>=128 | w>=8 && h>=8 && w*h>=128 |
|  | Method II | w>=16 && h>=16 | w>=8 && h>=8 && w*h>=128 | - | w>=8 && h>=8 && w*h>=128 |
|  | Method III | w>=8 && h>=8 && w*h>=128 | w>=8 && h>=8 && w*h>=128 | - | w>=8 && h>=8 && w*h>=128 |

Proposed VVC Draft changes

Method I: Block size restriction on PROF

In Method I, PROF conditions is modified as follows.

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE. - sps_affine_prof_enabled_flag is equal to 0. - fallbackModeTriggered is equal to 1. - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - cbHeight * cbWidth is greater than or equal to 128. - Otherwise, cbProfFlagLX set equal to TRUE.

Method II: Block size restriction on AFFINE MERGE

In Method II, merge data syntax for AFFINE MERGE is modified as follows.

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 && (cbWidth * cbHeight) >= 128 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| …… |  |
| } |  |

Method III: Block size restriction on AFFINE INTER and AFFINE MERGE

In Method III, merge data syntax for AFFINE MERGE is modified same as Method II and coding unit syntax for AFFINE INTER is modified as follows.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 cbWidth >= 8 && cbHeight >= 8 && (cbWidth * cbHeight) >= 128) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …… |  |
| } |  |

Experimental results

The proposed method has been implemented on top of the VTM6.0 with default setting and tested using JVET SDR common test condition [3]. The experimental results for Method I reportedly show 0.01% (-0.xx%) BD-rate changes with 100% (xxx%) and 101% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. The experimental results for Method II reportedly show 0.02% (-xxx%) BD-rate changes with 100% (xxx%) and 101% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. Finally, the experimental results for Method III reportedly show -0.04% (xxx%) BD-rate changes with 103% (xxx%) and 101% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations.

Method I: Block size restriction on PROF

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | 0.01% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.05% | 0.01% | 0.04% | 101% | 101% |  |  |  |  |  |
| Class B | -0.01% | 0.05% | 0.00% | 100% | 102% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.02% | 0.04% | 0.06% | 100% | 101% | -0.03% | -0.14% | 0.10% | 100% | 105% |
| Class E |  |  |  |  |  | 0.00% | 0.59% | 0.88% | 100% | 104% |
| Overall | 0.01% | 0.05% | 0.03% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.03% | 0.03% | 101% | 103% | 0.01% | 0.16% | -0.13% | 100% | 102% |
| Class F | 0.01% | 0.00% | 0.00% | 101% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Method II: Block size restriction on AFFINE MERGE

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.03% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.02% | 0.01% | 0.02% | 101% | 101% |  |  |  |  |  |
| Class B | 0.01% | 0.11% | 0.11% | 100% | 102% | #VALUE! | #VALUE! | #VALUE! | #NUM! | 105% |
| Class C | 0.02% | 0.04% | 0.12% | 99% | 101% | -0.04% | 0.13% | 0.06% | 99% | 105% |
| Class E |  |  |  |  |  | -0.12% | 0.38% | 0.08% | 100% | 105% |
| Overall | 0.02% | 0.05% | 0.06% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | 105% |
| Class D | 0.00% | -0.04% | 0.03% | 100% | 102% | 0.05% | 0.33% | 0.13% | 99% | 103% |
| Class F | 0.01% | 0.01% | 0.04% | 101% | 103% | #VALUE! | #VALUE! | #VALUE! | #NUM! | 103% |

Method III: Block size restriction on AFFINE INTER and AFFINE MERGE

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.03% | 102% | 100% |  |  |  |  |  |
| Class A2 | 0.01% | -0.02% | 0.01% | 103% | 101% |  |  |  |  |  |
| Class B | -0.04% | 0.04% | -0.02% | 103% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.11% | -0.05% | -0.02% | 104% | 101% | -0.11% | 0.05% | -0.04% | 107% | 104% |
| Class E |  |  |  |  |  | -0.03% | 0.47% | 0.21% | 105% | 104% |
| Overall | -0.04% | -0.01% | -0.02% | 103% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.20% | -0.03% | -0.11% | 105% | 103% | -0.15% | -0.04% | 0.24% | 106% | 102% |
| Class F | -0.11% | -0.09% | 0.01% | 103% | 102% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Additionally, simple encoder optimization is applied on Method III. In the current affine mode decision process, affine block is selected when cost of the affine block is lower than cost of the normal block * lambda, where value of lambda is 1.05. In this proposal, the value of lambda is just controlled to 1.25. The experimental results for Method III using simple encoder optimization reportedly show -0.xx% (-0.xx%) BD-rate changes with xxx% (xxx%) and xxx% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations.

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class B | -0.06% | -0.06% | -0.06% | 101% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.11% | -0.02% | 0.03% | 100% | 97% | -0.09% | -0.11% | -0.03% | 106% | 99% |
| Class E |  |  |  |  |  | 0.02% | 0.76% | 0.50% | 104% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.21% | -0.02% | 0.00% | 99% | 92% | -0.18% | -0.33% | 0.29% | 105% | 100% |
| Class F | -0.11% | -0.11% | -0.06% | 99% | 98% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

In this contribution, block size aligning methods for affine mode using PROF are proposed for to align with BDOF. In Method I, block size restriction on PROF process is applied. In Method II, block size restriction on AFFINE MERGE is applied. Finally, in Method III, block size restriction for AFFINE INTER and AFFINE MERGE is aligned. The experimental results reportedly show minor changes in coding performance and run-time compared to VTM6.0 as anchor in CTC [3]. It is recommended to consider this method in the next VTM.

Reference

- J. Luo, et al., “CE4-2.1: Prediction refinement with optical flow for affine mode”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0070, 15th Meeting: Gothenburg, SE, 3–12 July. 2019.

- B. Bross, et al., “Versatile Video Coding (Draft 6)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-MO2001, 15th Meeting: Gothenburg, SE, 3–12 July. 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).