| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0284 |
| --- | --- |

| Title: | Non-CE8: Modified DM mode selection under IBC mode using left and above block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang Dong Jiang Jucai Lin Feiyang Zeng | Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua(ZHEJIANG DAHUA TECHNOLOGY CO.,LTD) |  |  |

_____________________________

- Abstract

This contribution proposes to change the DM mode of current chroma block from DC to the intra prediction mode of the left or above neighbouring intra block when the current luma block applies IBC mode while current chroma block applies intra mode. Compared to the VTM-6.0 anchors, the proposed method reportedly leads to {Y, U, V} BD-rate variations of { %,  %,  %} for AI configuration.

Introduction

In the VTM-6.0[1], when the current luma block applies IBC mode and current chroma block applies intra mode, the default DM mode is set to DC.

Proposal

This contribution is designed under the condition that the current luma block applies IBC mode and current chroma block applies intra prediction mode. The description of the proposed method is as follows :

1. If current block’s width<height,

i. if the left-neighbour block applies intra mode, and the intra mode of the left block is mode a. Then the DM mode of current chroma block is set to a.

ii. else if the left-neighbour block do not apply intra mode, while the above-neighbour block applies intra mode. And the intra mode of the above block is mode b. Then the DM mode of current chroma block is set to b.

iii. otherwise if the left and above neighbouring blocks do not apply intra mode neither, the default process will be applied.

2. If current block’s width>height,

i. if the above-neighbour block applies intra mode, and the intra mode of the above block is mode a. Then the DM mode of current chroma block is set to a.

ii. else if the above-neighbour block do not apply intra mode, while the left-neighbour block applies intra mode. And the intra mode of the left block is mode b. Then the DM mode of current chroma block is set to b.

iii. otherwise if the left and above neighbouring blocks do not apply intra mode neither, the default process will be applied.

The positions of the left and above neighbouring block are shown in Figure 1, where L stands for the left-neighbour block, and A stands for the above-neighbour block.

Figure 1

Experimental results

The proposed modification is implemented on VTM-6.0 reference software. The simulations are basically performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-N1010 [2], but configure parameters HashME and BDPCM are equal to 0, IBC is equal to 1. Apart from the test sequences of JVET-N1010, Table 1 also illustrated the simulation results of the proposed method for the four 1080P test sequences in TGM420 (i.e. Class SCC).

  - BD-rate performance of the proposed Method

Table1. Performance of the proposed method

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.12% | -0.01% |  |  |
| Class C | 0.00% | -0.04% | 0.08% |  |  |
| Class E | 0.00% | 0.03% | 0.11% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.02% | 0.00% |  |  |
| Class F | -0.01% | 0.04% | 0.00% |  |  |
| Class SCC | -0.12% | -0.11% | -0.10% |  |  |

Conclusions

This contribution proposes to change the DM mode of current chroma block from DC to the intra prediction mode of the left or above neighbouring intra block in certain condition. According to the experimental results, the average BD-rate gains are  %,  %,  % luminance and both chrominance components.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).