| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0285 |
| --- | --- |

| Title: | Non-CE4:Intra prediction mode of CIIP selection using left adjacent coded block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feiyang Zeng Dong Jiang Jucai Lin Cheng Fang | Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua(ZHEJIANG DAHUA TECHNOLOGY CO.,LTD) |  |  |

_____________________________

- Abstract

This contribution proposes to replace the default planar mode with the intra prediction mode of the neighbor coded block when performaning the intra prediction in CIIP mode. Compared to the VTM-6.0 anchors, the method reportedly leads to {Y, U, V} BD-rate variations of {-0.01%, -0.01%, -0.01%} for RA configuration.

Introduction

In the VTM-6.0[1], the default intra mode of CIIP is planar mode.

Proposal

In the CIIP mode, if the left adjacent coded block exists and intra prediction is employed, the intra prediction mode of the left adjacent coded block is used as the intra prediction mode of the current block. Otherwise, the intra prediction mode of the current block is still planar mode.

Experimental results

The proposed modification is implemented on VTM-6.0 reference software. The simulation is performed on Linux cluster using VTM RA configurations under the common test condition described in JVET-N1010 [2].

Table1. Performance of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.02% | -0.01% |  |  |
| Class C | -0.02% | -0.03% | 0.04% |  |  |
| Class E | 0.00% | 0.02% | -0.05% |  |  |
| Overall | -0.01% | -0.01% | -0.01% |  |  |
| Class D | 0.03% | -0.02% | 0.01% |  |  |
| Class F |  |  |  |  |  |

Conclusions

This contribution presents a CIIP intra prediction mode replacement method. Combining the results between VTM-6.0 and the proposed method, the average BD-rate can gains 0.01%, 0.01%, 0.01% for luminance and both chrominance components.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).