| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0287 |
| --- | --- |

| Title: | Non-CE4:Modefied MVD derivation method for bidirectional MMVD using the magnitude of MV |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Jiang Feiyang Zeng Jucai Lin Cheng Fang | Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua(ZHEJIANG DAHUA TECHNOLOGY CO.,LTD) |  |  |

_____________________________

- Abstract

In this contribution, a MVD derivation method for bidirectional MMVD is introduced by comparing the magnitude of the MV. The MVD with the smaller MV amplitude is derived from the MVD with the larger MV amplitude. Compared to the VTM-6.0 anchors, the method reportedly leads to {Y, U, V} BD-rate variations of {-0.06%, -0.07%, -0.11%} for RA configuration.

Introduction

In the bidirectional prediction MMVD of VTM6.0[1], if POC distance of the L0 is larger or equal to POC distance of the L1, the L1 MVD is derived with the scaled or mirrored L0 MVD[2]. Otherwise, L0 MVD is derived with the scaled or mirrored L1 MVD.

Figure 1 shows the MVD derivation process considering the POC difference and reference picture type in MMVD scheme.

Figure 1. MVD derivation process of the MMVD

Proposal

In the bidirectional prediction MMVD, if the magnitude of the MV0 is larger or equal to the magnitude of the MV1, the L1 MVD is derived with the scaled or mirrored L0 MVD. Otherwise, L0 MVD is derived with the scaled or mirrored L1 MVD.

Figure 2 shows the MVD derivation process considering the magnitude of the MVX and reference picture type. In here, MAG_MVX means the magnitude of the MVX. If the MAG_MV0 >= MAG_MV1,   ScaleFactor1 = MAG_MV1/ MAG_MV0. Otherwise, ScaleFactor1 = MAG_MV0/ MAG_MV1.

Figure 2. MVD derivation process of the Method One

Experimental results

The proposed modification is implemented on VTM-6.0 reference software. The simulation is performed on Linux cluster using VTM RA configurations under the common test condition described in JVET-N1010 [3].

Table1. Performance of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.02% | 0.05% |  |  |
| Class C | 0.00% | -0.13% | -0.21% |  |  |
| Class E | -0.26% | -0.08% | -0.16% |  |  |
| Overall | -0.06% | -0.07% | -0. 11% |  |  |
| Class D | 0.35% | 0.05% | 0.04% |  |  |
| Class F |  |  |  |  |  |

Conclusions

This contribution presents a MVD derivation method for bidirectional MMVD. Combining the results between VTM-6.0 and the proposed methods, the average BD-rate can gains 0.06%, 0.07%, 0.11% for luminance and both chrominance components.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- JVET-O0415, “Non-CE4 : Fix of POC distance condition for MMVD”, Naeri Park, Junghak Nam, Hyeongmun Jang, Jaehyun Lim, 15th Meeting: Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).