| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0290 |
| --- | --- |

| Title: | CE5-related: Clipping of intermediate value in CC-ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | +81 50-5123-6336 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes an additional clipping on intermediate value of cross-component adaptive loop filter (CC-ALF) in CE5-2.1. The proposed method performs a clip operation on delta value which is added to source image by CC-ALF in order to reduce the storage size. Experimental results reportedly shows average BD-rates (Y,U,V) of (0.13%, -7.58% -6.00%), (0.17%, -10.68%, -10.03%), and (XX %, XX %, XX %) for AI, RA, and LDB cases, respectively. Relative to CE5-2.1, the BD-rates  (Y,U,V) change is reportedly (0.00%, 0.01%, 0.01%), (0.01%, 0.01%, 0.05%), (XX%) in AI, RA and LDB respectively.

Introduction

CE5-2.1 CC-ALF [1][2] calculates a delta value for a chroma sample from the corresponded luma samples, and the delta value is added to adjust the chroma sample. However, in the case the delta value is processed in the other processing pipeline (e.g. luma ALF filtering stage), the intermediate delta values need to be stored. Therefore, it is desirable to put a limitation on the range of the delta values in order to reduce the size of storage.

Proposed method (WD changes relative to CE5.2.1)

The proposed method adds a clip operation for a delta value, which is added to source image. The clip range is determined according to the internal bit-depth.

  - The variable sum is derived as follows:

sum = 	f[ 0 ] * recPictureL[ hx, vy + yM2 ] +
	f[ 1 ] * recPictureL[ hx+xM1, vy + yM1 ] +
	f[ 2 ] * recPictureL[ hx, vy + yM1 ] +
	f[ 3 ] * recPictureL[ hx+xP1, vy + yM1 ] +
	f[ 4 ] * recPictureL[ hx+xM2, vy ] +
	f[ 5 ] * recPictureL[ hx+xM1, vy ] +
	f[ 6 ] * recPictureL[ hx, vy ] +
	f[ 7 ] * recPictureL[ hx+xP1, vy ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy ] +		(8-1289)
	f[ 4 ] * recPictureL[ hx+xM2, vy+yP1 ] +
	f[ 8 ] * recPictureL[ hx+xM1, vy+yP1 ] +
	f[ 9 ] * recPictureL[ hx, vy+yP1 ] +
	f[ 10 ] * recPictureL[ hx+xP1, vy+yP1 ] +
	f[ 4 ] * recPictureL[ hx+xP2, vy+yP1 ] +
	f[ 11 ] * recPictureL[ hx+xM1, vy + yP2 ] +
	f[ 12 ] * recPictureL[ hx, vy + yP2 ] +
	f[ 13 ] * recPictureL[ hx+xP1, vy + yP2 ] +
	f[ 0 ] * recPictureL[ hx, vy + yP3 ] +

sum = curr + ( sum + 64 ) >> 7 )		(8-1290)

range = 1<<(BitDepthC − 4)

sum = curr + Clip3( − range , range − 1, ( sum + 64 ) >> 7 ) )					(8-1290)

The position of the clip operation is shows in Fig. 1

Clip

Clip

Fig. 1: (a) Placement of clip operation with respect to CC-ALF

Experimental results

The proposed method has been implemented based on CE-5.2.1 CC-ALF software and tested according to the common test condition defined in JVET-N1010.

Table 1: Experimental results (VTM-6 anchor)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -3.49% | -4.16% | 103% | 104% |
| Class A2 | 0.13% | -8.65% | -8.91% | 102% | 104% |
| Class B | 0.10% | -10.56% | -8.57% | 101% | 104% |
| Class C | 0.13% | -4.17% | -3.90% | 102% | 103% |
| Class E | 0.19% | -10.17% | -3.44% | 102% | 104% |
| Overall | 0.13% | -7.58% | -6.00% | 102% | 104% |
| Class D | 0.11% | -2.74% | -3.27% | 101% | 102% |
| Class F | 0.18% | -6.56% | -6.63% | 101% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -5.69% | -5.36% | 102% | 102% |
| Class A2 | 0.22% | -11.61% | -15.64% | 101% | 103% |
| Class B | 0.21% | -17.64% | -14.05% | 101% | 102% |
| Class C | 0.09% | -5.03% | -4.29% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -10.68% | -10.03% | 101% | 102% |
| Class D | 0.10% | -4.46% | -4.47% | 100% | 101% |
| Class F | 0.19% | -4.74% | -4.61% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.11% | -8.25% | -6.74% | 101% | 101% |
| Class E | 0.22% | -15.75% | -6.29% | 103% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.19% | -8.17% | -6.00% | 101% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2: Experimental results (CE5-2.1 anchor)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5-2.1 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.03% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Class F | 0.00% | 0.02% | 0.01% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over CE5-2.1 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.13% | 0.12% | 100% | 100% |
| Class B | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.05% | 100% | 100% |
| Class D | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.01% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over CE5-2.1 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.08% | -0.31% | 100% | 100% |
| Class E | -0.01% | -0.11% | 0.09% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | -0.07% | 0.05% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

References

[1] K. Misra, F. Bossen, A. Segall, “Cross-Component Adaptive Loop Filter for chroma,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O0636, July 2019

[2] K. Misra, F. Bossen, A. Segall, “CE5-2.1, CE5-2.2: Cross Component Adaptive Loop Filter”, JVET-P0080, October 2019

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).