| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0293-v2 |
| --- | --- |

| Title: | AHG15: Signaling scaling matrix for LFNST case |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Tomohiro Ikai | Tel: Email: | tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In VVC-6, scaling matrices (i.e. spatial frequency weighting) are applied even in the case of LFNST coefficients, which does not correspond to conventional transform coefficients. This contribution proposes a fix for this problem by introducing LFNST specific scaling matrices, which are signalled in APS. The proposed method provides transform coefficient control capability in the LFNST transform case as well as the conventional case.

Introduction

In VVC-6, when scaling list is enabled, scaling lists are specified by matrixId according to sizeId, prediction mode and colour component as show in Table 1.

Table 1.  Specification of matrixId according to sizeId, prediction mode and colour component in VVC-6

| sizeId | CuPredMode | cIdx (Colour component) | matrixId |
| --- | --- | --- | --- |
| 2, 3, 4, 5, 6 | MODE_INTRA | 0 (Y) | 0 |
| 1, 2, 3, 4, 5, 6 | MODE_INTRA | 1 (Cb) | 1 |
| 1, 2, 3, 4, 5, 6 | MODE_INTRA | 2 (Cr) | 2 |
| 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 0 (Y) | 3 |
| 1, 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 1 (Cb) | 4 |
| 1, 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 2 (Cr) | 5 |

In previous meeting, we proposed a scheme in JVET-O0383 [1] by using 2nd scaling after LFNST, but the 2nd scaling’s complexity was mentioned in the review. This contribution proposes another option which signals LFNST specific scaling matrix. The LFNST specific scaling matrix is applied in the same way as the current scaling matrix, thus this proposal supports transform coefficient control capability without 2nd scaling.

Proposal

We propose to signal LFNST specific matrix by extending matrixId from 0..5 to 0..8 as shown in Table 2.

Table 2.  Specification of matrixId according to sizeId, prediction mode, colour component and lfnstIdx in proposed scheme

| sizeId | CuPredMode | cIdx (Colour component) | lfnstIdx | matrixId |
| --- | --- | --- | --- | --- |
| 2, 3, 4, 5, 6 | MODE_INTRA | 0 (Y) | 0 | 0 |
| 1, 2, 3, 4, 5, 6 | MODE_INTRA | 1 (Cb) | 0 | 1 |
| 1, 2, 3, 4, 5, 6 | MODE_INTRA | 2 (Cr) | 0 | 2 |
| 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 0 (Y) | 0 | 3 |
| 1, 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 1 (Cb) | 0 | 4 |
| 1, 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 2 (Cr) | 0 | 5 |
| 2, 3 | MODE_INTRA | 0 (Y) | 1,2 | 6 |
| 2, 3 | MODE_INTRA | 1 (Cb) | 1,2 | 7 |
| 2, 3 | MODE_INTRA | 2 (Cr) | 1,2 | 8 |

- If lfnstIdx is equal to 0, scaling matrix with matrixId in the range of 0 to 5 is applied to conventional transform coefficients.

- Otherwise (if lfnstIdx is not equal to 0), scaling matrix with matrixId in the range of 6 to 8 is applied to LFNST’s coefficients.

Because LFNST uses two set of LFNST kernel, one for 4xN and Nx4 blocks, the other for other blocks, we signal scaling matrix with sizeId being 2 and 3 only for LFNST case, which is applied for 4xN and Nx4 blocks and other blocks respectively.

When LFNST is applied, the number of coefficients is 16 regardless of block size. Therefore, we signal only 16 weighting values for LFNST case (matrixId > 5)

| lfnst_scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( sizeId = 2; sizeId < 4; sizeId++ ) |  |
| for( matrixId = 6; matrixId < 9; matrixId ++ ) { |  |
| scaling_list_pred_mode_flag[ sizeId ][ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ sizeId ][ matrixId ] ) |  |
| scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] | ue(v) |
| else { |  |
| coefNum = 16 |  |
| nextCoef = 8 |  |
| for( i = 0; i < coefNum; i++ ) { |  |
| x = DiagScanOrder[ 2 ][ 2 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 2 ][ 2 ][ i ][ 1 ] |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ sizeId ][ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

This proposal defines slice_lfnst_scaling_list_aps_id in slice header for signaling newly scaling matrices. Scaling matrices for LFNST are signaled in APS as same as conventional scaling matrices.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| slice_scaling_list_present_flag | u(1) |
| if( slice_scaling_list_present_flag ) |  |
| slice_scaling_list_aps_id | u(3) |
| if( sps_lfnst_enabled_flag ) |  |
| slice_lfnst_scaling_list_aps_id | u(3) |
| } |  |
| … |  |

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = LFNST_SCALING_APS ) |  |
| lfnst_scaling_list_data() |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The proposed method performs scaling process with scaling matrices for LFNST coefficients as well as the conventional case reasonably. In addition, because proposed method does not change the process related to transform, quantization and scaling, no extra processes will not happen.

Experimental Results

The proposed method is implemented on top of VTM software (version 7977eb8e). The simulations are conducted in the condition of ‘All Intra’, ‘Random Access’ with scalingLists.txt as attached in this contribution.

We’ve checked subjective quality. Figure 2 shows a comparison of decoded images. As shown in this figure, red circle area, where LFNST is applied, are clearer in proposal than in VTM-6 with scaling matrix.

Figure 2: Comparison of decoded images between proposal and VTM-6

Conclusion

In this contribution, we propose to signal scaling matrix for LFNST case. The proposed method provides transform coefficient control capability in all conventional and LFNST transform cases. It is recommended to adopt this method in the next VVC WD and VTM software.

Reference

- T. Hashimoto, E. Sasaki, T. Chujoh, T. Ikai, “Non-CE7: Harmonization of scaling matrix and LFNST,” JVET-O0383, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 15th Meeting: Gothenburg, SE, 3–12 July 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).