| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET- P0293_WD |
| --- | --- |

| Title: | AHG15: Signaling scaling matrix for LFNST case |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6277 tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft based on JVET-O2001-vE. The changes are highlighted in yellow.

…

- 7.3 Syntax in tabular form

### 7.3.1 NAL unit syntax

#### 7.3.2.5 Adaptation parameter set syntax

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = LFNST_SCALING_APS ) |  |
| lfnst_scaling_list_data() |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.16 Scaling list data syntax

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( sizeId = 1; sizeId < 7; sizeId++ ) |  |
| for( matrixId = 0; matrixId < 6; matrixId ++ ) { |  |
| if( ! ( ( ( sizeId = = 1 ) && ( matrixId % 3 = = 0 ) ) ¦ ¦ ( ( sizeId = = 6 ) && ( matrixId % 3 != 0 ) ) ) ) { |  |
| scaling_list_pred_mode_flag[ sizeId ][ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ sizeId ][ matrixId ] ) |  |
| scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] | ue(v) |
| else { |  |
| nextCoef = 8 |  |
| coefNum = Min( 64, ( 1 << ( sizeId << 1 ) ) ) |  |
| if( sizeId > 3 ) { |  |
| scaling_list_dc_coef_minus8[ sizeId − 4 ][ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ sizeId − 4 ][ matrixId ] + 8 |  |
| } |  |
| for( i = 0; i < coefNum; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if ( !( sizeId = = 6 && x >= 4 && y >= 4) ) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ sizeId ][ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| lfnst_scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( sizeId = 2; sizeId < 4; sizeId++ ) |  |
| for( matrixId = 6; matrixId < 9; matrixId ++ ) { |  |
| scaling_list_pred_mode_flag[ sizeId ][ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ sizeId ][ matrixId ] ) |  |
| scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] | ue(v) |
| else { |  |
| coefNum = 16 |  |
| nextCoef = 8 |  |
| for( i = 0; i < coefNum; i++ ) { |  |
| x = DiagScanOrder[ 2 ][ 2 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 2 ][ 2 ][ i ][ 1 ] |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ sizeId ][ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.3.6 Slice header syntax

#### 7.3.6.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| non_reference_picture_flag | u(1) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type = = GDR_NUT ) |  |
| recovery_poc_cnt | ue(v) |
| if( nal_unit_type = = IDR_W_RADL ¦ ¦ nal_unit_type = = IDR_N_LP ¦ ¦ nal_unit_type = = CRA_NUT ¦ ¦ NalUnitType = = GDR_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( slice_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag && !pps_temporal_mvp_enabled_idc ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B && !pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag && !pps_five_minus_max_num_subblock_merge_cand_plus1 ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| slice_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_sign_flag | u(1) |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| if ( !pps_dep_quant_enabled_flag ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| slice_scaling_list_present_flag | u(1) |
| if( slice_scaling_list_present_flag ) |  |
| slice_scaling_list_aps_id | u(3) |
| if( sps_lfnst_enabled_flag ) |  |
| slice_lfnst_scaling_list_aps_id | u(3) |
| } |  |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### 7.4.3.5 Adaptation parameter set semantics

Each APS RBSP shall be available to the decoding process prior to it being referred, included in at least one access unit with TemporalId less than or equal to the TemporalId of the coded slice NAL unit that refers it or provided through external means.

Let aspLayerId be the nuh_layer_id of an APS NAL unit. If the layer with nuh_layer_id equal to aspLayerId is an independent layer (i.e., vps_independent_layer_flag[ GeneralLayerIdx[ aspLayerId ] ] is equal to 1), the APS NAL unit containing the APS RBSP shall have nuh_layer_id equal to the nuh_layer_id of a coded slice NAL unit that referrs it. Otherwise, the APS NAL unit containing the APS RBSP shall have nuh_layer_id either equal to the nuh_layer_id of a coded slice NAL unit that referrs it, or equal to the nuh_layer_id of a direct dependent layer of the layer containing a coded slice NAL unit that referrs it.

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within an access unit shall have the same content.

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS, SCALING_APS or LFNST_SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 7-2. When aps_params_type is equal to 1 (LMCS_APS), the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

Table 7-2 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | LFNST_SCALING_APS | Scaling list parameters for LFNST |
| 4..7 | Reserved | Reserved |

NOTE 1 – Each type of APSs uses a separate value space for adaptation_parameter_set_id.

NOTE 2 – An APS NAL unit (with a particular value of adaptation_parameter_set_id and a particular value of  aps_params_type) can be shared across pictures, and different slices within a picture can refer to different ALF APSs.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the APS RBSP syntax structure. aps_extension_flag equal to 1 specifies that there are aps_extension_data_flag syntax elements present in the APS RBSP syntax structure.

aps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all aps_extension_data_flag syntax elements.

…

#### 7.4.3.16 Scaling list data semantics

scaling_list_pred_mode_flag[ sizeId ][ matrixId ] equal to 0 specifies that the values of the scaling list are the same as the values of a reference scaling list. The reference scaling list is specified by scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ]. scaling_list_pred_mode_flag[ sizeId ][ matrixId ] equal to 1 specifies that the values of the scaling list are explicitly signalled.

scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] specifies the reference scaling list used to derive ScalingList[ sizeId ][ matrixId ], the derivation of ScalingList[ sizeId ][ matrixId ] is based on scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] as follows:

- If scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] is equal to 0, the scaling list is inferred from the default scaling list ScalingList[ sizeId ][ matrixId ][ i ] as specified in Table 7-7, Table 7-8, Table 7-9 and Table 7-10 for i = 0..Min( 63, ( 1  <<  ( sizeId  <<  1 ) ) − 1 ).

- Otherwise, the scaling list is inferred from the reference scaling list as follows:

refMatrixId = matrixId −	
	scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] * ( sizeId  = =  6 ? 3 : 1 ) 	(7-59)

ScalingList[ sizeId ][ matrixId ][ i ] = ScalingList[ sizeId ][ refMatrixId ][ i ]
	with i =0..Min( 63, ( 1  <<  ( sizeId  <<  1 ) ) − 1 )	(7-60)

If sizeId is equal to 1, the value of refMatrixId shall not be equal to 0 or 3. Otherwise, if sizeId is less than or equal to 5, the value of scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] shall be in the range of 0 to matrixId, inclusive. Otherwise (sizeId is equal to 6), the value of scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] shall be in the range of 0 to matrixId / 3, inclusive.

Table 7-5 – Specification of sizeId

| Size of quantization matrix | sizeId |
| --- | --- |
| 1x1 | 0 |
| 2x2 | 1 |
| 4x4 | 2 |
| 8x8 | 3 |
| 16x16 | 4 |
| 32x32 | 5 |
| 64x64 | 6 |

Table 7-6 – Specification of matrixId according to sizeId, prediction mode and colour component

| sizeId | CuPredMode | cIdx (Colour component) | lfnstIdx | matrixId |
| --- | --- | --- | --- | --- |
| 2, 3, 4, 5, 6 | MODE_INTRA | 0 (Y) | 0 | 0 |
| 1, 2, 3, 4, 5, 6 | MODE_INTRA | 1 (Cb) | 0 | 1 |
| 1, 2, 3, 4, 5, 6 | MODE_INTRA | 2 (Cr) | 0 | 2 |
| 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 0 (Y) | 0 | 3 |
| 1, 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 1 (Cb) | 0 | 4 |
| 1, 2, 3, 4, 5, 6 | MODE_INTER, MODE_IBC | 2 (Cr) | 0 | 5 |
| 2, 3 | MODE_INTRA | 0 (Y) | 1,2 | 6 |
| 2, 3 | MODE_INTRA | 1 (Cb) | 1,2 | 7 |
| 2, 3 | MODE_INTRA | 2 (Cr) | 1,2 | 8 |

scaling_list_dc_coef_minus8[ sizeId −4 ][ matrixId ] plus 8 specifies the value of the variable ScalingFactor[ 4 ][ matrixId ][ 0 ][ 0 ] for the scaling list for the 16x16 size when sizeId is equal to 4 and specifies the value of ScalingFactor[ 5 ][ matrixId ][ 0 ][ 0 ] for the scaling list for the 32x32 size when sizeId is equal to 5, and specifies the value of ScalingFactor[ 6 ][ matrixId ][ 0 ][ 0 ] for the scaling list for the 64x64 size when sizeId is equal to 6. The value of scaling_list_dc_coef_minus8[ sizeId −4 ][ matrixId ] shall be in the range of −7 to 247, inclusive.

When scaling_list_pred_mode_flag[ sizeId ][ matrixId ] is equal to 0, scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] is equal to 0 and sizeId is greater than 3, the value of scaling_list_dc_coef_minus8[ sizeId −4 ][ matrixId ] is inferred to be equal to 8.

When scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] is not equal to 0 and sizeId is greater than 3, the value of scaling_list_dc_coef_minus8[ sizeId −4 ][ matrixId ] is inferred to be equal to scaling_list_dc_coef_minus8[ sizeId −4 ][ refMatrixId ], where the value of refMatrixId is given by Equation (7-59).

scaling_list_delta_coef specifies the difference between the current matrix coefficient ScalingList[ sizeId ][ matrixId ][ i ] and the previous matrix coefficient ScalingList[ sizeId ][ matrixId ][ i − 1 ], when scaling_list_pred_mode_flag[ sizeId ][ matrixId ] is equal to 1. The value of scaling_list_delta_coef shall be in the range of −128 to 127, inclusive. The value of ScalingList[ sizeId ][ matrixId ][ i ] shall be greater than 0. When  scaling_list_pred_mode_flag[ sizeId ][ matrixId ] is equal to 1 and scaling_list_delta_coef is not present, the value of ScalingList[ sizeId ][ matrixId ][ i ] is inferred to be 0.

Table 7-7 – Specification of default values of ScalingList[ 1 ][ matrixId ][ i ] with i = 0..3

| i | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| ScalingList[ 1 ][ 1,2,4,5 ][ i ] | 16 | 16 | 16 | 16 |

Table 7-8 – Specification of default values of ScalingList[ 2 ][ matrixId ][ i ] with i = 0..15

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ScalingList[ 2 ][ 0..5 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

Table 7-9 – Specification of default values of ScalingList[ 3..5 ][ matrixId ][ i ] with i = 0..63

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| i − 16 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| i − 32 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| i − 48 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

Table 7-10 – Specification of default values of ScalingList[ 6 ][ matrixId ][ i ] with i = 0..63

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| i − 16 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| i − 32 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| i − 48 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

Table 7-X – Specification of default values of ScalingList[ 2..3 ][ 6..8 ][ i ] with i = 0..15

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ScalingList[ 2..3 ][ 6..8 ][ i ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

The five-dimensional array ScalingFactor[ sizeId ][sizeId][ matrixId ][ x ][ y ], with x, y = 0..( 1  <<  sizeId ) − 1, specifies the array of scaling factors according to the variables sizeId specified in Table 7-5 and matrixId specified in Table 7-5.

The elements of the quantization matrix of size 2x2, ScalingFactor[ 1 ][ 1 ][ matrixId ][ x ][ y ], with i = 0..3, matrixId = 1, 2, 4, 5, are derived as follows:

x = DiagScanOrder[ 1 ][ 1 ][ i ][ 0 ]		(7-61)

y = DiagScanOrder[ 1 ][ 1 ][ i ][ 1 ]		(7-62)

ScalingFactor[ 1 ][ 1 ][ matrixId ][ x ][ y ] = ScalingList[ 1 ][ matrixId ][ i ]	(7-63)

The elements of the quantization matrix of size 4x4, ScalingFactor[ 2 ][ 2 ][ matrixId ][ x ][ y ], with i = 0..15, matrixId = 0..5, are derived as follows:

x = DiagScanOrder[ 2 ][ 2 ][ i ][ 0 ]		(7-64)

y = DiagScanOrder[ 2 ][ 2 ][ i ][ 1 ]		(7-65)

ScalingFactor[ 2 ][ 2 ][ matrixId ][ x ][ y ] = ScalingList[ 2 ][ matrixId ][ i ]	(7-66)

The elements of the quantization matrix of size 4x4, ScalingFactor[ 2 ][ 2 ][ matrixId ][ x ][ y ], with i = 0..15, matrixId = 6..8, are derived as follows:

x = DiagScanOrder[ 2 ][ 2 ][ i ][ 0 ]		(7-X)

y = DiagScanOrder[ 2 ][ 2 ][ i ][ 1 ]		(7-X)

ScalingFactor[ 2 ][ 2 ][ matrixId ][ x ][ y ] = ScalingList[ 2 ][ matrixId ][ i ]	(7-X)

The elements of the quantization matrix of size 8x8, ScalingFactor[ 3 ][ 3 ][ matrixId ][ x ][ y ], with i = 0..63, matrixId = 0..5, are derived as follows:

x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ]		(7-67)

y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ]		(7-68)

ScalingFactor[ 3 ][ 3 ][ matrixId ][ x ][ y ] = ScalingList[ 3 ][ matrixId ][ i ]	(7-69)

The elements of the quantization matrix of size 8x8, ScalingFactor[ 3 ][ 3 ][ matrixId ][ x ][ y ], with i = 0..63, matrixId = 6..8, are derived as follows:

x = DiagScanOrder[ 2 ][ 2 ][ i ][ 0 ]		(7-X)

y = DiagScanOrder[ 2 ][ 2 ][ i ][ 1 ]		(7-X)

ScalingFactor[ 3 ][ 3 ][ matrixId ][ x ][ y ] = i < 16 : ScalingList[ 3 ][ matrixId ][ i ] : 0	(7-X)

The elements of the quantization matrix of size 16x16, ScalingFactor[ 4 ][ 4 ][ matrixId ][ x ][ y ], with i = 0..63, j = 0..1, k = 0..1, matrixId = 0..5, are derived as follows:

x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ]		(7-70)

y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ]		(7-71)

ScalingFactor[ 4 ][ 4 ][ matrixId ][ x * 2 + k ][ y * 2 + j ] = ScalingList[ 4 ][ matrixId ][ i ]	(7-72)

ScalingFactor[ 4 ][ 4 ][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ matrixId ] + 8	(7-73)

The elements of the quantization matrix of size 32x32, ScalingFactor[ 5 ][ 5 ][ matrixId ][ x ][ y ], with i = 0..63, j = 0..3, k = 0..3, matrixId = 0..5, are derived as follows:

x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ]		(7-74)

y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ]		(7-75)

ScalingFactor[ 5 ][ 5 ][ matrixId ][ x * 4 + k ][ y * 4 + j ] = ScalingList[ 5 ][ matrixId ][ i ]	(7-76)

ScalingFactor[ 5 ][ 5 ][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ matrixId ] + 8	(7-77)

The elements of the quantization matrix of size 64x64, ScalingFactor[ 6 ][ 6 ][ matrixId ][ x ][ y ], i = 0..63, j = 0..7, k = 0..7, matrixId = 0, 3, are derived as follows:

x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ]		(7-78)

y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ]		(7-79)

ScalingFactor[ 6 ][ 6 ][ matrixId ][ x * 8 + k ][ y * 8 + j ] = ScalingList[ 6 ][ matrixId ][ i ]	(7-80)

ScalingFactor[ 6 ][ 6 ][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 2 ][ matrixId ] + 8	(7-81)

When ChromaArrayType is equal to 3, the elements of the chroma quantization matrix of size 64x64, ScalingFactor[ 6 ][ 6 ][ matrixId ][ x ][ y ], with i = 0..63, j = 0..7, k = 0..7, matrixId = 1, 2, 4 and 5 are derived as follows:

x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ]		(7-82)

y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ]		(7-83)

ScalingFactor[ 6 ][ 6 ][ matrixId ][ x * 8 + k ][ y * 8 + j ] = ScalingList[ 5 ][ matrixId ][ i ]	(7-84)

ScalingFactor[ 6 ][ 6][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ matrixId ] + 8	(7-85)

For a quantization matrix with rectangular size, the five-dimensional array ScalingFactor[ wId ][hId][ matrixId ][ x ][ y ], with wId = 0..6, hId = 0..6, matrixId = 0..8, x = 0..( 1  <<  wId ) − 1, y = 0..( 1  << hId ) − 1, and wId != hId, specifies the array of scaling factors for size (1 << wId)x(1 << hId). The elements of ScalingFactor[ wId ][hId][ matrixId ][ x ][ y ] with matrixId = 0..5 are generated by using ScalingList[ maxSizeId ][ matrixId ][ i ] with maxSizeId = max( wId, hId ), as follows:

k = min( maxSizeId, 3 ), 
x = DiagScanOrder[ k ][ k ][ i ][ 0 ]
y = DiagScanOrder[ k ][ k ][ i ][ 1 ]
ratioW = ( 1 << wId )  /  ( 1 << k )
ratioH = ( 1 << hId )  /  ( 1 << k )
diffWH = 1 << abs(wId − hId)		(7-86)
if( wId > hId )
	ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] =
		ScalingList[ maxSizeId ][ matrixId ]
			[ Raster2DiagScanPos[ k ][ k ][ ( 1 << k ) * ( ( y * diffWH ) / ratioW ) + x / ratioW ] ]
else( wId < hId )
	ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] =
		ScalingList[ maxSizeId ][ matrixId ]
			[ Raster2DiagScanPos[ k ][ k ][ ( 1 << k ) * ( y / ratioH ) + (x * diffWH) / ratioH ] ]

The elements of ScalingFactor[ wId ][hId][ matrixId ][ x ][ y ] are with matrixId = 6..8 as follows:

k = (wId >=3 && hId >= 3) ? 3 : 2
x = DiagScanOrder[ k ][ k ][ i ][ 0 ]
y = DiagScanOrder[ k ][ k ][ i ][ 1 ]

ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] = 
          ScalingList[ k ][ matrixId ][ Raster2DiagScanPos[ k ][ k ][ ( 4 * y + x ) ] ]

NOTE – A quantization matrix element ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] can be zeroed out when any of the following conditions is true

- x is greater than 32

- y is greater than 32

- The decoded tu is not coded by default transform mode (i.e. transform type is not equal to 0 ) and x is greater than 16

- The decoded tu is not coded by default transform mode (i.e. transform type is not equal to 0 ) and y is greater than 16

- 8.7.2 	Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-942)

bdShift = Max( 20 − bitDepth, 0 )		(8-943)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )		(8-944)

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * slice_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to codedCIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-945)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.5.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

res [ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-946)

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If cIdx is equal to codedCIdx, the following applies:

resSamples[ x ][ y ] = res[ x ][ y ]	(8-947)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

resSamples[ x ][ y ] = cSign * res[ x ][ y ]	(8-948)

- Otherwise, the following applies:

resSamples[ x ][ y ] = ( cSign * res[ x ][ y ] ) >> 1	(8-949)

- 8.7.3	Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0 and transform_skip_flag[ xTbY ][ yTbY ] is equal to 0, the following applies:

qP = Qp′Y		(8-950)

- Otherwise, if cIdx is equal to 0 (and transform_skip_flag[ xTbY ][ yTbY ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, Qp′Y )		(8-951)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr		(8-952)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-953)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(8-954)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-955)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  1  :  0 ) + 		(8-956)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

bdOffset = ( 1 << bdShift ) >> 1		(8-957)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 80, 90, 102 } } with j = 0..1, k = 0..5.

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is derived as follows:

–	If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ] is equal to 1.

–	Otherwise if lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2

- m[ x ][ y ] = ScalingFactor[ log2LfnstSize ][ log2LfnstSize ][ matrixId ][ x ][ y ], 
			with matrixId as specified in Table 7-5

–	Otherwise, the following applies:

m[ x ][ y ] = ScalingFactor[ Log2( nTbW ) ][ Log2( nTbH ) ][ matrixId ][ x ][ y ], 
			with matrixId as specified in Table 7-5		(8-958)

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-959)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(8-960)

- When BdpcmFlag[ xTbY ][ yYbY ] is equal to 1, dz[ x ][ y ] is modified as follows:

–	If BdpcmDir[ xTbY ][ yYbY ] is equal to 0 and x is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x − 1 ][ y ] + dz[ x ][ y ])	(8-961)

–	Otherwise, if BdpcmDir[ xTbY ][ yYbY ] is equal to 1 and y is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x ][ y − 1 ] + dz[ x ][ y ])	(8-962)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = ( dz[ x ][ y ] * ls[ x ][ y ] +bdOffset )  >>  bdShift	(8-963)

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-964)

…

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).