| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0294-v1 |
| --- | --- |

| Title: | Non-CE3 : Simplification of MPM derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Daehyeok Gwon Heeji Han Haechul Choi Hahyun Lee Jungwon Kang | Tel: Email: | +82-42-821-1149 gr141082@hanbat.ac.kr sarhakor@hanbat.ac.kr choihc@hanbat.ac.kr hanilee@etri.re.kr jungwon@etri.re.kr |
| Source: | Hanbat National University(HNU) Electronics and Telecommunications Research Institute(ETRI) |  |  |

_____________________________

- Abstract

This contribution proposes simplification methods for the intra MPM derivation process. The VVC WD 6 [1] has 7 categories of MPM lists based on intra modes of the left and above neighboring blocks. However, MPM lists constructed in two categories are identical. Therefore, one of the two categories is not needed. The first proposed clean-up method unifies the two categories into one category, which provides the coding efficiency 0.00% and 0.00% BD-rate in AI and RA configuration, respectively. At the last meeting, the number of MPM derivation categories increased from 5 to 7 with minor gain. Two additional conditions should also be checked to derive a MPM list at the worst case. To simplify the MPM derivation, the second proposed method reduces the number of categories of MPM lists from 7 to 5. It has the coding performance of 0.02% and 0.01% BD-rate in AI and RA configuration, respectively. The third proposed method harmonizes the first and second methods. This harmonized method has 4 categories of MPM lists and the coding performance of 0.02% and 0.01% BD-rate in AI and RA configuration, respectively.

Introduction

This contribution reports results of simplification methods for the MPM derivation. In current VVC WD 6 [1], 7 categories of MPM lists are used to derive MPMs depending on the intra modes of the left and above neighbouring blocks like Fig. 1. Table 1 and Fig. 2 show the MPM derivation process in VVC WD 6.

Figure 1 : The derivation positions of MPM candidate

Table 1 : Categories of MPM derivation in VVC WD 6

| Categories |  | MPM list |
| --- | --- | --- |
| Both angular and they are the same |  | {Left, Left-1, Left+1, Left-2, Left+2} |
| Both angular and they are different | Max–Min=1 | {Left, Above, Min-1, Max+1, Min-2} |
|  | Max–Min≥62 | {Left, Above, Min+1, Max-1, Min+2} |
|  | Max–Min=2 | {Left, Above, Min+1, Min-1, Max+1} |
|  | Otherwise | {Left, Above, Min-1, Min+1, Max-1} |
| One angular, the other non-angular |  | {Max, Max-1, Max+1, Max-2, Max+2} |
| Both Left and Above are non-angular |  | {DC, VER, HOR, VER-4, VER+4} |

Figure 2 : Flowchart of MPM derivation in VVC WD 6

Proposed Methods

2.1 Method I

The proposed clean-up method unifies two categories of MPM derivation process that generate identical MPM lists. In current VVC WD 6, the two categories derive MPMs as following:

- Both left(A) and above(B) are angular mode and they are the same : { Left, Left-1, Left+1, Left-2, Left+2 }

- One neighbouring mode is angular and the other neighbouring mode is non-angular : { Max, Max-1, Max+1, Max-2, Max+2 }

In the above two categories, Max is equal to Left. Therefore, the two categories generate the identical MPMs. Therefore, one of the two categories is not needed. The proposed method I unifies these two categories into one category. The proposed MPM list is constructed like Table 2 and Fig. 3. The modified category is marked with yellow in Table 2. In this contribution, distinct angular modes mean angular modes that are different each other.

Table 2 : Categories of MPM derivation in proposed method I

| Categories |  | MPM list |
| --- | --- | --- |
| The number of angular modes is equal to 0 (“Both Left and Above are non-angular”) |  | {DC, VER, HOR, VER-4, VER+4} |
| The number of distinct angular modes is equal to 2 (“Both angular and they are different”) | Max–Min=1 | {Left, Above, Min-1, Max+1, Min-2} |
|  | Max–Min≥62 | {Left, Above, Min+1, Max-1, Min+2} |
|  | Max–Min=2 | {Left, Above, Min+1, Min-1, Max+1} |
|  | Otherwise | {Left, Above, Min-1, Min+1, Max-1} |
| The number of distinct angular modes is equal to 1 (“One angular and the other non-angular” or “Both angular and they are the same”) |  | {Max, Max-1, Max+1, Max-2, Max+2} |

Figure 3 : Flowchart of MPM derivation in the proposed method I

2.2 Method II

The simplification of MIP[2] was adopted at the last meeting, it removed the DC mode from MPM lists when at least one of neighbour blocks has angular mode. However, it increased the number of MPM derivation categories from 5 to 7 with minor gain. Two additional conditions should also be checked to derive a MPM list at the worst case. For example, at the worst case, 5 conditions are checked in VVC WD 6, whereas 3 conditions are checked in VVC WD 5[3]. To simply the MPM derivation, the proposed method II reduces the number of categories from 7 to 5 categories. The proposed method checks 3 conditions at the worst case, such as the same number of conditions as VVC WD 5. Categories and MPM lists modified relative to VVC WD 6 are marked with yellow in Table 3. It constructs MPM lists like Table 3 and Fig. 4. The method II does not include the method I. Therefore, the first and forth MPM lists are identical like VVC WD 6.

Table 3 : Categories of MPM derivation in proposed method II

| Categories |  | MPM list |
| --- | --- | --- |
| Both angular and they are the same |  | {Left, Left-1, Left+1, Left-2, Left+2} |
| Both angular and they are different | Max–Min>3 | {Left, Above, Min+1, Max-1, Min+2} |
|  | Otherwise | {Left, Above, Min-1, Max+1, Min-2} |
| One angular, the other non-angular |  | {Max, Max-1, Max+1, Max-2, Max+2} |
| Both Left and Above are non-angular |  | {DC, VER, HOR, VER-4, VER+4} |

Figure 4 : Flowchart of MPM derivation in the proposed method II

2.3 Method III (Method I + Method II)

VVC WD 6 has 7 categories of MPM lists and two categories of them generate identical MPM lists. To further simply MPM derivation, the method III harmonizes the method I and method II like Table 4 and Fig. 5. Modified categories and MPM lists relative to VVC WD 6 are marked as yellow. It has 4 categories of MPM lists and does not have identical MPM lists.

Table 4 : Categories of MPM derivation in method III (Method I + Method II)

| Categories |  | MPM list |
| --- | --- | --- |
| The number of angular modes is equal to 0 (“Both Left and Above are non-angular”) |  | {DC, VER, HOR, VER-4, VER+4} |
| The number of distinct angular modes is equal to 2 (“Both angular and they are different”) | Max–Min>3 | {Left, Above, Min+1, Max-1, Min+2} |
|  | Otherwise | {Left, Above, Min-1, Max+1, Min-2} |
| The number of distinct angular modes is equal to 1 (“One angular, the other non-angular” or “Both angular and they are the same”) |  | {Max, Max-1, Max+1, Max-2, Max+2} |

Figure 5 : Flowchart of MPM derivation in the proposed method III (Method I + Method II)

Experimental Results

The simulations are performed according to common test conditions (CTC) as described in [4]. AI and RA results are reported.

3.1 Method I

Table 5: Results of proposed clean-up method over VTM6.0[5]

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

3.2 Method II

Table 6: Results of proposed simplification method over VTM6.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | 0.06% | 100% | 100% |
| Class A2 | 0.03% | 0.06% | -0.01% | 100% | 100% |
| Class B | 0.02% | 0.05% | 0.02% | 100% | 100% |
| Class C | 0.05% | 0.01% | 0.06% | 100% | 100% |
| Class E | 0.00% | 0.06% | 0.05% | 100% | 100% |
| Overall | 0.02% | 0.05% | 0.03% | 100% | 100% |
| Class D | -0.01% | 0.09% | -0.09% | 100% | 100% |
| Class F | 0.02% | 0.04% | -0.07% | 101% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

3.2 Method III (Method I + Method II)

Table 7: Results of proposed simplification method over VTM6.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | 0.06% | 100% | 100% |
| Class A2 | 0.03% | 0.06% | -0.01% | 100% | 100% |
| Class B | 0.02% | 0.05% | 0.02% | 100% | 100% |
| Class C | 0.05% | 0.01% | 0.06% | 100% | 100% |
| Class E | 0.00% | 0.06% | 0.05% | 100% | 100% |
| Overall | 0.02% | 0.05% | 0.03% | 100% | 100% |
| Class D | -0.01% | 0.09% | -0.09% | 100% | 100% |
| Class F | 0.02% | 0.04% | -0.07% | 101% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | -0.04% | 100% | 100% |
| Class A2 | 0.03% | 0.01% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.10% | 100% | 100% |
| Class C | 0.00% | 0.07% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.04% | 100% | 100% |
| Class D | -0.04% | -0.12% | 0.10% | 100% | 100% |
| Class F | 0.01% | -0.17% | 0.04% | 100% | 100% |

Conclusion

In this contribution, simplification methods of MPM derivation are proposed. The method I unifies two categories of MPM derivation process that generate identical MPMs. As the result, the method I has 6 categories for MPM derivation without any BD-rate changes. The method II reduces the number categories from 7 categories of VVC WD 6 to 5 categories. The method III harmonizes the method I and method II. It is reportedly shown that both the method II and the method III provide overall BD-rate by 0.02%, 0.05%, 0.03% under All Intra (AI) configuration and 0.01%, 0.01%, 0.04% under Random Access (RA) configuration.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2001, July 2019

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, March 2019

- Jonathan Pfaff et al. “Non-CE3: Simplifications of MIP,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O0925, July 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

- VTM 6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

Patent rights declaration(s)

HNU and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).