| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0298-v2 |
| --- | --- |

| Title: | CE7-related: Unification of CCB check method and bypass coding between two residual coding modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to simplify the CCB (Context Coded bins) check method and unify the syntax design after CCB count exceeds the maximum number between transform residual and transform skip (TS) residual. Two items are proposed. In Item1, the CCB check method is changed from per coefficient flag to per coefficient of each pass in VTM6.0. This CCB check method is same as that of CE7-1.3b [2]. In Item2, level mapping is disabled after CCB count on top of CE7-1.3b-alternative method [2]. This Item2 is also same concept of JVET-O0406 [3]. As a modified Item2, bypass sign flag is excluded from the pass of bypass-coded coefficients on Item2. In this item, syntaxes of transform residual and TS residual after CCB count exceeds the maximum number are exactly the same.

Simulation results reportedly are shown as follows:

Item1:

ClassF:  0.02% AI,  0.02% RA,  -0.01% LB  ClassTGM:  0.04% AI,  0.00% RA,  -0.01%LB

Item1 (Low QP):

ClassF:  0.08%AI,  -0.01%RA,  0.03%LB  ClassTGM:  0.10%AI,  0.06%RA,  0.00%LB

Item2

ClassF:  0.01% AI,  0.02% RA,  0.00% LB  ClassTGM:  0.05% AI,  0.04% RA,  -0.06%LB

Item2 (Low QP):

ClassF:  -1.21%AI,  -1.55%RA,  -0.44%LB  ClassTGM:  -1.36%AI,  -2.01%RA,  -0.91%LB

Introduction

In VTM6.0, the transform skip (TS) residual levels of a coefficient group (CG) are coded in the following three passes:

- Pass 1: for (n=0, n <= numSbCoeff – 1, n++)

sig_coeff_flag (CABAC if CCB remains, otherwise bypass)

coeff_sign_flag (same as above)

Gt 1 flag (abs_level_gtx_flag[0]) (same as above)

parity flag (par_level_flag) (same as above)

- Pass 2: for (n=0, n <= numSbCoeff – 1, n++)

Gt 3 flag (abs_level_gtx_flag[1]) (same as above)

Gt 5 flag (abs_level_gtx_flag[2]) (same as above)

Gt 7 flag (abs_level_gtx_flag[3]) (same as above)

Gt 9 flag (abs_level_gtx_flag[4]) (same as above)

- Pass 3: for (n=0, n <= numSbCoeff – 1, n++)

rice = cctx.templateAbsSumTS(n, coeff)

abs_remainder[n]

level mapping

In CE7-1.3b-alternative Item [2], which is same concept of JVET-O0406, the transform skip (TS) residual levels of a coefficient group (CG) are coded in the following three passes:

- Pass 1: for (n=0, n <= numSbCoeff – 1 && CCB>=4, n++)

sig_coeff_flag (CABAC)

coeff_sign_flag (same as above)

Gt 1 flag (abs_level_gtx_flag[0]) (same as above)

parity flag (par_level_flag) (same as above)

FirstPassBypassPos = n

- Pass 2: for (n=0, n <= FirstPassBypassPos && CCB>=4, n++)

Gt 3 flag (abs_level_gtx_flag[1]) (same as above)

Gt 5 flag (abs_level_gtx_flag[2]) (same as above)

Gt 7 flag (abs_level_gtx_flag[3]) (same as above)

Gt 9 flag (abs_level_gtx_flag[4]) (same as above)

SecondPassBypassPos = n

- Pass 3(a): for (n = 0; n < FirstPassBypassPos; n++)

baseLevel = (n >= SecondpassBypassPos ? 2 : 10)

rice = cctx.templateAbsSumTS(n, coeff, baseLevel)

abs_remainder[n]

level mapping

- Pass 3(b): for (n = FirstPassBypassPos; n < numSbCoeff - 1; n++)

baseLevel = 0

rice = cctx.templateAbsSumTS(n, coeff, baseLevel)

abs_remainder[n]

coeff_sign_flag[n] (bypass)

level mapping

Proposed methods

Item 1

The proposed Item 1 is implemented on VTM6.0.

In the proposed Item 1, the CCB check method is changed from per coefficient flag to per coefficient of each pass.

This CCB check method is same as that of CE7-1.3b.

The Item 1 reduces the number of CCB check to one fourth, which is more hardware friendly.

The Item1 realizes the unification of CCB check method between transform residual coding and TS residual coding.

The Table1 below shows the comparison of TS-CCB check method between VTM6.0 and proposed Item.

Table 1: the comparison of TS-CCB check method between VTM6.0 and proposed Item

| TS residual coding (VTM6.0) | TS residual coding (proposed Item) |
| --- | --- |
| residual_ts_coding( ){ | residual_ts_coding( ) { |
| for(n=0, n <= numSbCoeff – 1, n++ ) // Pass1 | for(n=0, n <= numSbCoeff – 1, n++ ) // Pass1 |
| { | { |
| if (CCB >= 1) | if (CCB >= 4) |
| sig_coeff_flag (CABAC) | sig_coeff_flag (CABAC) |
| CCB-- | CCB-- |
| Else | coeff_sign_flag (CABAC) |
| sig_coeff_flag (bypass) | CCB-- |
| if (CCB >= 1) | abs_level_gtx_flag[0] (CABAC) |
| coeff_sign_flag (CABAC) | CCB-- |
| CCB-- | par_level_flag (CABAC) |
| Else | CCB-- |
| coeff_sign_flag (bypass) | else |
| if (CCB >= 1) | sig_coeff_flag (bypass) |
| abs_level_gtx_flag[0] (CABAC) | coeff_sign_flag (bypass) |
| CCB-- | abs_level_gtx_flag[0] (bypass) |
| Else | par_level_flag (bypass) |
| abs_level_gtx_flag[0] (bypass) | } |
| if (CCB >= 1) |  |
| par_level_flag (CABAC) |  |
| CCB-- |  |
| Else |  |
| par_level_flag (bypass) |  |
| } |  |
| for(n=0, n <= numSbCoeff – 1, n++ ) // Pass2 | for(n=0, n <= numSbCoeff – 1, n++) // Pass2 |
| { | { |
| if (CCB >= 1) | if (CCB >= 4) |
| abs_level_gtx_flag[1] (CABAC) | abs_level_gtx_flag[1] (CABAC) |
| CCB-- | CCB-- |
| Else | abs_level_gtx_flag[2] (CABAC) |
| abs_level_gtx_flag[1] (bypass) | CCB-- |
| if (CCB >= 1) | abs_level_gtx_flag[3] (CABAC) |
| abs_level_gtx_flag[2] (CABAC) | CCB-- |
| CCB-- | abs_level_gtx_flag[4] (CABAC) |
| Else | CCB-- |
| abs_level_gtx_flag[2] (bypass) | else |
| if (CCB >= 1) | abs_level_gtx_flag[1] (bypass) |
| abs_level_gtx_flag[3] (CABAC) | abs_level_gtx_flag[2] (bypass) |
| CCB-- | abs_level_gtx_flag[3] (bypass) |
| Else | abs_level_gtx_flag[4] (bypass) |
| abs_level_gtx_flag[3] (bypass) |  |
| if (CCB >= 1) |  |
| abs_level_gtx_flag[4] (CABAC) |  |
| CCB-- |  |
| Else |  |
| abs_level_gtx_flag[4] (bypass) |  |
| } | } |
| for (n=0, n <= numSbCoeff – 1, n++) // Pass3 | for (n=0, n <= numSbCoeff – 1, n++) //Pass3 |
| … | … |
| } | } |

Item 2

Item 2 is implemented on CE7-1.3b-alternative method.

In the proposed Item 2, the level mapping is disabled in the Pass 3(b). Disabling level mapping reduces hardware complexity

This Item is same concept of the processing of method2 in JVET-O0406 [3], and realizes more unified syntax between transform residual and transform skip residual than CE7-1.3b-alternative method.

The processing of Item2 is as follows:

- Pass 1: for (n=0, n <= numSbCoeff – 1 && CCB>=4, n++)

sig_coeff_flag (CABAC)

coeff_sign_flag (same as above)

Gt 1 flag (abs_level_gtx_flag[0]) (same as above)

parity flag (par_level_flag) (same as above)

FirstPassBypassPos = n

- Pass 2: for (n=0, n <= FirstPassBypassPos && CCB>=4, n++)

Gt 3 flag (abs_level_gtx_flag[1]) (same as above)

Gt 5 flag (abs_level_gtx_flag[2]) (same as above)

Gt 7 flag (abs_level_gtx_flag[3]) (same as above)

Gt 9 flag (abs_level_gtx_flag[4]) (same as above)

SecondPassBypassPos = n

- Pass 3(a): for (n = 0; n < FirstPassBypassPos; n++)

baseLevel = (n >= SecondpassBypassPos ? 2 : 10)

rice = cctx.templateAbsSumTS(n, coeff, baseLevel)

abs_remainder[n]

level mapping

- Pass 3(b): for (n = FirstPassBypassPos; n < numSbCoeff - 1; n++)

baseLevel = 0

rice = cctx.templateAbsSumTS(n, coeff, baseLevel)

dec_abs_level[n]

coeff_sign_flag[n] (bypass)

level mapping

Modified Item 2

In the modified Item 2, the coeff_sign_flag is excluded from pass3 (b) to another loop. This Item realizes the complete same syntax between transform residual syntax and TS residual syntax after CCB count exceeds the maximum number. Note that there is no BD-rate change of this Item with Item 2.

The processing of this Item is as follows:

- Pass 1: for (n=0, n <= numSbCoeff – 1 && CCB>=4, n++)

sig_coeff_flag (CABAC)

coeff_sign_flag (same as above)

Gt 1 flag (abs_level_gtx_flag[0]) (same as above)

parity flag (par_level_flag) (same as above)

FirstPassBypassPos = n

- Pass 2: for (n=0, n <= FirstPassBypassPos && CCB>=4, n++)

Gt 3 flag (abs_level_gtx_flag[1]) (same as above)

Gt 5 flag (abs_level_gtx_flag[2]) (same as above)

Gt 7 flag (abs_level_gtx_flag[3]) (same as above)

Gt 9 flag (abs_level_gtx_flag[4]) (same as above)

SecondPassBypassPos = n

- Pass 3(a): for (n = 0; n < FirstPassBypassPos; n++)

baseLevel = (n >= SecondpassBypassPos ? 2 : 10)

rice = cctx.templateAbsSumTS(n, coeff, baseLevel)

abs_remainder[n]

level mapping

- Pass 3(b): for (n = FirstPassBypassPos; n < numSbCoeff - 1; n++)

baseLevel = 0

rice = cctx.templateAbsSumTS(n, coeff, baseLevel)

dec_abs_level[n]

level mapping

- Pass 4(b): for (n =FirstPassBypassPos; n < numSbCoeff-1; n++)

coeff_sign_flag[n] (bypass)

Simulation results

Table2, 3 show the simulation results of the proposed Item1 and 2. These simulations are conducted using VTM-6.0 under common test conditions of [1]. Table4, 5 show the simulation results of the proposed Item 1 and 2 in low QP case. Note that the modified Item 2 doesn’t affect the BD-rate and only results of Item 2 is presented.

The simulation time of Class A1 and ClassA2 in Table6 and Table7 is not accurate.

Table6 shows the simulation results of the proposed Item 2 over CE7-1.3b alternative method. Table7 shows the simulation results of the proposed Item 2 in low QP case over CE7-1.3b alternative method.

Table 2: Simulation results of Item 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | 0.03% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.05% | 100% | 99% |
| Class C | 0.00% | -0.06% | -0.01% | 99% | 99% |
| Class E | 0.00% | 0.08% | 0.02% | 100% | 99% |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 99% |
| Class D | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class F | 0.02% | 0.00% | 0.06% | 101% | 100% |
| TGM | 0.04% | 0.05% | 0.06% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.13% | 0.07% | 100% | 100% |
| Class B | -0.01% | 0.12% | -0.03% | 100% | 100% |
| Class C | 0.02% | 0.05% | 0.10% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.03% | 100% | 100% |
| Class D | -0.08% | -0.22% | -0.10% | 100% | 101% |
| Class F | 0.02% | -0.03% | -0.07% | 101% | 99% |
| TGM | 0.00% | -0.02% | 0.00% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.07% | 0.26% | 101% | 101% |
| Class C | -0.04% | -0.27% | 0.05% | 100% | 100% |
| Class E | -0.01% | 0.24% | 0.54% | 100% | 98% |
| Overall | -0.01% | 0.00% | 0.26% | 100% | 100% |
| Class D | -0.04% | 0.35% | -0.04% | 100% | 99% |
| Class F | -0.01% | -0.26% | 0.08% | 100% | 97% |
| TGM | -0.01% | 0.01% | 0.06% | 101% | 99% |

Table 3: Simulation results of Item 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.01% | 100% | 101% |
| Class A2 | 0.01% | 0.01% | 0.02% | 100% | 99% |
| Class B | 0.00% | -0.01% | -0.05% | 101% | 99% |
| Class C | 0.01% | -0.14% | 0.09% | 100% | 99% |
| Class E | 0.00% | 0.04% | 0.03% | 100% | 99% |
| Overall | 0.00% | -0.02% | 0.01% | 100% | 99% |
| Class D | -0.01% | 0.02% | -0.02% | 100% | 101% |
| Class F | 0.01% | 0.03% | -0.11% | 102% | 100% |
| TGM | 0.05% | 0.10% | 0.08% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.08% | 100% | 100% |
| Class A2 | 0.01% | 0.09% | 0.04% | 101% | 99% |
| Class B | -0.01% | 0.14% | -0.03% | 101% | 100% |
| Class C | -0.02% | 0.07% | 0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.08% | 0.04% | 101% | 100% |
| Class D | -0.06% | -0.19% | -0.07% | 101% | 101% |
| Class F | 0.02% | 0.06% | 0.07% | 101% | 100% |
| TGM | 0.04% | 0.06% | 0.08% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.18% | -0.05% | 101% | 101% |
| Class C | -0.03% | -0.13% | -0.17% | 100% | 100% |
| Class E | 0.05% | -0.53% | -0.28% | 101% | 98% |
| Overall | 0.00% | -0.10% | -0.15% | 101% | 100% |
| Class D | 0.00% | -0.09% | 0.86% | 101% | 99% |
| Class F | 0.00% | -0.07% | 0.29% | 101% | 97% |
| TGM | -0.06% | 0.04% | 0.09% | 101% | 99% |

Table 4: Simulation results of Item 1 in LowQP case

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 93% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Class B | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class E | 0.01% | -0.01% | -0.01% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.01% | 97% | 100% |
| Class D | 0.01% | -0.01% | 0.03% | 100% | 102% |
| Class F | 0.08% | 0.03% | 0.00% | 100% | 101% |
| TGM | 0.10% | 0.07% | 0.08% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.01% | 0.03% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.02% | -0.05% | 0.05% | 100% | 101% |
| Class F | -0.01% | -0.16% | -0.07% | 100% | 101% |
| TGM | 0.06% | 0.10% | 0.07% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class C | -0.01% | 0.01% | 0.00% | 100% | 100% |
| Class E | 0.00% | -0.06% | 0.07% | 100% | 100% |
| Overall | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.00% | 0.03% | -0.04% | 99% | 100% |
| Class F | 0.03% | 0.08% | -0.03% | 100% | 101% |
| TGM | 0.00% | 0.05% | 0.07% | 99% | 98% |

Table 5: Simlation results of Item 2 in LowQP case

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | 0.01% | 93% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Class B | -0.03% | -0.01% | 0.00% | 100% | 101% |
| Class C | -0.07% | -0.04% | -0.04% | 100% | 101% |
| Class E | -0.05% | -0.08% | -0.06% | 100% | 100% |
| Overall | -0.04% | -0.03% | -0.02% | 98% | 100% |
| Class D | -0.07% | -0.05% | -0.06% | 99% | 101% |
| Class F | -1.21% | -0.45% | -0.45% | 99% | 101% |
| TGM | -1.36% | -0.95% | -0.96% | 99% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.01% | 0.01% | -0.01% | 100% | 101% |
| Class C | -0.05% | -0.02% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.08% | -0.12% | -0.13% | 100% | 100% |
| Class F | -1.55% | -0.59% | -0.51% | 100% | 102% |
| TGM | -2.01% | -0.72% | -0.69% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class C | -0.02% | -0.01% | 0.01% | 100% | 101% |
| Class E | -0.08% | -0.01% | 0.00% | 100% | 101% |
| Overall | -0.03% | -0.01% | 0.00% | 100% | 101% |
| Class D | -0.04% | -0.02% | -0.03% | 100% | 101% |
| Class F | -0.44% | -0.31% | -0.30% | 100% | 101% |
| TGM | -0.91% | -0.43% | -0.38% | 99% | 100% |

Table 6: Simulation results of Item 2 over CE7-1.3b-alternative-method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE7-1.3b-alt |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | #DIV/0! | #DIV/0! |
| Class A2 | 0.00% | 0.00% | 0.01% | #DIV/0! | #DIV/0! |
| Class B | 0.00% | -0.02% | 0.00% | #DIV/0! | #DIV/0! |
| Class C | 0.00% | -0.08% | 0.10% | #DIV/0! | #DIV/0! |
| Class E | 0.00% | -0.02% | 0.03% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | -0.02% | 0.03% | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.06% | 0.07% | #DIV/0! | #DIV/0! |
| Class F | 0.01% | 0.08% | -0.09% | #DIV/0! | #DIV/0! |
| TGM | 0.01% | 0.02% | 0.03% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE7-1.3b-alt |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.07% | #DIV/0! | #DIV/0! |
| Class A2 | 0.00% | 0.00% | -0.04% | #DIV/0! | #DIV/0! |
| Class B | -0.01% | -0.05% | -0.09% | #DIV/0! | #DIV/0! |
| Class C | -0.03% | -0.02% | -0.01% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.03% | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.14% | -0.09% | #DIV/0! | #DIV/0! |
| Class F | 0.01% | 0.10% | -0.07% | #DIV/0! | #DIV/0! |
| TGM | 0.07% | 0.07% | 0.05% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE7-1.3b-alt |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.07% | 0.10% | #DIV/0! | #DIV/0! |
| Class C | -0.01% | -0.13% | -0.01% | #DIV/0! | #DIV/0! |
| Class E | 0.02% | -0.62% | -0.22% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | -0.17% | -0.02% | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.21% | 0.88% | #DIV/0! | #DIV/0! |
| Class F | -0.09% | -0.34% | -0.25% | #DIV/0! | #DIV/0! |
| TGM | 0.03% | 0.11% | 0.13% | #DIV/0! | #DIV/0! |

Table 7: Simulation results of Item 2 in Low QP case over CE7-1.3b-alternative-method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE7-1.3b-alternative |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.00% | #DIV/0! | #DIV/0! |
| Class A2 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Class B | -0.01% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Class C | -0.02% | -0.01% | -0.03% | #DIV/0! | #DIV/0! |
| Class E | -0.02% | -0.01% | -0.03% | #DIV/0! | #DIV/0! |
| Overall | -0.01% | -0.01% | -0.01% | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.02% | -0.02% | #DIV/0! | #DIV/0! |
| Class F | 0.00% | 0.05% | 0.06% | #DIV/0! | #DIV/0! |
| TGM | 0.05% | 0.08% | 0.06% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE7-1.3b-alternative |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.01% | 0.00% | -0.02% | #DIV/0! | #DIV/0! |
| Class C | -0.01% | 0.01% | 0.00% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.04% | -0.03% | 0.00% | #DIV/0! | #DIV/0! |
| Class F | -0.04% | -0.03% | 0.17% | #DIV/0! | #DIV/0! |
| TGM | 0.03% | 0.16% | 0.19% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE7-1.3b-alternative |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.01% | 0.02% | #DIV/0! | #DIV/0! |
| Class C | 0.00% | -0.01% | 0.05% | #DIV/0! | #DIV/0! |
| Class E | -0.03% | -0.02% | 0.03% | #DIV/0! | #DIV/0! |
| Overall | -0.01% | -0.01% | 0.03% | #DIV/0! | #DIV/0! |
| Class D | -0.03% | 0.00% | 0.08% | #DIV/0! | #DIV/0! |
| Class F | 0.05% | -0.03% | -0.03% | #DIV/0! | #DIV/0! |
| TGM | 0.09% | 0.14% | 0.09% | #DIV/0! | #DIV/0! |

Conclusion

The proposed Item1 aims to simplify CCB check method of TS residual coding. The proposed Item 2 aims to unify two residual syntaxes between transform residual and TS residual. There are no coding losses in all classes by those two Items, and especially in Low QP case, Item 2 shows interesting gains in ClassF and TGM. We suggest to adopt this Item 1 or Item 2 or modified Item 2 into the test model.

Reference

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-O1010, Geneva, CH, Oct. 2019.

[2] Mohammed Golam Sarwer, Ruling Liao, Daniel Luo, Yan Ye, “CE7-1.3: Simplification of transoform-skip residual coding“, JVET-P0072, October 2019

[3] Yusuke Kato, Kiyofumi Abe, Tadamasa Toma, “ Non-CE7: Unification of syntaxes after CCB count exceeds the maximum number between transform residual and transform skip residual“, JVET-O0406, Gothenburg 2019

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).