| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0304-v1 |
| --- | --- |

| Title: | CE4-related: Simplification of blending weights and motion field storage in geometric merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Jie Chen, Yan Ye, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In core experiment 4, the complexity and coding performance of geometric merge mode (GEO) are studied. In GEO mode, two masks needed to be stored, one for the blending weights and the other for the motion field storage. The memory needed to store these two masks ranges from 1.9 to 3.4 Gbytes based on the setting of number of partitioning modes supported in GEO. In this contribution, a method is proposed to reduce the memory usage. The masks for a block are directly cropped from two sets of pre-defined masks, one set for blending weights and the other set for the motion field storage. The number of pre-defined masks in a set is set to 16 and 12 when the number of angles supported in GEO is 32 and 24, respectively. The size of the pre-defined masks for {blending weights, motion field storage} is set to {192x192, 48x48} and {224x224, 56x56} when the number of steps supported in GEO is 5 and 4, respectively. With the proposed method, the memory usage can be reduced by 84-91%. It is reported that, under the CTC, the overall performance impact are:

CE4-1.1 140 modes + proposed method: RA:  -0.27%/-0.32%/-0.38% with xx% EncT and xx% DecT

LDB: -0.53%/-0.40%/-0.38% with xx% EncT and xx% DecT

CE4-1.2 108 modes + proposed method: RA:  -0.33%/-0.36%/-0.46% with xx% EncT and xx% DecT

LDB: -0.63%/-0.58%/-0.57% with xx% EncT and xx% DecT

CE4-1.2  80 modes + proposed method: RA:  -0.32%/-0.38%/-0.45% with xx% EncT and xx% DecT

LDB: -0.61%/-0.51%/-0.57% with xx% EncT and xx% DecT

Introduction

In CE4, the coding performance of geometric merge mode (GEO) is evaluated. The GEO mode shares the same concept as that of triangle partition modes (TPM) while providing more flexible partitioning option for motion compensation. In the GEO mode, a block is split into two partitions, and the two partitions can be in either rectangular or non-rectangular shape. Then, same as TPM, the two partitions are inter predicted with its own uni-prediction motion vectors. After predicting each of the geometric partitions, the sample values along the partitioning edge are adjusted using a blending process with adaptive weights. This is the prediction signal for the whole block, and transform and quantization process will be applied to the whole block as in other prediction modes. Finally, the motion field of a block predicted using the geometric partition mode is stored in 4x4 subblocks.

Due to the huge number of partitioning modes, it may be difficult to store all the adaptive weights and the masks for motion field storage for each block size and partitioning mode in real implementation. Taking the 140-mode variation of GEO in CE4 as an example, the memory needed to store all this information is calculated as follows:

- For blending weights:8×8+8×16+8×32+8×64+16×8+16×16+16×32+16×64+32×8+32×16+32×32+32×64+64×8+64×16+64×32+64×64+64×128+128×64+128×128×140×4=26,414,080 bits=3,301,760 bytes≅3.3 Mbytes

- For motion field storage:2×2+2×4+2×8+2×16+4×2+4×4+4×8+4×16+8×2+8×4+8×8+8×16+16×2+16×4+16×8+16×16+16×32+32×16+32×32×140×2=825,440 bits=103,180 bytes≅103 Kbytes

Memory storage requirement for all 3 variations of GEO in CE4 is listed in the table below.

|  | 140 partitioning modes | 108 partitioning modes | 80 partitioning modes |
| --- | --- | --- | --- |
| blending weights | 3,301,760 bytes | 2,547,072 bytes | 1,886,720 bytes |
| motion storage | 103,180 bytes | 79,596 bytes | 58,960 bytes |
| Overall | 3,404,940 bytes ≈ 3.4 Gbytes | 2,626,668 bytes ≈ 2.6 Gbytes | 1,945,680 bytes ≈ 1.9 Gbytes |

If the blending weights and the mask for motion field storage are on-the-fly calculated, the number of multiplications, shifts and additions for a block of size WxH are listed as follows:

- Multiplication: 3+2×W×H+0×(W×H/16);

- Shift: 6+3×W×H+8×(W×H/16);

- Addition: 10+6×W×H+7×(W×H/16).

So for an 8x8 block, 2.04 multiplications, 3.59 shifts and 6.59 additions are required per sample. Therefore, it could be undesirable to calculate the weight masks for each GEO coded blocks on-the-fly.

| Equations | × | << | + |
| --- | --- | --- | --- |
| Per block | 3 | 6 | 10 |
| displacementX=angleIdx | 0 | 0 | 0 |
| displacementY=displancementX+NumAngles≫2%NumAngles | 1 | 1 | 1 |
| rho=distanceIdx×stepSize≪scaleStep+DisdisplacementX≪wIdx+DisdisplacementY≪hIdx | 1 | 3 | 2 |
| stepSize=stepDis+77 | 0 | 0 | 1 |
| scaleStep=wIdx≥hIdx ?hIdx-3 :wIdx-3 | 0 | 0 | 1 |
| wIdx=log2CuW | 0 | 0 | 0 |
| hIdx=log2CuH | 0 | 0 | 0 |
| whRatio=wIdx≥hIdx ?wIdx-hIdx :(hIdx-wIdx) | 0 | 0 | 1 |
| angleN=angleIdx % 88-(angleIdx % 8)8-(angleIdx % 8)angleIdx % 8wIdx≥hIdx and angleIdx≫3% 1=1wIdx≥hIdx and angleIdx≫3% 1=0wIdx<hIdx and angleIdx≫3% 1=1wIdx<hIdx and angleIdx≫3% 1=0 | 1 | 0 | 1 |
| thresScaler=wIdx+hIdx≫1-1 | 0 | 1 | 2 |
| thresLower=32≫thresScaler | 0 | 1 | 0 |
| thresUpper=32-thresLower | 0 | 0 | 1 |
| Per luma sample | 2 | 3 | 6 |
| distFromLine=x≪1+1×DisdisplacementX+y≪1+1×DisdisplacementY-rho | 2 | 2 | 4 |
| distScaled=Clip3(0, 26,absdistFromLine+4≫3) | 0 | 1 | 1 |
| sampleWeightLxy=distFromLine≤0 ?GeoFilterdistScaled:8-GeoFilterdistScaled | 0 | 0 | 1 |
| Per 4x4 subblock | 0 | 8 | 7 |
| Cnt=sampleWeightLxsubblk≪2ysubblk≪2+sampleWeightLxsubblk≪2(ysubblk≪2)+3+sampleWeightL(xsubblk≪2)+3ysubblk≪2+sampleWeightL(xsubblk≪2)+3(ysubblk≪2)+3 | 0 | 8 | 7 |
| motionMaskxsubblkysubblk=Cnt≥thresUpper ? 1 :(Cnt≤threLower ?0 :2) | 0 | 0 | 0 |

Proposed methods

To reduce the memory needed for storing the adaptive weights and the masks for motion field storage, it is proposed to derive all the information from two sets of pre-defined masks, one for the blending weights derivation and the other for the masks of motion field storage.

Let g_sampleWeightL[] and g_motionMask[] represent the pre-defined masks for blending weights derivation and the masks of motion field storage, respectively. Assuming that N represents the number of pre-defined masks in each set, and N is set to NA >> 1, where NA is the number of angles supported in GEO. M×M represents the size of the pre-defined masks for the blending weights, and M is set to 128 + ((ND – 1) × (128 >> S)) << 1, where ND is the number of steps supported in GEO mode and S is set to ND – 1. So for the 32 angles with 5 steps setting, N is set to 16 and M is set to 192. For the 24 angles with 4 steps setting, N is set to 12 and M is set to 224.

For a block of size W×H with geometric partitioning index K, the blending weights for luma samples are derived as follows:

- Variables angle φ and distance ρ are obtained from a look-up table using the geometric partitioning index K. The look-up table is the same as that used in CE4-1.1.

- Variables offsetX and offsetY are calculated as follows:

offsetX=(M-W)≫1φ % N = 8 or (φ % N ≠ 0 and H ≥ W)M-W≫1+φ<N ?ρ×W≫S :-(ρ×W≫S)otherwise

offsetY=M-H≫1+φ<N ?ρ×H≫S :-(ρ×H≫S)φ % N = 8 or (φ % N ≠ 0 and H ≥ W)M-H≫1otherwise

- sampleWeightLxy=g_sampleWeightL[φ % N][x+offsetX][y+offsetY]

An illustrations of blending weights derivation is shown in Figure 1. In the figure, NA and ND are set to 32 and 4, respectively.

Figure 1. An illustration of the blending weights derivation for a 16x8 block

The blending weights for chroma samples are obtained from the weight of top-left luma sample for each corresponding 2x2 luma subblock, which is the same as the method used in CE4-1.1 and CE4-1.2.

The mask for motion field storage is derived as follows:

- Variables offsetXmotion and offsetYmotion are calculated as follows:

offsetXmotion=((M≫2)-(W≫2))≫1φ % N = 8 or (φ % N ≠ 0 and H ≥ W)((M≫2)-(W≫2))≫1+φ<N ?ρ×W≫(S+2) :-(ρ×W≫(S+2))otherwise

offsetYmotion=((M≫2)-(H≫2))≫1+φ<N ?ρ×H≫(S+2) :-(ρ×H≫(S+2))φ % N = 8 or (φ % N ≠ 0 and H ≥ W)((M≫2)-(H≫2))≫1otherwise

- motionMaskxsubblkysubblk=g_motionMask[φ % N][xsubblk+offsetXmotion][ysubblk+offsetYmotion]

It is noted that the pre-defined masks are calculated using the same equations in CE4-1.1 and CE4-1.2. More specifically, the following equations are used to calculate the pre-defined masks g_sampleWeightL[] with φ ranges from 0 to N-1.

displacementX=φ

displacementY=displancementX+(32≫2) % 32

rho=DisdisplacementX≪8+DisdisplacementY≪8

offset=1≪8-M≫1

distFromLine=(x+offset)≪1+1×DisdisplacementX+(y+offset)≪1+1×DisdisplacementY-rho

distScaled=Clip3(0, 26,absdistFromLine+4≫3)

sampleWeightLxy=distFromLine≤0 ?GeoFilterdistScaled:8-GeoFilterdistScaled

, where (x, y) represents the position of each luma sample in the pre-defined mask, Dis[] and GeoFilter[] are two look-up tables as shown in Table 2 and Table 3, respectively.

Table 2. The look-up table for Dis[]

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 64 | 63 | 59 | 53 | 45 | 36 | 24 | 12 | 0 | -12 | -24 | -36 | -45 | -53 | -59 | -63 |
| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Dis[idx] | -64 | -63 | -59 | -53 | -45 | -36 | -24 | -12 | 0 | 12 | 24 | 36 | 45 | 53 | 59 | 63 |

Table 3. The look-up table for GeoFilter[]

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter[idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| Idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| GeoFilter[idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

With the proposed method, the memory needed for the blending weights and the mask for motion field storage can be reduced by 84-91%. The number of bits needed to store the pre-defined masks are listed as follows:

- For blending weights:M×M×N×4

- For motion field storage:M×M/16×N×2

|  | 140 partitioning modes | 108 partitioning modes | 80 partitioning modes |
| --- | --- | --- | --- |
| blending weights | 294,912 bytes | 401,408 bytes | 301,056 bytes |
| motion storage | 9,216 bytes | 12,544 bytes | 9,408 bytes |
| Overall | 304,128 bytes | 413,952 bytes | 310,464 bytes |
| Original design | 3,404,940 bytes | 2,626,668 bytes | 1,945,680 bytes |
| % of reduction compared to original geometric design | 91.1% | 84.2% | 84.0% |

Simulation results

The proposed modification is implemented on top of CE4-1.1 and CE4-1.2 software. Three tests are evaluated:

Test 1: CE4-1.1 140 modes + proposed methods

Test 2: CE4-1.2 108 modes + proposed methods

Test 3: CE4-1.2 80 modes + proposed methods

As compared to the GEO results in CE4, the proposed method obtains majority of the coding gain of CE4.1.1 and CE4-1.2. Moreover, the results showed that there is negligible impact on coding performance in random access condition for test 2 and test 3. Besides, the impact on both the encoding time and decoding time is similar to the CE4 results.

Table 1. Summary results of all tests

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 140 modes | -0.36% | -0.45% | -0.54% | 104% | 100% | -0.72% | -0.66% | -0.76% | 105% | 100% |
| Test1: 140 modes + proposed method | -0.27% | -0.32% | -0.38% |  |  | -0.53% | -0.40% | -0.38% |  |  |
| 108 modes | -0.34% | -0.42% | -0.54% | 104% | 100% | -0.73% | -0.69% | -0.76% | 104% | 99% |
| Test2: 108 modes + proposed method | -0.33% | -0.36% | -0.46% |  |  | -0.63% | -0.58% | -0.57% |  |  |
| 80 modes | -0.34% | -0.39% | -0.52% | 103% | 100% | -0.68% | -0.46% | -0.60% | 103% | 99% |
| Test3: 80 modes + proposed method | -0.32% | -0.38% | -0.45% |  |  | -0.61% | -0.51% | -0.57% |  |  |

Table 2. CE4-1.1 140 modes + proposed method

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.10% | -0.22% |  |  |
| Class A2 | -0.17% | -0.22% | -0.21% |  |  |
| Class B | -0.18% | -0.17% | -0.24% |  |  |
| Class C | -0.55% | -0.75% | -0.78% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.32% | -0.38% |  |  |
| Class D | -0.32% | -0.60% | -0.39% | 105% | 102% |
| Class F | -0.19% | -0.32% | -0.21% |  |  |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.16% | -0.61% |  |  |
| Class C | -0.62% | -0.88% | -0.65% |  |  |
| Class E | -0.82% | -0.15% | 0.35% |  |  |
| Overall | -0.53% | -0.40% | -0.38% |  |  |
| Class D | -0.42% | -0.42% | -0.28% | 107% | 97% |
| Class F | -0.33% | -0.57% | 0.15% |  |  |

Table 3. CE4-1.2 108 modes + proposed method

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.12% | -0.23% |  |  |
| Class A2 | -0.23% | -0.27% | -0.21% |  |  |
| Class B | -0.21% | -0.14% | -0.29% |  |  |
| Class C | -0.65% | -0.89% | -1.02% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.36% | -0.46% |  |  |
| Class D | -0.40% | -0.71% | -0.50% | 105% | 104% |
| Class F | -0.25% | -0.40% | -0.35% |  |  |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.47% | -0.44% |  |  |
| Class C | -0.75% | -0.89% | -1.10% |  |  |
| Class E | -0.91% | -0.36% | -0.08% |  |  |
| Overall | -0.63% | -0.58% | -0.57% |  |  |
| Class D | -0.59% | -0.27% | -0.21% | 106% | 98% |
| Class F | -0.36% | -0.39% | 0.25% |  |  |

Table 4. CE4-1.2 80 modes + proposed method

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.08% | -0.23% |  |  |
| Class A2 | -0.20% | -0.26% | -0.18% |  |  |
| Class B | -0.22% | -0.20% | -0.33% |  |  |
| Class C | -0.66% | -0.93% | -0.97% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.38% | -0.45% |  |  |
| Class D | -0.40% | -0.69% | -0.57% | 104% | 102% |
| Class F | -0.26% | -0.33% | -0.29% |  |  |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.36% | -0.70% |  |  |
| Class C | -0.74% | -0.92% | -1.01% |  |  |
| Class E | -0.91% | -0.23% | 0.25% |  |  |
| Overall | -0.61% | -0.51% | -0.57% |  |  |
| Class D | -0.63% | -0.67% | -0.25% | 105% | 96% |
| Class F | -0.42% | -0.09% | -0.25% |  |  |

Conclusion

In this contribution, a simplified method for deriving blending weights and the mask for motion field storage is proposed. For each block size and partitioning mode, the blending weights and the mask for motion field storage are cropped from the two sets of pre-defined masks. With the proposed method, the memory needed to store all this information can be reduced by 84-91%. Besides, majority of the coding gain of the original GEO design can be obtained. Considering the trade-off between complexity and coding performance, we recommend to adopt this contribution to VVC.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).