| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0307-v3 |
| --- | --- |

| Title: | AHG16:Early Implementation of VVC software player and Demonstration on Mobile device |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jeeva Raj A Sagar Kotecha Shailesh Ramamurthy Abhishek Chelawat Jayasanker J Anil Kumar Bedgujar Neha Mary Thomas Saurabh Agrawal Vijayakumar GR Kaustubh Patankar | Tel: Email: | +91-8066601146 +91-8066601027 jeeva.raj@ittiam.com sagar.kotecha@ittiam.com Shailesh.Ramamurthy@ittiam.com Abhishek.Chelawat@ittiam.com Jayasanker.J@ittiam.com Anil.kumarbedgujar@ittiam.com Neha.Marythomas@ittiam.com saurabh.agrawal@ittiam.com vijayakumar.gr@ittiam.com kaustubh.patankar@ittiam.com |
| Source: | Ittiam systems private limited |  |  |

_____________________________

# Abstract

A real time decoder is the most important enabler for widespread adoption of any video coding technology or Standard. In this informative input contribution to JVET, We provide salient aspects of an early implementation of a VVC software decoder running on ARM based mobile clients. The software decoder has been implemented by Ittiam, with chosen modules optimized for ARM, is compatible with the syntax of VTM 6.0.

# Introduction

One of Ittiam’s major focus areas is embedded software implementations, optimized for the widely-prevalent ARM based mobile platform.

The following table indicates which specific tools have been supported (1:ON) by our early implementation and which tools are not supported (0: OFF), vis-à-vis the associated support in VTM 6.0. Note that in the table below, non-binary values are actual numerical values associated with the parameters – e. g. CTUSize=128. The tool names and abbreviations in the following table have the standard connotations as defined in the VVC Specification.

Table 1. Tools of VTM 6.0 vis-à-vis this early implementation on ARM

| Tool |  | VTM 6.0 (1: ON, 0: OFF) | Early ARM Implementation (1: ON, 0: OFF) |
| --- | --- | --- | --- |
| Config Tools | CTUSize | 128 | 128 |
|  | DualITree | 1 | 1 |
|  | QT,BT,TT | 1 | 1 |
|  | MTS | 1 | 0 |
|  | SBT | 1 | 0 |
|  | LFNST | 1 | 0 |
|  | ISP | 1 | 0 |
|  | MMVD | 1 | 1 |
|  | Affine | 1 | 1 |
|  | SubPuMvp | 1 | 1 |
|  | MaxNumMergeCand | 6 | 6 |
|  | CCLM | 1 | 0 |
|  | DepQuant | 1 | 0 |
|  | AMVR | 1 | 1 |
|  | ALF | 1 | 0 |
|  | GBi | 1 | 0 |
|  | BDOF | 1 | 1 |
|  | CIIP | 1 | 0 |
|  | Triangle | 1 | 0 |
|  | IBC | 1 | 0 |
|  | LMCS | 1 | 0 |
|  | MIP | 1 | 0 |
|  | DMVR | 1 | 1 |
|  | SMVD | 1 | 1 |
|  | JointCbCr | 1 | 0 |
|  | PROF | 1 | 0 |
| Non-Config Tools | 67 Intra Modes | 1 | 1 |
|  | PDPC | 1 | 1 |
|  | Large Transform | 1 | 1 |
|  | De-blocking | 1 | 1 |

Using Single Instruction Multiple Data (SIMD) on ARM, the performance achieved in our early implementation of VVC decoder demo is characterized below.

- 1920x1080@24fps on 4 cores of Cortex-A75 clocked at 2.5GHz. Tiles have been used towards a multi-threaded decoding application

- Device used: Google Pixel 3 with Android 9.0 Pie

- For the aforementioned tools, the decoder in a free-running mode was able to achieve an average decoding speed corresponding to 35 frames per second. Hence, in subsequent versions of this decoder, when all tools of VTM6.0 are integrated, we project that a real-time decoding of 1080p @ 24fps on the chosen ARM based platform would be feasible

- For effective parallelization during VVC decoding, a general recommendation is to have multiple tiles configured during VVC video encoding and stream creation.

# Playback with Software Decoder on Mobile devices

The said VVC software player can be demonstrated running on a mobile device. Pre-encoded bit streams are used for playback on this device. Some salient details have been indicated below.

- Device: Google Pixel 3 (OS: Android 9.0 Pie Processor: ARM Cortex-A75)

- Demo includes media player and renderer

- Demo runs on 4 cores of ARM Cortex- A75

# A demonstration of VVC playback can be shown during the JVET meeting.

# Profiling information for the early implementation of VVC decoder on ARM

Profiling of software decoder on ARM was performed using GPROF. Please find below the module level profile data for Kimono1080p24fps with QP 27 and cactus 1080p50fps with QP27. The module names and abbreviations have their standard connotations as per VVC Specification.

[1] and [2] contain a complexity analysis of HEVC. From the data therein, it can be seen that the percentage of MC in VVC is about the same as in HEVC, and could further reduce, when ALF and other intra tools have been integrated into this set up.

# Conclusion

Ittiam has implemented an early version of VVC software decoder ARM Cortex-A75. The results confirm that a real-time implementation of VVC decoding on mobile clients, exercising the salient tools of VTM6.0, is immensely feasible.

# References

- Frank Bossen, “On software complexity”, JCTVC-G757, JCT-VC 7rd Meeting, Geneva, November, 2010

- Frank Bossen et al, “HEVC Complexity and Implementation Analysis”, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 22, No. 12, December 2012