| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0308 |
| --- | --- |

| Title: | AHG12: On signalling of tile partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jihoon Do Dohyeon Park Yong-Uk Yoon Jae-Gon Kim (KAU) | Tel: Email: | +82-2-300-0414 jhdo@kau.kr dhpark@kau.kr yuyoon@kau.kr jgkim@kau.ac.kr |
| Source: | Korea Aerospace University |  |  |

_____________________________

- Abstract

VVC WD 6 [1] includes signaling for brick, tile and slices in PPS.

This contribution proposes some modifications on signaling of tile and brick information in the PPS as follows.

- Proposal 1: It is proposed to separate uniform_tile_spacing_flag into uniform_tile_row_spacing_flag and uniform_tile_col_spacing_flag with additional one syntax element.

- Proposal 2: It is proposed to replace the syntax elements of tile_row_height_minus1[i] and tile_column_width_minus1[i] into tile_row_height_delta[i] and tile_column_width_delta[i, which represents the difference between i-th and (i-1)-th tile row and tile column], respectively.

- Proposal 3: It is proposed to remove the syntax element of num_tiles_in_pic_minus1 when uniform_tile_spacing_flag is equal to 1.

Proposal 1

In VVC WD 6 [1], if single_tile_in_pic_flag is equal to 0, uniform_tile_spacing_flag is signaled for indicating whether tile shape is uniform or not.

The uniform_tile_spacing_flag is set equal to 1 when all of tile widths are same and tile heights are same except the tiles located at boundaries of bottom and left of picture. Figure 1 is the general case of 6K effective ERP and CMP schemes that are described in OMAF clauses D.6.3 and D.6.4 [2]. Figure 1a and 1c are the cases that only one side of tile shape (tile height) is equal .In current WD, Figure 1a and 1c are regarded as a non-uniform tile structure.

For constructing more intuitive syntax in tile and saving signaling bits in the case that one side of tile size is uniform, we propose to indicate separately the uniformity of horizontal and vertical tile shape. In other words, it is proposed to signal whether the tile widths are the same or the tile heights are the same by replacing the syntax elements of uniform_tile_spacing_flag with two syntax elements of uniform_tile_row_spacing_flag and uniform_tile_col_spacing_flag as shown in the following syntax.

When both tile width and tile height are uniform as shown in Figure 1b, the signaling of the proposed modification is inefficient, but overall it may be more efficient when diverse tile structure are considered.

| a) 6K effective ERP scheme of OMAF clause D.6.3. 6×2 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and tile row heights of 2048 and 256 luma samples | b) 6K effective CMP scheme of OMAF clause D.6.4. 3×6 tile grid with tile column widths of 768, 768, and 384 luma samples and all tile row heights equal to 768 luma samples. | c) 6K effective CMP scheme. 6×3 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and all tile row heights equal to 768 luma samples. |
| --- | --- | --- |

Figure 1. VVC tile and brick partitioning to achieve 6K effective resolutions for 360° video.

The proposed modifications for this change are shown highlighted below comapred with VVC WD6.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| uniform_tile_col_spacing_flag | u(1) |
| uniform_tile_row_spacing_flag | u(1) |
| if(uniform_tile_rows_spacing_flag ) { |  |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_rows_minus1 | ue(v) |
| for(i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[i] | ue(v) |
| } |  |
| if(uniform_tile_columns_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| } else { |  |
| num_tile_cols_minus1 | ue(v) |
| for(i = 0; i < num_tile_cols_minus1; i++ ) |  |
| tile_column_width _minus1[i] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 + 1; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus2 [ i ]+1; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

The proposed modifications on semantics for this change are shown below comapred with VVC WD 6. The semantics of uniform_tile_spacing_flag is deleted and semantics for theh uniform_tile_row_spacing_flag and uniform_tile_row_spacing_flag are added.

- 7.4.3.4 Picture parameter set RBSP semantics

…

uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_cols_width_minus1 and tile_rows_height_minus1. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_columns_minus1 and num_tile_rows_minus1 and a list of syntax element pairs tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1.

uniform_tile_row_spacing_flag equal to 1 specifies that tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_rows_height_minus1. uniform_tile_row_spacing_flag equal to 0 specifies that tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_rows_minus1 and a list of syntax element tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_row_spacing_flag is inferred to be equal to 1.

uniform_tile_col_spacing_flag equal to 1 specifies that tile column boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_cols_width_minus1. uniform_tile_col_spacing_flag equal to 0 specifies that tile column boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_columns_minus1 and a list of syntax element tile_column_width_minus1[ i ]. When not present, the value of uniform_tile_col_spacing_flag is inferred to be equal to 1.

Proposal 2

According to the use case of OMAF and 360 formats [3], the difference between the width and height of the i-th tile and (i-1)-th tile is almost zero. Therefore, the values of tile width and height can be efficiently represented as the difference between the consecutive values of each of them in terms of reducing signaling bits.

In this document, we propose to change the syntax elements of tile_row_height_minus1[i] and tile_column_width_minus1[i] into tile_row_height_delta[i] and tile_column_width_delta[i]. In addition, the syntax elements of tile_row_sign_flag[i] and tile_column_sign_flag[i] are used to indicate the sign of the difference when the difference is not equal to 0. The proposed syntax changes are shown highlighted below comapred with VVC WD 6.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| tile_column_width_delta[ i ] | ue(v) |
| if(tile_column_width_delta[ i ] !=0 && i !=0) |  |
| tile_column_sign_flag[i] | u(1) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| tile_row_height_delta[ i ] | ue(v) |
| if(tile_row_height _delta[ i ] !=0 && i !=0) |  |
| tile_row_sign_flag[i] | u(1) |
| } |  |

The proposed modifications on semantics for these change are shown below comapred with VVC WD6. The semantics of tile_column_width_delta[i], tile_row_heght_delta[i], tiel_column_sing_flag[i], and tile_row_sign_flag[i] are added.

- 7.4.3.4 Picture parameter set RBSP semantics

…

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs.

Tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs.

tile_column_width_delta[i] when i is greater than 0 specifies the difference between the width of i-th tile and and the width of (i–1)-tile in units of CTBs. Tile_column_width_delta[0] specifies (the width of the 0-th tile – 1) in units of CTBs.

tile_row_height_delta[i] when i is greater than 0 specifies the difference between the height of i-th tile and and the height of (i–1)-tile in units of CTBs. Tile_row_height_delta[0] specifies (the height of the 0-th tile – 1) in units of CTBs.

tile_column_sign_flag[i] equal to 1 indicates a positive sign for tile_column_width_delta[i]. tile_column_sign_flag[i] equal to 0 indicates a negative sign for tile_column_width_delta[i].

tile_row_sign_flag[i] equal to 1 indicates a positive sign for tile_row_height_delta[i]. tile_row_sign_flag[i] equal to 0 indicates a negative sign for tile_row_height_delta[i].

The removed values of tile_column_width_minus[1] and tile_row_height_minus[1] in the proposal can be derived by:

tile_column_width_minus1[i] = (i = = 0)? Tile_column_width_delta[0] :

tile_column_width_minus1[i-1] + ((tile_column_width_delta[i] == 0)? 0 :

(tile_column_sign_flag[i])? Tile_column_width_delta[i]: − tile_column_width_delta[i])

tile_row_height_minus1[i] = (i = = 0)? Tile_row_height_delta[0] :

tile_row_height_minus1[i-1] + ((tile_row_height_delta[i] == 0)? 0 :

(tile_row_sign_flag[i])? Tile_row_height_delta[i]: − tile_row_height_delta[i])

Proposal 3

In VVC WD 5 [4], the width and height of picture are signaled in SPS. When uniform_tile_spacing_flag is equal to 1, each value of tile width and tile height is signaled once. So, it causes parsing dependency between SPS and PPS when we derive the syntax element of num_tile_in_pic_minus1. To address this issue, JVET-O0236 [5] proposed to signal num_tile_in_pic_minus1 when uniform_tile_spacing_flag is equal to 1 for avoiding parsing dependency.
In VVC WD 6, the width and height of the picture are signaled in PPS. Additionally adding the syntax element of CTU size which is signaled in SPS to the PPS and using the syntax element of tile width, tile height, picture width and picture height in PPS, it is possible to derive num_tile_in_pic_minus1 without parsing dependency when uniform_tile_spacing_flag is equal to 1.

Therefore, we propose to remove the signaling of the syntax element of num_tile_in_pic_minus1 and add the syntax element of log2_cut_size_minus5_in_PPS in PPS. The proposed syntax changes are shown highlighted below comapred with VVC WD 6.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| log2_ctu_size_minus5_in_PPS | u(2) |
| single_tile_in_pic_flag | u(1) |
| … |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 + 1NumTilesInPic; i++ ) { |  |
| if( RowHeight[ i ] > 1 ) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| if( RowHeight[ i ] > 2 ) |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

The proposed modifications on semantics including the derivation of NumTilesInPic for this change are shown below comapred with VVC WD 6. The semantics of num_tiles_in_pic_minus1 is deleted.

- 7.4.3.4 Picture parameter set RBSP semantics

…

log2_ctu_size_minus5_in_PPS plus 5 specifies the luma coding tree block size of each CTU. The value of log2_ctu_size_minus5_inPPS shall be equal to log2_ctu_size_minus5 which is signaled in SPS.

Num_tiles_in_pic_minus1 plus 1 specifies the number of tiles in each picture referring to the PPS. The value of num_tiles_in_pic_minus1 shall be equal to NumTilesInPic – 1. When not present, the value of num_tiles_in_pic_minus1 is inferred to be equal to NumTilesInPic − 1.

The variable NumTilesInPic is set by using signaled information of tile.

NumTilesInPic = (uniform_tile_spacing_flag=0?)?

(num_tile_row_minus1+1) * (num_tile_columns_minus1+1) :

Ceil((pic_width_in_luma_sample/CTBsizeY)/(tile_columns_minus1+1))*Ceil((pic_height_in_luma_sample/CTBsizeY)/(tile_rows_minus1+1))

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, July 2019.

- “Preliminary WD of ISO/IEC 23090-2 2nd edition OMAF,” MPEG N18587, Aug. 2019.

- Y. Ye, and J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib,” JVET-M1004, January 2019.

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, Apr. 2019.

- R. Sjöberg, M. Pettersson, and M. Damghanian, “Making PPS parsing independent of SPS,” JVET-O0236, July 2019.

Patent rights declaration(s)

Korea Aerospace University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).