| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0309-v1 |
| --- | --- |

| Title: | Non-CE4: Improved Signaling Method for Merge Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

The related flag coding for merge/skip mode in VTM6.0 is not aligned with the selected probability distribution of each mode. A new signaling method that aligned with the distribution is proposed. It is reported that the modification brings 0.xx% and 0.xx% gains for RA and LB configurations respectively compared to VTM-6.0.

Introduction

Currently, there are five modes for merge mode, namely, regular merge mode, MMVD mode, sub-block mode, CIIP mode, and triangle mode. VTM6.0 adopts the signaling method proposed in JVET-O0249 [1]. In the signaling method (as shown in Figure 1), regular merge mode is coded with long bins while it has high selected probability.

|  | → |  |
| --- | --- | --- |

Figure 1  The signaling method in JVET-O0249

Proposed Modification

Based on the the probability distribution of each mode, a new signaling method that aligned with the distribution is proposed. The new method is shown in Figure 2. The decoding condition of the related flags and the inferred value if the decoding condition is not satisfied are both given in Table 1.

Figure 2  The proposed signaling method

Table 1 The decoding condition of the related flags

| Related Flag | Decoding Condition | Inferred Value (if decoding condition is not satisfied) |
| --- | --- | --- |
| Subblock merge flag | SubblcokAvailable | 0 |
| Regular merge flag | MMVDAvailable ¦¦ CIIPAvailable ¦¦ TriangleAvailable | 1 |
| MMVD merge flag | MMVDAvailable && (CIIPAvailable ¦¦ TriangleAvailable) | MMVDAvailable |
| CIIP flag | CIIPAvailable && TriangleAvailable | CIIPAvailable |

SubblcokAvailable = MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8

MMVDAvailable = sps_mmvd_enabled_flag

CIIPAvailable  = sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth*cbHeight >= 64 && cbWidth < 128 && cbHeight < 128

TriangleAvailable = sps_triangle_enabled_flag && MaxNumTriangleMergeCand >= 2 && slice_type = = B && cbWidth*cbHeight >= 64

The related text is modified as follows.

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 SubblcokAvailable ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦ ¦ ( sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B ) ) MMVDAvailable ¦¦ CIIPAvailable ¦¦ TriangleAvailable) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag MMVDAvailable && (CIIPAvailable ¦¦ TriangleAvailable)) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 CIIPAvailable && TriangleAvailable) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-6.0 is served as the experimental platform. The RA and LB configurations specified in [2] are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

We have two tests in the experiment.

Test1: The proposed method.

Test2: Both the anchor and the proposed method uses the same default context initialization values for the related flag.

The results of Test 1 and Test 2 are given in Table 1 and Table 2. It is reported that the modification in Test 1 brings 0.xx% and 0.xx% gains for RA and LB configurations respectively compared to VTM-6.0. And Test 2 brings 0.xx% and 0.xx% gains for RA and LB configurations respectively compared to VTM-6.0.

Table 1 Experimental Results of Test 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.14% | 0.12% | 101% | 103% |
| Class C | -0.01% | 0.11% | 0.06% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.05% | -0.14% | -0.16% | 101% | 103% |
| Class F | -0.06% | -0.02% | 0.05% | 101% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.05% | 0.14% | -0.02% | 100% | 99% |
| Class E | -0.25% | 0.40% | 0.06% | 100% | 98% |
| Overall |  |  |  |  |  |
| Class D | -0.05% | 0.24% | 0.36% | 100% | 99% |
| Class F |  |  |  |  |  |

Table 2 Experimental Results of Test 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.01% | -0.06% | 100% | 100% |
| Class C | -0.12% | -0.10% | -0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.19% | -0.32% | -0.36% | 100% | 100% |
| Class F | -0.03% | -0.08% | -0.09% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.13% | -0.10% | -0.15% | 99% | 98% |
| Class E | -0.18% | 0.31% | -0.19% | 98% | 89% |
| Overall |  |  |  |  |  |
| Class D | -0.14% | -0.31% | 0.44% | 99% | 99% |
| Class F |  |  |  |  |  |

Conclusion

In this contribution, a new signaling method that aligned with the distribution is proposed. The proposed method brings better coding gains. Based on the results, it is suggested to adopt them into the VTM.

Reference

- H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “Non-CE4: Merge Modes Signaling”, JVET-O049, Gothenburg, SE, 3–12 July 2019.

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).