| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0313 |
| --- | --- |

| Title: | Non-CE6: Simplification of LFNST LUT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Shrestha Ankit Kumar Bumshik Lee Yonggil Lee Junsuk Park | Email: | sandeepshrestha407@chosun.kr ankitku@chosun.kr bslee@chosun.ac.kr yglee@humaxdigital.com parkjs@humaxdigital.com |
| Source: | Chosun University and Humax |  |  |

_____________________________

- Abstract

In VTM-6.0 [1], four LFNST transform kernel sets [2] with kernel sizes 16×48 and 16×16 are defined. One of the LFNST transform kernel set (lfnstTrSetIdx) among four transform sets is selected based on predModeIntra. In this contribution, a LFNST LUT is newly defined to save the total memory size of LFNST LUT by removing the LUT for WAIP. In our proposed method, lfnstTrSetIdx for MIP and WAIP is decided to 0 and 1, respectively and lfnstTrSetIdx for predModeIntra from 67 to 94 is removed in the proposed LFNST LUT. The proposed LFNST LUT can save 28 bytes out of 96 bytes of memory size of LFNST LUT. The experimental results show Y: 0.00%, U: 0.00%, V: 0.00%, EncT: 99, DecT: 106% for AI and Y: 0.00%, U: 0.00%, V: 0.00%, EncT: 100%, DecT: 100% for RA.

Introduction

In VTM-6.0, one of the LFNST transform kernel set (lfnstTrSetIdx) [2] is selected based on predModeIntra using the LFNST LUT, which is shown in the Table.1.

Table 1. lfnstTrSetIdx based on predModeIntra

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |

However, the LSNFT LUT also defines the WAIP modes. In addition, it was decided that the mode for MIP can be mapped to PLANAR mode in [4]. The current LFNST LUT defines PLANAR, DC, directional mode as well as WAIP and MIP. Thus, the bit amount of LFNST LUT can be saved by not defining WAIP and MIP.

Proposed method

In this contribution, we propose to check if the current block is MIP or WAIP prior to LFNST LUT. If the intra_mip_flag is true, lfnstTrSetIdx can be set to 0 and if the predModeIntra is WAIP, then we set the lfnstTrSetIdx to 1. If both conditions are not satisfied, then we select lfnstTrSetIdx based on the proposed LFNST LUT. Thus, our proposed LFNST LUT can be defined as Fig. 1 under the consideration of MIP and WAIP. The proposed LFNST LUT can save the 28 bytes by removing the predModeIntra ranging from 67 to 94.

Fig. 1. Proposed LFNST LUT and lfnstTrSetIdx derivation

Experimental results

The proposed method has been implemented on VTM-6.0 reference software and experimentally evaluated for VTM test according to common test conditions for AI, RA and LDB configurations [3]

Table 1. Simulation results for AI configurations over VTM-6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 108% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 106% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 106% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 106% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 106% |

Table 2. Simulation results of RA configurations over VTM-6.0

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 103% |

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

- A.K. Ramasubramonian, G.Van der Auwera, T.Biatek, L. Pham van, M. Karczewicz , “Non-CE3: MIP simplifications” JVET-O0187, 15th Meeting: Gothenburg, SE, 3–12 July 2019

Patent rights declaration(s)

Chosun University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).