| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0319-v2 |
| --- | --- |

| Title: | CE7-related: Re-position GT3 flag into the first coding pass in TS residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Chen, F. Le Léannec, T. Poirier, F. Galpin | Tel: Email: | +33(0)256856325 ya.chen@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In order to unify the residual coding structure between transform residual and transform skip residual coding, this contribution proposes re-positioning abs_level_gtx_flag[1] (gt3_flag) into the first coding pass for residual coding of the transform skip (TS) residual block. The proposed method reportedly provides ClassF: 0.03% AI, 0.01% RA, -0.05% LDB luma BD-rates, and ClassTGM: 0.06% AI, 0.04% RA, -0.15% LDB luma BD-rates, respectively, versus the VTM6.0.

# Introduction

In VTM6.0[1], for the transform residual coding, the residual syntax elements are coded with 3 passes coding structure, and the abs_level_gtx_flag[1] (gt3_flag) is coded in the first coding pass as shown in Figure 1.

Figure 1: residual coding structure for transform blocks

For the transform skip residual (TS) coding, the residual syntax elements are also coded with 3 passes coding structure, while the abs_level_gtx_flag[1] (gt3_flag) is coded in the second coding pass as shown in Figure 2.

Figure 2: residual coding structure for transform skip blocks

Table 1 shows two residual coding structures of the two residual coding modes in VTM5.0.

Table 1. Two residual coding structures of transform residual and TS residual coding in VTM6.0

| Transform residual coding | TS residual coding |
| --- | --- |
| residual_coding( ) { | residual_ts_coding( ) { |
| for( SubBlock loop ) { | for( SubBlock loop ) { |
| coded_sub_block_flag | coded_sub_block_flag |
| for( Pass 1 ) { | for( Pass 1 ) { |
| sig_coeff_flag | sig_coeff_flag |
| abs_level_gtx_flag[0] | coeff_sign_flag |
| par_level_flag | abs_level_gtx_flag[0] |
| abs_level_gtx_flag[1] | par_level_flag |
| } | } |
| for( Pass 2-1 ) { | for( Pass 2 ) { |
| abs_remainder | for( j=1; j<5; j++ ) { |
| } | abs_level_gtx_flag[j] |
| for( Pass 2-2 ) { | } |
| dec_abs_level | } |
| } |  |
| for( Pass 3 ) { | for( Pass 3 ) { |
| coeff_sign_flag | abs_remainder |
| } | } |
| } | } |
| } | } |

# CE description

It is proposed to re-position abs_level_gtx_flag[1] (gt3_flag) into the first coding pass for residual coding of the TS residual block as shown in Table 2. As a result, it aligns the residual coding structure of TS residual coding to the transform residual coding.

Table 2. Two residual coding structures of transform residual and TS residual coding in this proposal

| Transform residual coding | TS residual coding |
| --- | --- |
| residual_coding( ) { | residual_ts_coding( ) { |
| for( SubBlock loop ) { | for( SubBlock loop ) { |
| coded_sub_block_flag | coded_sub_block_flag |
| for( Pass 1 ) { | for( Pass 1 ) { |
| sig_coeff_flag | sig_coeff_flag |
| abs_level_gtx_flag[0] | coeff_sign_flag |
| par_level_flag | abs_level_gtx_flag[0] |
| abs_level_gtx_flag[1] | par_level_flag |
| } | abs_level_gtx_flag[1] |
|  | } |
| for( Pass 2-1 ) { | for( Pass 2 ) { |
| abs_remainder | for( j=2; j<5; j++ ) { |
| } | abs_level_gtx_flag[j] |
| for( Pass 2-2 ) { | } |
| dec_abs_level | } |
| } |  |
| for( Pass 3 ) { | for( Pass 3 ) { |
| coeff_sign_flag | abs_remainder |
| } | } |
| } | } |
| } | } |

# Experimental results

The proposed method was integrated into the VTM6.0 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM6.0 reference software under the CE7 test conditions [3]. Table 3 and Table 4 summarize the experimental results for this proposal over standard QPs 22, 27, 32, and 37, and over low QPs 2, 7, 12, and 17, respectively. Please note the run time results may not be accurate in the tables. The detailed simulation results can be found in the attached spreadsheets.

Table 3. Summary of the average BD bitrate results of this proposal over standard QPs 22, 27, 32, and 37

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 106% | 105% |
| Class A2 | 0.00% | -0.01% | 0.00% | 106% | 106% |
| Class B | 0.00% | 0.02% | -0.04% | 105% | 105% |
| Class C | 0.00% | -0.05% | 0.03% | 106% | 106% |
| Class E | 0.00% | 0.06% | -0.08% | 106% | 105% |
| Overall | 0.00% | 0.00% | -0.02% | 106% | 105% |
| Class D | -0.02% | 0.07% | 0.03% | 105% | 108% |
| Class F | 0.03% | 0.03% | 0.01% | 101% | 106% |
| TGM | 0.06% | 0.04% | 0.05% | 100% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.02% | 107% | 107% |
| Class A2 | 0.00% | 0.01% | 0.03% | 105% | 110% |
| Class B | -0.01% | 0.07% | 0.05% | 98% | 97% |
| Class C | -0.01% | 0.16% | 0.09% | 101% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.05% | 102% | 101% |
| Class D | -0.01% | -0.20% | -0.05% | 101% | 84% |
| Class F | 0.01% | -0.03% | 0.06% | 100% | 97% |
| TGM | 0.04% | 0.00% | 0.01% | 101% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.03% | -0.30% | 104% | 103% |
| Class C | -0.01% | -0.02% | -0.21% | 103% | 106% |
| Class E | -0.23% | -0.26% | -0.03% | 100% | 93% |
| Overall | -0.06% | -0.08% | -0.20% | 103% | 101% |
| Class D | -0.02% | 0.23% | 0.39% | 101% | 98% |
| Class F | -0.05% | 0.22% | 0.82% | 100% | 101% |
| TGM | -0.15% | -0.06% | -0.08% | 99% | 98% |

Table 4. Summary of the average BD bitrate results of this proposal over standard QPs 2, 7, 12, and 17

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 113% | 93% |
| Class A2 | -0.01% | 0.00% | 0.01% | 106% | 90% |
| Class B | 0.00% | 0.00% | 0.03% | 110% | 90% |
| Class C | 0.00% | 0.00% | 0.01% | 115% | 93% |
| Class E | -0.02% | -0.03% | -0.01% | 118% | 93% |
| Overall | 0.00% | -0.01% | 0.01% | 112% | 92% |
| Class D | -0.01% | 0.02% | 0.00% | 116% | 94% |
| Class F | 0.05% | -0.01% | -0.02% | 113% | 90% |
| TGM | 0.00% | 0.00% | 0.01% | 107% | 88% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.01% | 0.00% | 97% | 89% |
| Class C | -0.01% | -0.01% | 0.01% | 97% | 89% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.01% | 0.00% | 100% | 90% |
| Class F | 0.01% | -0.06% | -0.03% | 94% | 81% |
| TGM | -0.05% | -0.01% | -0.02% | 98% | 80% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | -0.02% | 99% | 88% |
| Class C | -0.01% | 0.00% | 0.00% | 104% | 92% |
| Class E | 0.00% | -0.06% | 0.03% | 106% | 93% |
| Overall | 0.00% | -0.01% | 0.00% | 103% | 91% |
| Class D | -0.01% | 0.03% | -0.09% | 103% | 93% |
| Class F | 0.13% | 0.12% | 0.05% | 100% | 79% |
| TGM | 0.01% | 0.06% | 0.13% | 94% | 68% |

Bin to bit ratio analysis is conducted according to CE7 testing conditions. For CTC conditions, the bin to bit ratio analysis is provided in Table 5. For low QP conditions, the bin to bit ratio analysis is provided in Table 6.

Table 5. Summary of CABAC bin to bit results for under CTCs.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | 0.00% | 1.19 |  |  | 1.19 |  | 0.00% | 1.15 | 1.15 | 0.03% | 1.16 | 1.16 | 0.01% | 1.19 | 1.19 | 0.00% | 106% | 105% |
| Class A2 | 0.00% |  | -0.01% |  | 0.00% | 1.18 |  |  | 1.18 |  | -0.02% | 1.16 | 1.16 | -0.02% | 1.16 | 1.16 | 0.00% | 1.18 | 1.18 | 0.00% | 106% | 106% |
| Class B | 0.00% |  | 0.02% |  | -0.04% | 1.15 |  |  | 1.15 |  | 0.01% | 1.13 | 1.13 | -0.02% | 1.13 | 1.13 | 0.00% | 1.15 | 1.15 | 0.00% | 105% | 105% |
| Class C | 0.00% |  | -0.05% |  | 0.03% | 1.08 |  |  | 1.08 |  | -0.31% | 1.09 | 1.08 | -0.35% | 1.09 | 1.09 | 0.00% | 1.07 | 1.07 | 0.00% | 106% | 106% |
| Class E | 0.00% |  | 0.06% |  | -0.08% | 1.11 |  |  | 1.11 |  | 0.11% | 1.10 | 1.10 | 0.01% | 1.10 | 1.10 | 0.01% | 1.10 | 1.10 | 0.01% | 106% | 105% |
| Overall | 0.00% |  | 0.00% |  | -0.02% | 1.14 |  |  | 1.14 |  | -0.06% | 1.13 | 1.12 | -0.08% | 1.13 | 1.13 | 0.00% | 1.14 | 1.14 | 0.00% | 106% | 105% |
| Class D | -0.02% |  | 0.07% |  | 0.03% | 1.06 |  |  | 1.06 |  | 0.14% | 1.07 | 1.07 | 0.28% | 1.08 | 1.08 | 0.00% | 1.04 | 1.04 | -0.01% | 105% | 108% |
| Class F | 0.03% |  | 0.03% |  | 0.01% | 1.16 |  |  | 1.16 |  | 0.14% | 1.17 | 1.17 | 0.12% | 1.18 | 1.18 | 0.00% | 1.16 | 1.16 | 0.01% | 101% | 106% |
| Class TGM | 0.06% |  | 0.04% |  | 0.05% | 1.20 |  |  | 1.20 |  | 0.20% | 1.33 | 1.33 | 0.12% | 1.33 | 1.33 | -0.02% | 1.20 | 1.20 | -0.01% | 100% | 105% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.02% |  | -0.01% |  | 0.02% | 1.18 |  | 1.18 |  | 0.01% |  | 1.14 | 1.14 | 0.01% | 1.18 | 1.18 | -0.01% | 1.22 | 1.22 | -0.01% | 107% | 107% |
| Class A2 | 0.00% |  | 0.01% |  | 0.03% | 1.17 |  | 1.17 |  | 0.00% |  | 1.14 | 1.14 | -0.04% | 1.21 | 1.21 | 0.00% | 1.22 | 1.22 | 0.00% | 105% | 110% |
| Class B | -0.01% |  | 0.07% |  | 0.05% | 1.14 |  | 1.14 |  | -0.01% |  | 1.10 | 1.10 | -0.05% | 1.17 | 1.17 | -0.01% | 1.19 | 1.19 | 0.01% | 98% | 97% |
| Class C | -0.01% |  | 0.16% |  | 0.09% | 1.05 |  | 1.05 |  | 0.16% |  | 1.03 | 1.03 | 0.13% | 1.13 | 1.13 | 0.01% | 1.09 | 1.09 | 0.02% | 101% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | 0.07% |  | 0.05% | 1.13 |  | 1.13 |  | 0.04% |  | 1.10 | 1.10 | 0.01% | 1.17 | 1.17 | 0.00% | 1.17 | 1.17 | 0.00% | 102% | 101% |
| Class D | -0.01% |  | -0.20% |  | -0.05% | 1.02 |  | 1.02 |  | 0.08% |  | 1.01 | 1.01 | 0.05% | 1.14 | 1.14 | 0.04% | 1.02 | 1.02 | -0.02% | 101% | 84% |
| Class F | 0.01% |  | -0.03% |  | 0.06% | 1.11 |  | 1.11 |  | -0.06% |  | 1.08 | 1.08 | 0.10% | 1.16 | 1.16 | -0.02% | 1.15 | 1.15 | -0.02% | 100% | 97% |
| Class TGM | 0.04% |  | 0.00% |  | 0.01% | 1.15 |  | 1.15 |  | 0.21% |  | 1.22 | 1.22 | 0.02% | 1.36 | 1.36 | -0.01% | 1.22 | 1.22 | 0.02% | 101% | 104% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | -0.03% |  | -0.30% | 1.18 | 1.18 |  |  | -0.47% |  | 1.16 | 1.15 | -0.68% | 1.29 | 1.29 | 0.02% | 1.24 | 1.24 | 0.01% | 104% | 103% |
| Class C | -0.01% |  | -0.02% |  | -0.21% | 1.09 | 1.09 |  |  | 0.03% |  | 1.11 | 1.11 | -0.11% | 1.27 | 1.27 | -0.02% | 1.15 | 1.15 | -0.01% | 103% | 106% |
| Class E | -0.23% |  | -0.26% |  | -0.03% | 1.11 | 1.11 |  |  | -0.14% |  | 1.11 | 1.11 | -0.14% | 1.36 | 1.36 | 0.00% | 1.18 | 1.18 | -0.05% | 100% | 93% |
| Overall | -0.06% |  | -0.08% |  | -0.20% | 1.13 | 1.13 |  |  | -0.23% |  | 1.13 | 1.13 | -0.35% | 1.30 | 1.30 | 0.00% | 1.19 | 1.19 | -0.01% | 103% | 101% |
| Class D | -0.02% |  | 0.23% |  | 0.39% | 1.05 | 1.05 |  |  | -0.04% |  | 1.08 | 1.08 | -0.04% | 1.31 | 1.31 | 0.04% | 1.10 | 1.10 | 0.03% | 101% | 98% |
| Class F | -0.05% |  | 0.22% |  | 0.82% | 1.19 | 1.18 |  |  | -0.14% |  | 1.21 | 1.21 | -0.01% | 1.30 | 1.30 | 0.04% | 1.21 | 1.21 | -0.01% | 100% | 101% |
| Class TGM | -0.15% |  | -0.06% |  | -0.08% | 1.19 | 1.19 |  |  | 0.10% |  | 1.29 | 1.29 | 0.06% | 1.49 | 1.49 | -0.02% | 1.30 | 1.30 | -0.02% | 99% | 98% |

Table 6. Summary of CABAC bin to bit results for under Low QP conditions.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | -0.01% | 1.02 |  |  | 1.02 |  | 0.00% | 0.95 | 0.95 | 0.01% | 0.96 | 0.96 | 0.00% | 1.02 | 1.02 | -0.01% | 113% | 93% |
| Class A2 | -0.01% |  | 0.00% |  | 0.01% | 0.90 |  |  | 0.90 |  | -0.04% | 0.84 | 0.84 | -0.05% | 0.84 | 0.84 | 0.01% | 0.91 | 0.91 | 0.01% | 106% | 90% |
| Class B | 0.00% |  | 0.00% |  | 0.03% | 0.93 |  |  | 0.93 |  | -0.06% | 0.86 | 0.86 | -0.08% | 0.87 | 0.87 | 0.00% | 0.94 | 0.94 | -0.01% | 110% | 90% |
| Class C | 0.00% |  | 0.00% |  | 0.01% | 0.83 |  |  | 0.83 |  | -0.02% | 0.74 | 0.74 | -0.06% | 0.75 | 0.75 | -0.01% | 0.83 | 0.83 | 0.00% | 115% | 93% |
| Class E | -0.02% |  | -0.03% |  | -0.01% | 1.01 |  |  | 1.01 |  | 0.10% | 0.94 | 0.94 | 0.11% | 0.94 | 0.94 | 0.08% | 1.01 | 1.01 | 0.07% | 118% | 93% |
| Overall | 0.00% |  | -0.01% |  | 0.01% | 0.93 |  |  | 0.93 |  | -0.01% | 0.86 | 0.86 | -0.03% | 0.87 | 0.87 | 0.01% | 0.94 | 0.94 | 0.01% | 112% | 92% |
| Class D | -0.01% |  | 0.02% |  | 0.00% | 0.80 |  |  | 0.80 |  | 0.07% | 0.71 | 0.71 | 0.08% | 0.73 | 0.73 | 0.00% | 0.81 | 0.81 | -0.01% | 116% | 94% |
| Class F | 0.05% |  | -0.01% |  | -0.02% | 0.96 |  |  | 0.96 |  | -0.03% | 0.87 | 0.87 | -0.05% | 0.85 | 0.85 | -0.04% | 0.94 | 0.94 | -0.02% | 113% | 90% |
| Class TGM | 0.00% |  | 0.00% |  | 0.01% | 0.87 |  |  | 0.87 |  | -0.02% | 0.80 | 0.80 | -0.13% | 0.80 | 0.80 | -0.01% | 0.87 | 0.87 | 0.02% | 107% | 88% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! | 0.96 |  | #DIV/0! |  | #DIV/0! |  | 0.89 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | 1.15 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! | 0.84 |  | #DIV/0! |  | #DIV/0! |  | 0.77 | #DIV/0! | #DIV/0! | 1.03 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class B | 0.02% |  | 0.04% |  | -0.01% | 0.87 |  | 0.87 |  | -0.05% |  | 0.80 | 0.80 | -0.04% | 1.03 | 1.04 | 0.91% | 1.11 | 1.12 | 0.75% | 97% | 89% |
| Class C | 0.00% |  | 0.01% |  | 0.00% | 0.77 |  | 0.77 |  | 0.01% |  | 0.68 | 0.68 | -0.09% | 0.96 | 0.97 | 1.83% | 1.03 | 1.04 | 1.36% | 97% | 89% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! | 0.86 |  | #DIV/0! |  | #DIV/0! |  | 0.78 | #DIV/0! | #DIV/0! | 1.02 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class D | 0.00% |  | 0.03% |  | 0.00% | 0.75 |  | 0.75 |  | 0.05% |  | 0.66 | 0.66 | -0.06% | 0.93 | 0.95 | 1.98% | 1.00 | 1.01 | 1.23% | 100% | 90% |
| Class F | 0.03% |  | -0.05% |  | 0.00% | 0.90 |  | 0.90 |  | 0.10% |  | 0.81 | 0.82 | 0.06% | 0.93 | 0.95 | 1.86% | 1.01 | 1.02 | 1.33% | 94% | 81% |
| Class TGM | -0.05% |  | -0.02% |  | -0.02% | 0.84 |  | 0.84 |  | -0.04% |  | 0.77 | 0.77 | 0.02% | 0.86 | 0.87 | 1.36% | 0.93 | 0.95 | 1.21% | 98% | 80% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.02% |  | -0.02% | 0.93 | 0.93 |  |  | 0.00% |  | 0.86 | 0.86 | 0.02% | 1.03 | 1.03 | 0.00% | 1.10 | 1.10 | 0.00% | 99% | 88% |
| Class C | -0.01% |  | 0.00% |  | 0.00% | 0.82 | 0.82 |  |  | -0.06% |  | 0.73 | 0.73 | -0.06% | 0.98 | 0.98 | 0.01% | 1.05 | 1.05 | 0.01% | 104% | 92% |
| Class E | 0.00% |  | -0.06% |  | 0.03% | 1.01 | 1.01 |  |  | 0.07% |  | 0.94 | 0.94 | 0.06% | 1.18 | 1.18 | 0.00% | 1.24 | 1.24 | 0.00% | 106% | 93% |
| Overall | 0.00% |  | -0.01% |  | 0.00% | 0.92 | 0.92 |  |  | 0.00% |  | 0.84 | 0.84 | 0.01% | 1.06 | 1.06 | 0.00% | 1.12 | 1.12 | 0.00% | 103% | 91% |
| Class D | -0.01% |  | 0.03% |  | -0.09% | 0.79 | 0.79 |  |  | -0.02% |  | 0.71 | 0.71 | -0.01% | 0.97 | 0.97 | 0.02% | 1.04 | 1.04 | 0.01% | 103% | 93% |
| Class F | 0.13% |  | 0.12% |  | 0.05% | 0.92 | 0.92 |  |  | 0.00% |  | 0.84 | 0.84 | -0.05% | 1.03 | 1.03 | -0.03% | 1.09 | 1.09 | -0.05% | 100% | 79% |
| Class TGM | 0.02% |  | 0.06% |  | 0.13% | 0.85 | 0.85 |  |  | 0.06% |  | 0.79 | 0.79 | 0.05% | 0.91 | 0.91 | 0.01% | 0.99 | 0.99 | -0.02% | 94% | 68% |

# Conclusions

This contribution proposes to unify residual coding structure between transform residual and transform skip residual coding by re-positioning abs_level_gtx_flag[1] (gt3_flag) into the first coding pass for TS residual coding. There are no coding losses in all classes by this unification. We suggest to adopt this proposal into the test model.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM6.0

- H. Schwarz and M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2027, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).