| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0322 |
| --- | --- |

| Title: | Non-CE4: CIIP size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geonjung Ko, Dongcheol Kim, Jason Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 greg.ko@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to restrict CIIP (combined inter merge and intra prediction) to blocks with width and height equal to or less than the maximum transform size. In the current VVC design, when the maximum transform size for the luma component is 32, the maximum intra prediction size of the regular intra prediction and that of CIIP are not aligned. The regular intra prediction can be applied to maximum 32x32 blocks, but CIIP can be applied to maximum 64x64 blocks. Thus, the hardware/software requires a capability to process 64x64 intra prediction because of CIIP. With the proposed method, if the maximum transform size is 32, the maximum intra prediction size is 32x32. The proposed method reports average luma BD-rate change of 0.04% for RA and 0.02% for LDB when the maximum transform size is set to 32.

# Introduction

VVC has adopted the maximum transform size signalling [1]. The maximum transform size for the luma component can be selected between 32 and 64. When the maximum transform size is 32, all intra predictions are performed on blocks with width and height equal to or less than 32. In case that width or height of the intra coding block is greater than 32, the block is partitioned until both width and height are equal to or less than 32. Besides the regular intra prediction, CIIP (combined inter merge and intra prediction) uses the intra prediction. The maximum block size for CIIP is fixed as 64x64.

Therefore, if the maximum transform size is 32, the maximum block size for the regular intra prediction is 32x32, but the maximum block size for the intra prediction of CIIP is 64x64. It means that because of CIIP the hardware/software should have a capability to process 64x64 intra prediction, even though the maximum block size of the regular intra prediction is 32x32.

# Proposed method

It is proposed to allow CIIP to blocks with both width and height equal to or less than the maximum transform size. Thus, if the maximum transform size is 32, the maximum CU size for CIIP is 32x32 and the maximum intra prediction size is also 32x32. Table 1 shows the proposed syntax change.

Table 1. Proposed syntax change

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 <= MaxTbSizeY && cbHeight < 128 <= MaxTbSizeY) ¦ ¦ ( sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 <= MaxTbSizeY && cbHeight < 128 <= MaxTbSizeY ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Experimental results

The proposed method was implemented on top of VTM-6.0 and tested under the common test conditions [2] except that the maximum transform size was set to 32. Table 2 and Table 3 summarize experimental results for random access and low delay B configurations, respectively. Under CTCs, the proposed method has no impact on the coding efficiency.

Table 2. Experimental results for RA configurations

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 + MaxTbSizeY=32 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.05% | 0.12% | 99% | 100% |
| Class A2 | 0.05% | 0.03% | 0.01% | 99% | 100% |
| Class B | 0.04% | 0.04% | 0.08% | 99% | 100% |
| Class C | -0.02% | 0.11% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.06% | 0.06% | 99% | 100% |
| Class D | 0.00% | 0.04% | 0.00% | 100% | 99% |
| Class F | 0.01% | 0.04% | 0.03% | 100% | 99% |

Table 3. Experimental results for LDB configurations

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 + MaxTbSizeY=32 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.17% | 0.31% | 98% | 99% |
| Class C | 0.02% | -0.04% | -0.08% | 99% | 100% |
| Class E | -0.01% | 0.06% | 0.25% | 99% | 99% |
| Overall | 0.02% | 0.07% | 0.16% | 99% | 99% |
| Class D | -0.01% | -0.07% | -0.02% | 100% | 100% |
| Class F | 0.13% | -0.07% | 0.62% | 99% | 99% |

# Conclusion

This contribution proposes to restrict the maximum CU size for CIIP to be (MaxTbSizeY x MaxTbSizeY). The proposed method decreases the hardware/software requirement for the intra prediction, which is needed to support CIIP for blocks greater than (MaxTbSizeY x MaxTbSizeY), with small BD-rate loss (0.04% for RA, 0.02% for LDB). It is suggested to adopt the proposed method in the next version of VVC draft text and the reference software.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th meeting, Gothenburg, SE, 3–12 July 2019.

[2] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th meeting, Geneva, CH, 19–27 Mar. 2019.