| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0327-v3 |
| --- | --- |

| Title: | AHG18: Configurable Maximum Transform Size for Mixed Lossy and Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hilmi E. Egilmez, Hongtao Wang, Yung-Hsuan Chao, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 analci@qti.qualcomm.com hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes to enable configurable maximum transform size, which is normatively disabled by restricting the maximum transform size down to 32 in the latest version of the VTM-6.0 software provided by the AHG18 for lossless coding. In a mixed lossy and lossless coding setting, the proposed modifications allow transform sizes up to 64 (as in current CTC) for lossy coded blocks and keeps the current 32-size restriction for lossless coded blocks.

# Introduction

In the latest version of VTM-6.0 software provided by the AHG18 for lossless coding, the maximum transform size is restricted to size 32. Although this restriction is needed for lossless coding of CUs in order to avoid introducing an additional scan type for 64x64 TUs (where the current scan type for 64x64 is designed to support the normative zero-out scheme), it is not needed for lossy coded blocks and prevents using 64x64 TUs already existing in VVC and CTC.

In the following section, the proposed modifications are described to support 64x64 TUs for lossy coded blocks in a mixed lossless and lossy coding setting.

# Proposed Method

The proposed method simply allows 64x64 TUs as in current VTM-6.0, if cu_transquant_bypass_flag is disabled. If it is enabled, it uses the same scheme in VTM-6.0 software provided by AHG18. The following draft text changes (on top of JVET-O2001-v14) reflect the proposed modifications.

#### 7.3.8.5 Coding unit syntax

…

| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| --- | --- |
| intra_bdpcm_flag | ae(v) |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= (cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY) && cbHeight <= (cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= (cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) && cbHeight <= (cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |

…

#### 7.3.8.9 Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType, chType ) { | Descriptor |
| --- | --- |
| InferTuCbfLuma = 1 |  |
| if( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !cu_sbt_flag ) { |  |
| if( tbWidth > ( cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) ¦ ¦ tbHeight > ( cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) ) { |  |
| verSplitFirst = ( tbWidth > ( cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) && tbWidth > tbHeight ) ? 1 : 0 |  |
| trafoWidth = verSplitFirst ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = !verSplitFirst ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight, chType ) |  |
| if( verSplitFirst ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType, chType ) |  |
| else |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, chType ) |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType, 0, chType ) |  |
| } |  |

…

#### 7.3.8.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ¦ ¦ CbWidth[ chType ][ x0 ][ y0 ] > ( cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > ( cu_transquant_bypass_flag ? MaxTbSizeYLossless : MaxTbSizeY ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |

…

#### 7.4.3.3 Sequence parameter set RBSP semantics

…

sps_max_luma_transform_size_64_flag equal to 1 specifies that the maximum transform size in luma samples is equal to 64. sps_max_luma_transform_size_64_flag equal to 0 specifies that the maximum transform size in luma samples is equal to 32.

When CtbSizeY is less than 64, the value of sps_max_luma_transform_size_64_flag shall be equal to 0.

The variables MinTbLog2SizeY, MaxTbLog2SizeY, MaxTbLog2SizeYLossless, MinTbSizeY, MaxTbSizeYLossless and MaxTbSizeY are derived as follows:

MinTbLog2SizeY = 2		(7-27)

MaxTbLog2SizeY = sps_max_luma_transform_size_64_flag  ?  6  :  5	(7-28)

MaxTbLog2SizeYLossless = 5

MinTbSizeY = 1  <<  MinTbLog2SizeY		(7-29)

MaxTbSizeY = 1  <<  MaxTbLog2SizeY		(7-30)

MaxTbSizeYLossless  = 1  <<   MaxTbLog2SizeYLossless

# Results

Anchor is AHG18 VTM-6.0-lossless branch. The following settings are used to run both the anchor and proposals: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. Moreover, QP value was also set as 0.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  |
| Class A1 | 2.2 | 2.2 | -0.02% | 2.3 | 2.3 | 0.00% |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.01% | 1.9 | 1.9 | 0.00% |  |  |  |
| Class B | 2.1 | 2.1 | 0.00% | 2.3 | 2.3 | 0.00% | 2.3 | 2.3 | 0.00% |
| Class C | 2.1 | 2.1 | 0.00% | 2.6 | 2.6 | 0.00% | 2.6 | 2.6 | 0.00% |
| Class D | 1.9 | 1.9 | 0.00% | 2.8 | 2.8 | 0.00% | 2.7 | 2.7 | 0.00% |
| Class E | 2.9 | 2.9 | 0.00% |  |  |  | 3.3 | 3.3 | 0.00% |
| Class F | 5.5 | 5.5 | 0.00% | 39.4 | 39.4 | 0.00% | 60.7 | 60.7 | 0.00% |
| TGM | 12.3 | 12.3 | 0.04% | 112.6 | 112.6 | 0.01% | 142.4 | 118.2 | -33.33% |
| All (A1, A2, B, C, E) | 2.2 | 2.2 | -0.01% | 2.3 | 2.3 | 0.00% | 2.6 | 2.6 | 0.00% |
| Overall | 4.0 | 4.0 | 0.00% | 25.1 | 25.1 | 0.00% | 35.6 | 26.7 | -4.17% |
| Enc Time[%] | 101% |  |  | 100% |  |  | #NUM! |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  | #NUM! |  |  |

Table 1. Results after maximum TU size restriction based on cu_transquant_bypass_flag.

*Pending Results from 1 LDB Sequence

# Recommendation

It is recommended to adopt the method described in Section 2 in VVC.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 15th meeting, Gothenburg, SE, 3-12 July 2019.

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).