| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0329-v4 |
| --- | --- |

| Title: | Simplification of Planar intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Semih Esenlik, Han Gao, Anand Meher Kotra, Elena Alshina | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

_____________________________

# Abstract

In VVC draft Specification, a clipping operation is applied for the width and height in intra planar prediction. It is asserted that the clipping operation is not necessary and can be removed. Compared to VTM6.0, Simulation results show negligible coding performance change, with 0.01% and 0.00% for all intra and random access configuration, respectively.

In version 2, presentation slides are uploaded

In version 3, additional result of low delay B is added

In version 4, excel sheet result missing in version 3 is added.

# Background

In current VVC draft specification [1], the prediciton for planar mode is defined as follows:

Inputs to this process are:

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbH and x = 0..nTbW, y = −1.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables nW and nH are derived as follows:

nW = Max( nTbW, 2 )			(8-135)

nH = Max( nTbH, 2 )			(8-136)

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1 and y = 0..nTbH − 1, are derived as follows:

predV[ x ][ y ] = ( ( nH − 1 − y ) * p[ x ][ −1 ] + ( y + 1 ) * p[ −1 ][ nTbH ] ) << Log2 ( nW ) 	(8-137)

predH[ x ][ y ] = ( ( nW − 1 − x ) * p[ −1 ][ y ] + ( x + 1 ) * p[ nTbW ][ −1 ] ) << Log2 ( nH ) 	(8-138)

predSamples[ x ][ y ] = ( predV[ x ][ y ] + predH[ x ][ y ] + nW * nH ) >> (Log2 ( nW ) + Log2 ( nH ) + 1 )	(8-139)

It is asserted that the highlighted clipping operation unnecessarily complicates the prediction process.

# Proposed method

It is proposed to removed the clipping operations and use the input width and height nTbW and nTbH directly. The modification to planar prediction is shown as follows, with text addition highlight and deletion strikethrough:

Inputs to this process are:

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbH and x = 0..nTbW, y = −1.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables nW and nH are derived as follows:

nW = Max( nTbW, 2 )			(8-135)

nH = Max( nTbH, 2 )			(8-136)

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1 and y = 0..nTbH − 1, are derived as follows:

predV[ x ][ y ] = ( ( nTbH − 1 − y ) * p[ x ][ −1 ] + ( y + 1 ) * p[ −1 ][ nTbH ] ) << Log2 ( nTbW ) 	(8-137)

predH[ x ][ y ] = ( ( nTbW − 1 − x ) * p[ −1 ][ y ] + ( x + 1 ) * p[ nTbW ][ −1 ] ) << Log2 ( nTbH ) 	(8-138)

predSamples[ x ][ y ] = ( predV[ x ][ y ] + predH[ x ][ y ] + nTbW * nTbH ) >> (Log2 ( nTbW ) + Log2 ( nTbH ) + 1 )	(8-139)

# Results

The proposed method was simulated under common test condition for VTM-6.0, and the results for all intra ,  random access, and low delay B configurations are summarized in Table 1 and 2, respectively. The results show no coding loss is introduced with the proposed removal of reference sample filtering.

Table 1: coding performance over VTM-6.0 under all intra configuration

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class A2 | 0.01% | 0.03% | -0.01% | 100% | 100% |
| Class B | 0.01% | 0.03% | -0.04% | 100% | 100% |
| Class C | 0.01% | -0.05% | 0.06% | 100% | 100% |
| Class E | 0.02% | 0.01% | -0.01% | 100% | 100% |
| Overall | 0.01% | 0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | -0.02% | 0.00% | 100% | 100% |
| Class F | 0.01% | -0.03% | -0.10% | 100% | 100% |

Table 2: coding performance over VTM-6.0 under random access configuration

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | -0.02% | 100% | 100% |
| Class A2 | 0.03% | 0.12% | 0.07% | 100% | 100% |
| Class B | 0.00% | 0.03% | 0.03% | 100% | 100% |
| Class C | -0.01% | -0.07% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | -0.03% | -0.20% | 0.01% | 100% | 99% |
| Class F | -0.02% | -0.13% | -0.04% | 100% | 100% |

Table 3: coding performance over VTM-6.0 under Low Delay B configuration

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.03% | 0.07% | 100% | 100% |
| Class C | -0.01% | -0.11% | 0.11% | 100% | 99% |
| Class E | -0.05% | 0.11% | -0.67% | 100% | 100% |
| Overall | -0.02% | 0.00% | -0.10% | 100% | 100% |
| Class D | -0.01% | 0.05% | 0.22% | 100% | 99% |
| Class F | 0.05% | -0.27% | 0.40% | 100% | 100% |

# Conclusion

This contribution proposes to remove clipping operation for the width and height of the planar predicted block. Simulation results show that the removal brings no coding performance change. It simplifies the VVC working draft and make the design coherent. It is suggested to include the proposed method in the next draft of the VVC specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, B. Bross, J. Chen, and S. Liu