| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0335-v2 |
| --- | --- |

| Title: | AHG15: Chroma QP mapping table for HDR |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu}@dolby.com |  |
| Source: | Dolby Laboratories, Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes to update the non-normative chroma QP mapping tables for HDR to improve the luma and chroma balance in VTM 6.0 for HDR common test conditions (CTC). It is proposed that:

- For class H1 (PQ content), use separate tables for Cb/Cr and joint CbCr. The tables are derived by absorbing the WCGPPSoffsets for P3 content into the mapping. More than 30% BDrate gain in wPSNRY can be achieved compared with HM16.18 anchor. Subjective evaluation has been performed on PQ reference display (Pulsar) and it is verified chroma coding performance is similar to the current VTM 6.0 anchor.

- For class H2 (HLG content), two candidate tables are provided for selection.

# Introduction

For chroma QP mapping inVTM6.0 [1], the following was decided in previous JVET meetings:

- the chroma QP table is signalled in the SPS (no default chroma QP table),

- separate mapping chroma QP tables for Cb, Cr and jointCbCr chroma coding are allowed

- chroma QP offset is applied after the chroma QP mapping table (i.e., chromaQP = Qpc (qPi) + offset)

## Chroma QP mapping for HDR CTC

For HDR CTC (PQ and HLG), it was also previously decided to keep the anchor QP settings for PQ and HLG.

For PQ CTC, chroma QP offset adjustment is applied at the encoder (WCGPPSEnable=1). Chroma QP offset (WCGPPSoffset) is computed using linear equations that have the following characteristics:

- larger luma QP results in more negative chroma QP offset

- different equations are used for Cb and Cr, and for BT.709 and P3

The derived WCGPPSoffset for P3Cb, P3Cr, 709Cb and 709Cr are shown in Figure 1.

Figure 1 WCGPPSoffset used for HDR-PQ common test conditions

## Impact of the order of operations for chroma QP table and QP offset

In VTM 5.0 and previous VTM versions, chroma QP offset was applied before the chroma QP table was applied.  In VTM 6.0, the order was reversed and the chroma QP offset is now applied after the chroma QP table is applied.

### PQ CTC

The consequences for PQ CTC of reversing the order of chroma QP offset and tables are illustrated in Figure 2 for the Cb component of P3 content. The impact on overall HDR PQ CTC luma and chroma coding performance is quantified in Table 1 and Table 2.

Figure 2 illustrates that applying the chroma QP offset after the chroma QP table (green curve, VTM 6.0) results in a combined chroma QP value that is non-monotonic and significantly smaller than the value obtained when the chroma QP offset is applied before the chroma QP table (red curve, VTM 5.0). (For comparison, the blue curve is for chroma QP table alone without the chroma QP offset.)

Figure 2 relationship between qPi and derived QpC (YCbCr case)

Table 1 BDRate of VTM6.0 versus VTM5.0 for HDR PQ content

| AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| -9.90% | 1.15% | 1.19% | -13.94% | -28.44% | -8.65% | 1.17% | 1.11% | -18.02% | -32.70% |

Table 2 BDRate of VTM6.0 versus HM16.18 for HDR PQ content

| AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| -46.38% | -23.60% | -23.40% | -58.05% | -49.74% | -43.94% | -28.23% | -28.04% | -59.64% | -68.05% |

The smaller final chroma QP values for VTM 6.0 compared to VTM 5.0 result in an overall coding performance loss in luma and large gain in chroma for AI and RA (Table 1). (Note: PSNRL100 and wPsnrY indicate luma performance. DE100 indicates overall chroma performance.) (Subjectively, no chroma difference between VTM5.0 and VTM6.0 was observed.)

Comparing VTM6.0 with HM16.18 (Table 2) for PQ content, the data show that the overall chroma gain is greater than the luma gain for both AI and RA.  For RA, for example, the overall chroma gain is 44% compared to luma gain of 28%.

The data shown in Table 1 and Table 2 suggest that better luma and chroma balance might be achieved for VTM 6.0 PQ CTC. In this contribution, a new non-normative encoder-side chroma QP table for PQ CTC is proposed to achieve better balance between luma and chroma coding performance.

### HLG CTC

For HLG, the chroma QP mapping process is the same as it is for SDR and no additional WCGPPSoffset is applied. Consequently, the change in the order of applying a chroma QP offset does not introduce the undesired impact seen in the HDR-PQ case.

As shown in Table 3, both luma and chroma have gain when comparing VTM 6.0 to VTM 5.0.

When compared to HM16.18 (Table 4), data show that the overall chroma gain is greater than the luma gain for both AI and RA.  For RA, for example, the overall chroma gain is 46% (PsnrU) and 50% (PsnrV) compared to luma gain on 28% (PsnrY).

Table 3 BDRate of VTM6.0 versus VTM5.0 for HDR HLG content

| AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
| PsnrY | PsnrU | PsnrV | PsnrY | PsnrU | PsnrV |
| -0.40% | 1.30% | 0.05% | -1.69% | -2.59% | -2.17% |

Table 4 BDRate of VTM6.0 versus HM16.18 for HDR HLG content

| AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
| PsnrY | PsnrU | PsnrV | PsnrY | PsnrU | PsnrV |
| -21.15% | -37.56% | -39.83% | -28.23% | -45.86% | -49.57% |

The data shown in Table 3 and Table 4 suggest that better luma and chroma balance might be achieved for VTM 6.0 HLG CTC. In this contribution, 3 candidate non-normative encoder-side chroma QP tables for HLG CTC are proposed to re-balance luma and chroma coding performance.

# Proposal

This contribution proposes to modify chroma QP mapping tables at encoder to achieve better luma and chroma balance for both PQ and HLG CTC.

## Class H1 (HDR-PQ) chroma QP mapping table derivation

The chroma QP mapping table QpC derivation for class H1 (HDR-PQ) follows the design criteria proposed in JVET-N0221 [3]. Since the WCGPPSoffset was absorbed in the derived QP mapping table, there is no need to turn on WCGPPSEnable and it is set to 0 for all the tests. We relied on subjective evaluation on a 4000-nit Pulsar display (the reference display for PQ signal) to choose QpC. We tested candidate functions to derive the following chroma QP tables:

- a single chroma QP table for Cb, Cr, and combined jointCbCr.; and

- separate chroma QP tables for each of Cb, Cr, and combined jointCbCr.

As had been discussed in previous meetings, separate chroma QP tables are preferable to a shared table [4]. Thus, our focus was on separate tables. The following cases were tested:

- separate BT.709 (709) tables,

- separate P3 tables,

- separate mixed 709 and P3 tables (709 content uses 709 table, P3 content uses P3 table).

The separate table for jointCbCr was computed as of the average of the Cb and Cr tables.

The test setting “VTM6.0 anchor with WCGPPSEnable=0” was used as the baseline reference against which candidate chroma QP tables were compared. Objectively, “VTM6.0 anchor with WCGPPSEnable=0” has the worst chroma coding performance. Subjectively, “VTM6.0 anchor with WCGPPSEnable=0” results in significant chroma artifacts at low bitrate.

During testing, it was observed that if BDRate of DE100 gain is close to or greater than 30% compared to “VTM6.0 anchor with WCGPPSEnable=0”, no significant chroma artifacts are seen during subjective viewing.

## Candidate chroma QP tables for PQ CTC

Objective test results for 4 candidate chroma QP tables are listed in Table 5.  Each candidate resulted in DE100 gain greater than 30% compared to “VTM6.0 anchor with WCGPPSEnable=0” and no distinction between chroma artifacts could be seen during subjective viewing. It is noted that VTM6.0 CTC anchor has the greatest loss in luma relative to WCGPPSEnable=0 (wPsnrY loss is 6.71% for RA), while separate table for P3 has smallest loss in luma relative to WCGPPSEnable=0 (wPsnrY loss is 3.55% for RA).

Table 5 BDRate of QpC candidates versus VTM6.0 anchor with WCGPPSEnable=0

|  | HDR PQ QpC vs. HDR PQ Anchor with WCGPPSEnable=0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI |  |  |  |  | RA |  |  |  |  |
| Test Case | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| VTM 6.0 official (WCGPPSEnable=1) | -42.54% | 7.22% | 8.07% | -44.77% | -65.14% | -44.69% | 6.02% | 6.71% | -53.98% | -74.48% |
| VTM 6.0 (WCGPPSEnable=0) QPc sep 709 | -40.94% | 5.58% | 6.44% | -47.25% | -64.91% | -34.43% | 4.09% | 4.66% | -46.68% | -57.91% |
| VTM 6.0 (WCGPPSEnable=0) QPc sep P3 | -33.89% | 4.13% | 4.84% | -40.96% | -50.24% | -30.59% | 3.04% | 3.55% | -41.16% | -51.54% |
| VTM 6.0 (WCGPPSEnable=0) sep mix 709/P3 | -39.26% | 5.25% | 6.08% | -45.63% | -60.97% | -33.43% | 3.89% | 4.47% | -45.13% | -55.86% |

The performance of the 4 candidate chroma QP tables compared to HM16.18 is shown in Table 6. The candidate separate chroma QP tables for P3 has the greatest luma gain.

Table 6 BDRate of QpC candidates versus HM16.18 in HDR PQ case

|  | HDR PQ QpC with WCGPPSEnable=0 versus HM16.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI |  |  |  |  | RA |  |  |  |  |
| Test Case | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| VTM 6.0 official (WCGPPSEnable=1) | -46.38% | -23.60% | -23.40% | -58.05% | -49.74% | -43.94% | -28.23% | -28.04% | -59.64% | -68.05% |
| VTM 6.0 (WCGPPSEnable=0) QPc sep 709 | -41.47% | -24.85% | -24.62% | -54.90% | -55.09% | -29.84% | -29.64% | -29.52% | -45.48% | -22.93% |
| VTM 6.0 (WCGPPSEnable=0) QPc sep P3 | -33.78% | -25.90% | -25.78% | -49.73% | -32.22% | -24.39% | -30.35% | -30.27% | -38.16% | -3.68% |
| VTM 6.0 (WCGPPSEnable=0) sep mix 709/P3 | -40.00% | -25.10% | -24.90% | -54.39% | -52.69% | -28.79% | -29.79% | -29.66% | -44.15% | -19.27% |

## Proposed chroma QP tables for PQ CTC

It is recommended that separate P3 tables be used for HDR PQ content. The separate P3 chroma QP tables provide the best luma performance and balance between luma and chroma.  It is also expected that most HDR PQ content will be created in the P3 colour space. It was also noted during testing that the subjective difference in color between the proposal and HDR CTC Anchor was observed to be very minor even at low bitrate, which indicates QpC is an acceptable alternative for encoder optimization.

The proposed changes to the encoder cfg for class H1 (PQ) are as follows (settings are for separate P3 separate tables):

SameCQPTablesForAllChroma           : 0

QpInValCb                            : 13 20 36 38 43 54

QpOutValCb                         : 13 21 29 29 32 37

QpInValCr                             : 13 20 37 41 44 54

QpOutValCr                          : 13 21 27 29 32 37

QpInValCbCr                         : 12 21 41 43 54

QpOutValCbCr                      : 12 22 30 32 37

The signalling overhead is 226 bits for the separate Cb, Cr, and jointCbCr chroma QP tables. The separate chroma QP tables are illustrated in Figure 3.

Figure 3 relationship between qPi and proposed QpC (separate table for Cb, Cr, and jointCbCr) for class H1

## Class H2 (HDR-HLG) chroma QP table

We tested several candidate chroma QP tables for HLG intended to improve the balance between luma and chroma compared to the chroma QP table currently used for HLG CTC. Of the chroma QP tables tested, the following 3 tables are proposed as candidates for adoption:

- T1: the HLG chroma QP table originally proposed in JVET-O0433[2], which mitigates subjective chroma artifacts for low bitrate, e.g. QP37 coding in VTM5.0.

- T2: the SDR CTC AI table used in VTM6.0,

- T3: the SDR CTC RA table used in VTM6.0.

The proposed tables are illustrated in Figure 4. Among all the tables, T1 (blue curve) has smallest chroma QP values for the higher range luma QP (qPi>32), which can provide best subjective chroma quality at low bitrate. T2 (red curve) has highest chroma QP values for all QP thus it is expected to have worst chroma quality both subjectively (at low bitrate) and objectively in terms of BDrate. T3 (green curve) has lowest chroma QP in middle range (luma qPi between 16 to 32) but has larger chroma QP than T1 and smaller chroma QP than T2 in high QP range. It is therefore expected to have slightly worse chroma quality than T1 and slightly better chroma quality than T2 at low bitrate (e.g. QP37), when chroma artifacts are subjectively visible. The objective test results for the 3 candidate chroma QP tables are listed in Table 7 (compared to HM16.18) and Table 8 (compared to VTM6.0). It is interesting to see that T3 exhibits slight better BDrate improvements in U/V PSNRs than T1 although subjectively it should be the opposite. This is mainly because the BDrate is calculated with mid-range QPs (22-37) that T3 has lower chroma QP and therefore not aligned very well with subjective quality.

Figure 4 relationship between qPi and tested QpC for class H2

Table 7 BDRate of VTM6.0 versus HM16.18 for HDR HLG content

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Case | PsnrY | PsnrU | PsnrV | PsnrY | PsnrU | PsnrV |
| VTM 6.0 official | -21.15% | -37.56% | -39.83% | -28.23% | -45.86% | -49.57% |
| T1: HLG table in [2] | -21.14% | -37.56% | -39.83% | -28.18% | -45.88% | -49.43% |
| T2: VTM6.0 SDR AI | -21.58% | -33.52% | -34.98% | -28.71% | -39.55% | -44.10% |
| T3: VTM6.0 SDR RA | -20.87% | -40.01% | -41.89% | -28.29% | -46.60% | -50.74% |

Table 8 BDRate of VTM6.0 candidates versus VTM6.0 anchor for HDR HLG content

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Case | PsnrY | PsnrU | PsnrV | PsnrY | PsnrU | PsnrV |
| T1: HLG table in [2] | 0.00% | 0.00% | 0.00% | 0.07% | -0.75% | -0.53% |
| T2: VTM6.0 SDR AI | -0.52% | 6.24% | 6.67% | -0.62% | 13.18% | 12.27% |
| T3: VTM6.0 SDR RA | 0.40% | -3.85% | -3.81% | -0.02% | -1.16% | -2.80% |

We performed subjective testing on a high-quality consumer display in HLG mode (LG C8 OLED display) (no HLG reference display is available in our lab) using visually lossless HEVC bitstreams (x265 10b visually lossless mode with HLG signalling and bit rate set at 40Mbps). Compared to the VTM6.0 anchor, the visual difference between T1 and the VTM6.0 anchor is much less than what we observed previously when comparing to the VTM5.0 anchor [2], suggesting that we may no longer need to reduce chroma QP aggressively for HLG when luma QP is large. For HLG CTC, we recommend subjective evaluation be done using a professional BVM display. If there are chroma issues observed in VTM6.0 anchor at lowest bitrate, T1 is recommended. Otherwise, if there are no chroma issues in T2, T2 is recommended since it has more  luma gain.

# Conclusion

This contribution provides results to show that the recent changes in VTM 6.0 regarding chroma QPc mapping and offsets make the final chroma coding QP values for class H1 (HDR-PQ) much smaller than the final chroma QP values for VTM5.0. As a result, there is an overall loss in luma coding performance and gain in chroma performance in VTM 6.0 CTC compared with VTM 5.0.  When compared to HM16.18, luma and chroma performance is therefore more unbalanced for VTM 6.0 CTC than it was for VTM 5.0 CTC.

This contribution proposes to update the non-normative chroma QP mapping tables for HDR to improve the luma and chroma balance in VTM 6.0 for HDR common test conditions (CTC).  For class H1 (HDR-PQ), separate P3 tables are proposed for Cb, Cr, and jointCbCr.  For class H2 (HDR-HLG), we recommend subjective evaluation be done using a professional BVM display to choose one of the tables from the 2 candidate HLG chroma QP tables proposed.

# Acknowledgement

We would like to thank InterDigital for crosschecking results.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding”, JVET-O2001-vE, Gothenburg, SE, July 2019

- T. Lu, F. Pu, P. Yin, S, McCarthy, W. Husak, T. Chen, “AHG15: chroma quantization parameters QpC table”, JVET-O0433, Gothenburg, SE, 3–12 July 2019

- T. Lu, F. Pu, P. Yin, S, McCarthy, W. Husak, T. Chen, “Chroma Quantization Parameter QpC Table for HDR Signal”, JVET-N0221, Geneva, CH, Mar. 2019

- L. Kerofsky, A. Segall, S-H Kim, and K. Miska, “Color Gamut Scalable Video Coding”, JCTVC-L0334, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).