| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0339 |
| --- | --- |

| Title: | AHG15: Signalling of CU level chroma QP offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhao Wang, Yan Ye, Jiancong Luo | Tel: Email: | (+86)13716516722 baixiu.wz@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

At the 15th JVET meeting, CU level chroma QP control proposed in JVET-O1168 was adopted into the VVC draft 6. In VVC draft 6, CU level chroma QP offsets are signalled in PPS by cb_qp_offset_list, cr_qp_offset_list and joint_cbcr_qp_offset_list for Cb, Cr and joint CbCr respectively. This contribution proposes to signal the joint_cbcr_qp_offset_list only when the joint CbCr residual coding tool is enabled. Two methods are proposed in this contribution. In the first method, it is proposed to move the signaling of CU level chroma QP offsets to the slice header. In the second method, it is proposed to signal the CU level chroma QP offsets in the APS by adding a new APS type.

Introduction

In VVC draft 6, the signalling of CU level chroma QP offset lists is shown in the following Table 1, where related syntax is highlighted with yellow.

Table 1. Syntax structure of CU level chroma QP offset in VVC draft 6.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| pps_weighted_pred_flag | u(1) |
| … |  |
| } |  |

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_chroma_qp_offset_enabled_flag && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| … |  |
| } |  |

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| if( split_cu_flag ) { |  |
| … |  |
| } |  |

| palette_coding( x0, y0, cbWidth, cbHeight, startComp, numComps ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType != DUAL_TREE_CHROMA && palette_escape_val_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

cu_chroma_qp_offset_enabled_flag equal to 1 specifies that the cu_chroma_qp_offset_flag may be present in the transform unit syntax. cu_chroma_qp_offset_enabled_flag equal to 0 specifies that the cu_chroma_qp_offset_flag is not present in the transform unit syntax. When ChromaArrayType is equal to 0, it is a requirement of bitstream conformance that the value of cu_chroma_qp_offset_enabled_flag shall be equal to 0.

cu_chroma_qp_offset_subdiv specifies the maximum cbSubdiv value of coding units that convey cu_chroma_qp_offset_flag. The value range of cu_chroma_qp_offset_subdiv is specified as follows:

- If slice_type is equal to I, the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2 * ( log2_ctu_size_minus2 − log2_min_qt_size_intra_slice_minus2 + MaxMttDepthY ), inclusive.

- Otherwise (slice_type is not equal to I), the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2* ( log2_ctu_size_minus2 − log2_min_qt_size_inter_slice_minus2 + MaxMttDepthY ), inclusive.

When not present, the value of cu_chroma_qp_offset_subdiv is inferred to be equal to 0.

chroma_qp_offset_list_len_minus1 plus 1 specifies the number of cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ], syntax elements that are present in the PPS. The value of chroma_qp_offset_list_len_minus1 shall be in the range of 0 to 5, inclusive.

cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ], specify offsets used in the derivation of Qp′Cb, Qp′Cr, and Qp′CbCr, respectively. The values of cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ] shall be in the range of −12 to +12, inclusive.

cu_chroma_qp_offset_flag when present and equal to 1, specifies that an entry in the cb_qp_offset_list[ ] is used to determine the value of CuQpOffsetCb, a corresponding entry in the cr_qp_offset_list[ ] is used to determine the value of CuQpOffsetCr, and a corresponding entry in the joint_cbcr_qp_offset_list[ ] is used to determine the value of CuQpOffsetCbCr. cu_chroma_qp_offset_flag equal to 0 specifies that these lists are not used to determine the values of CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr.

cu_chroma_qp_offset_idx, when present, specifies the index into the cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ] that is used to determine the value of CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr. When present, the value of cu_chroma_qp_offset_idx shall be in the range of 0 to chroma_qp_offset_list_len_minus1, inclusive. When not present, the value of cu_chroma_qp_offset_idx is inferred to be equal to 0.

When cu_chroma_qp_offset_flag is present, the following applies:

- The variable IsCuChromaQpOffsetCoded is set equal to 1.

- The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are derived as follows:

- If cu_chroma_qp_offset_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ cu_chroma_qp_offset_idx ]

CuQpOffsetCr = cr_qp_offset_list[ cu_chroma_qp_offset_idx ]

CuQpOffsetCbCr = joint_cbcr_qp_offset_list[ cu_chroma_qp_offset_idx ]

- Otherwise (cu_chroma_qp_offset_flag is equal to 0), CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are all set equal to 0.

When cu_chroma_qp_offset_enabled_flag is equal to 1, CU level chroma QP offset lists are signalled. However, the signalling of joint_cbcr_qp_offset_list doesn’t consider whether the coding tool of joint CbCr is enabled for the current video sequence, which is controlled by the element sps_joint_cbcr_enabled_flag signalled in the SPS. However, directly adding the condition of sps_joint_cbcr_enabled_flag before signalling joint_cbcr_qp_offset_list will introduce parsing dependency between PPS and SPS syntax elements, which is undesirable.

Proposed method

Method 1: Signalling the CU level chroma QP offset lists in slice header

In this proposed method, signalling of CU level chroma QP offset lists is moved from PPS to slice header. The proposed syntax structure is shown in the following Table 2, and the semantics are kept identical.

Table 2. Modified syntax structure of PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| ….. |  |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| pps_weighted_pred_flag | u(1) |
| … |  |
| } |  |

Table 3. Proposed syntax structure of slice header.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| chroma_qp_offset_enabled_flag | u(1) |
| if( chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Method 2: Signalling the CU level chroma QP offset lists in adaptation parameter set

A drawback of method 1 is that it could significantly increase the number of bits in the slice header, especially in the case a picture is coded using multiple slices. Therefore, an alternative method is proposed to move the signalling of the chroma QP offset lists into the adaptation parameter set (APS). In VVC draft 6, APS includes three types: ALF_APS, LMCS_APS and SCALING_APS, which carry the ALF parameters, luma mapping and chroma scaling (LMCS) parameters and scaling list data parameters, respectively. In this proposal, a new APS type CQPO_APS is proposed to signal the CU level chroma QP offset lists, as shown in the following. When aps_params_type is equal to CQPO_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

Table 4. APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | SCALING list data parameters |
| 3 | CQPO_APS | CU level chroma QP offsets |
| 4..7 | Reserved | Reserved |

Table 5. Proposed syntax structure of APS.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = CQPO_APS ) |  |
| cqpo_data() |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 6. CU level chroma QP offsets data syntax

| cqpo_data () { | Descriptor |
| --- | --- |
| cu_chroma_qp_offset_subdiv | ue(v) |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | ue(v) |
| cr_qp_offset_list[ i ] | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | u(1) |
| } |  |
| } |  |

Table 7. Proposed syntax structure of CU level chroma QP offset.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| slice_chroma_qp_offset_present_flag | u(1) |
| if( slice_chroma_qp_offset_present_flag ) |  |
| slice_cqpo_aps_id | u(3) |
| … |  |
| } |  |

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| … |  |
| if(cu_chroma_qp_offset_enabled_flag slice_chroma_qp_offset_present_flag && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| … |  |
| } |  |

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| … |  |
| if(cu_chroma_qp_offset_enabled_flag slice_chroma_qp_offset_present_flag && qgOnC && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| if( split_cu_flag ) { |  |
| … |  |
| } |  |

| palette_coding( x0, y0, cbWidth, cbHeight, startComp, numComps ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType != DUAL_TREE_CHROMA && palette_escape_val_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) { |  |
| if(cu_chroma_qp_offset_enabled_flag slice_chroma_qp_offset_present_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if(cu_chroma_qp_offset_enabled_flag slice_chroma_qp_offset_present_flag && !IsCuChromaQpOffsetCoded) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

slice_chroma_qp_offset_present_flag equal to 1 specifies that the CU level chroma QP offset is present for the current slice. slice_chroma_qp_offset_present_flag equal to 0 specifies that the CU level chroma QP offset is not present for the current slice. When slice_chroma_qp_offset_present_flag is not present, it is inferred to be equal to 0.

slice_cqpo_aps_id specifies the adaptation_parameter_set_id of the CQPO_APS that the slice refers to. The TemporalId of the CQPO_APS NAL unit having adaptation_parameter_set_id equal to slice_cqpo_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

When present, the value of slice_cqpo_aps_id shall be the same for all slices of a picture.

In the CTU/CU/TU level, the syntax regarding CU level chroma QP adjustment stays unchanged from VVC draft 6.

Conclusion

In this contribution, it is proposed to move the signalling of chroma QP offset lists from PPS into the APS (or the slice header, if slice header signalling overhead is not a big concern). This allows the signalling of the joint CbCr QP offset list to be conditioned upon whether the joint CbCr coding tool is used. Further, the proposed method is better aligned with the QP scaling list signalling, which is carried in APS in VVC draft 6.

References

Yu Han, Adarsh Ramasubramonian, Alican Nalci, “Cu level chroma QP control for VVC,” JVET-O1168, July 2019.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).