| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0341 |
| --- | --- |

| Title: | Non-CE4: Coding of interpolation filter index in non-merge inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Franck Galpin, Fabrice Le-Leannec, Edouard François 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 philippe.bordes@interdigital.com, fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VTM-6.0, in non-merge inter mode, the adaptive interpolation filter (AIF) index (hpelifidx) is inferred from the resolution of the motion vector difference (amvr_precision_idx) value.

It is proposed to encode the adaptive interpolation filter (AIF) index (hpelifidx) in case the amvr_precision_idx is equal to half-pel to increase the VVC specification capability.

It is reported that:

- when the VTM-6.0 encoder AIF algorithm is unchanged (hpelifidx derived from amvr_precision_idx), the performance of VTM-6.0 is almost unchanged: XX% in RA, XX% in LDB, and the encoder/decoder complexity is… compared to VTM-6.0 anchors.

- when the VTM-6.0 encoder AIF algorithm is modified so as to select best value of hpelifidx with RDO, the performance of VTM-6.0 is: XX% in RA, XX% in LDB, and the encoder/decoder complexity is… compared to VTM-6.0 anchors.

# Introduction

In VTM-6.0 [1], in non-merge inter mode, in case of the resolution of the adaptive motion vector difference (amvr_precision_idx) is HALF_PEL, the adaptive interpolation filter (AIF) index (hpelifidx) is set equal to 1, in other cases it is set equal to 0. The value of the adaptive motion vector difference resolution is stored into the motion-info buffer after the CU has been reconstructed (imv).

In case of non-merge inter modes, the value of hpelIfIdx is derived from amvr_precision_idx as:

hpelIfIdx = (amvr == HALF_PEL) ? 1 : 0		(eq.1)

However, the value of hpelifidx equal to 1 will impact the motion compensation process only if the value of the reconstructed motion vector (mv) component(s) is(are) half-pel. It may happen the amvr_precision_idx is HALF_PEL but the reconstructed mv may be not half-pel. This may reduce the potentiality/flexibility of an encoder that could exploit the selection of hpelifidx independently to amvr_precision_idx.

# Proposal

In non-merge inter mode, in case of the resolution of the adaptive motion vector difference (amvr_precision_idx) is HALF_PEL, it is proposed to encode the value of the adaptive interpolation filter index (hpelifidx) so that the selection of the regular interpolation filter (hpelifidx=0) may possible.

We implemented the proposed syntax with two encoder implementations:

Test-1: the VTM-6.0 encoder AIF algorithm is unchanged (hpelifidx derived from amvr_precision_idx, see eq.1).

Test-2: the VTM-6.0 encoder AIF algorithm is modified so as to select best value of hpelifidx with RDO.

Table 1: summary of proposed interpolation filter index coding (test-1).

|  | Proposal difference with VTM-6.0 (test-1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.05% | 0.02% |  |  |  |  |  |  |  |
| Class C | -0.01% | 0.01% | 0.05% |  |  | -0.07% | 0.07% | 0.09% |  |  |
| Class E |  |  |  |  |  | -0.14% | 0.20% | 0.49% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.08% | 0.01% |  |  | 0.02% | 0.34% | 0.45% |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  |  |  |  |  |

Table 2: summary of proposed interpolation filter index coding (test-2).

|  | Proposal difference with VTM-6.0 (test-1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.02% | -0.04% |  |  |  |  |  |  |  |
| Class C | -0.01% | 0.07% | 0.02% |  |  | -0.03% | 0.06% | 0.08% |  |  |
| Class E |  |  |  |  |  | -0.05% | 0.28% | 0.51% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | -0.06% | -0.08% |  |  | -0.09% | 0.01% | -0.17% |  |  |
| Class F | 0.00% | 0.01% | -0.04% |  |  |  |  |  |  |  |

# References

- Text of Versatile Video Coding (Draft 6), document JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- High efficiency video coding (HEVC), combined format range extensions (RExt), scalability (SHVC), and multi-view (MV-HEVC) extensions, ITU-T H.265, April 2015.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).