| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0345-v1 |
| --- | --- |

| Title: | Low-Delay B encoder configuration proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, R. Jullian, E. François | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

The coding gain of VVC over HEVC much higher in random access configuration than in low-delay B. This contribution proposes a new low-delay B encoder configuration, targeting improved coding efficiency. Basically, the GOP size is extended to 8 instead of 4. The proposed LDB GOP structure reportedly leads to -X.XX%, -Y.YY%, -Z.ZZ% BD-rate gain in Y, Cb and Cr components respectively, over the current CTC LDB configuration.

# Proposed LDB configuration

The proposed LDB configuration basically consists in extending the GOP size from 4 to 8.  Additionally, the reference pictures arrangement with the new GOP size is show by the following Table 1.

Table 1: Partial dump of the LDB configuration proposed for increased compression efficiency

… GOPSize : 8 # GOP Size (number of B slice = GOPSize-1) IntraQPOffset : -1 LambdaFromQpEnable : 1 # see JCTVC-X0038 for suitable parameters for IntraQPOffset, QPoffset, QPOffsetModelOff, QPOffsetModelScale when enabled # Type POC QPoffset QPOffsetModelOff QPOffsetModelScale CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 temporal_id #ref_pics_active_L0 #ref_pics_L0 reference_pictures_L0 #ref_pics_active_L1 #ref_pics_L1 reference_pictures_L1 Frame1: B 1 5 -6.5 0.2590 0 0 1.0 0 0 0 4 4 1 9 17 25 4 4 1 5 9 13 Frame2: B 2 4 -6.5 0.2590 0 0 1.0 0 0 0 4 4 1 2 10 18 4 4 1 2 6 10 Frame3: B 3 5 -6.5 0.2590 0 0 1.0 0 0 0 4 4 1 3 11 19 4 4 1 3 7 11 Frame4: B 4 4 -6.5 0.2590 0 0 1.0 0 0 0 4 4 1 4 12 20 4 4 1 4 8 12 Frame5: B 5 5 -6.5 0.2590 0 0 1.0 0 0 0 4 4 1 5 13 21 4 4 1 5 9 13 Frame6: B 6 4 -6.5 0.2590 0 0 1.0 0 0 0 4 4 1 6 14 22 4 4 1 6 10 14 Frame7: B 7 5 -6.5 0.2590 0 0 1.0 0 0 0 4 4 1 7 15 23 4 4 1 7 11 15 Frame8: B 8 1 0.0 0.0 0 0 1.0 0 0 0 4 4 1 8 16 24 4 4 1 8 12 16 …

# Obtained results

Table 2 depicts the performances obtained with the proposed LDB coding structure, compared to the VTM6 in the JVET coding test conditions [2]. As can be seen, -X.XX% gain is obtained in luma, -Y.YY% and -Z.ZZ% gain are obtained in Cb and Cr components.

Moreover, the coding and decoding time of the VTM are not increased with the new proposed LDB configuration.

Table 2: compression performances obtained with the proposed LDB GOP structure

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -3.10% | -4.26% | -4.03% | 94% | 92% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

A new LDB encoder configuration is proposed, which leads to -X.XX% average gain in Luma. The proposed configuration basically consists in a GOP size extended to 8, together with an adhoc arrangement of reference pictures. It is suggested to use this proposed configuration in the JVET CTC.

# References

[ 1] B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[ 2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring , “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, April 2019

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).