| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0347 |
| --- | --- |

| Title: | Comments on maximum MTT hierarchy depth and Min Cb size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Poirier, F. Galpin, F. Le Léannec, F. Urban, | Tel: Email: | tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VTM-6, the maximum multi-type tree (mtt) depth is signaled in the SPS and slice header as a fixed value, respectively for inter slice, intra luma slice and intra chroma slice (case of dual tree active for I slices). The value of the maximum mtt depth is in the range 0 to CtbSize – MinCbLog2SizeY. This range does not allow to reach the minimum Cb size using only mtt split in some configurations. This contribution proposes to increase the maximum range of max_mtt_hierarchy_depth and to define a constraint on the range for log2_min_luma_coding_block_size_minus2 syntax element.

# Introduction

In VTM-6, three SPS syntax elements are dedicated to specifying the maximum multi-type tree depth:

- sps_max_mtt_hierarchy_depth_inter_slice,

- sps_max_mtt_hierarchy_depth_intra_slice_luma and chroma

These 3 SPS parameters can be overridden by their analogous syntax elements in the slice header:

- slice_max_mtt_hierarchy_depth_luma

- slice_max_mtt_hierarchy_depth_chroma (present in the case of an I slice coded in dual tree configuration).

In the CTC, CtbSize is equal to 128 and MinCbLog2SizeY is equal to 4, thus max_mtt_hierarchy_depth shall be in the range 0 to 5.

In other conditions where CtbSize is equal to 32, MinCbLog2SizeY is equal to 8, max_mtt_hierarchy_depth shall be in the range 0 to 2. In this case, there is a lack of flexibility in VVC.

In this contribution, for each of the B, I-Luma and I-Chroma slice-components, it is proposed to specify that the value for sps_max_mtt_hierarchy_depth_intra_slice_luma shall be in the range 0 to 2*(CtbSize - MinCbSize).  This way, it allows to reach the minimum Cb size using only mtt split in all configurations.

Furthermore, MinCbLog2SizeY is allowed to be greater than CtbLog2SizeY, this may leads to impossible value for max_mtt_hierarchy_depth.

Another issue is that log2_min_luma_coding_block_size_minus2 value is not restricted in the specification and an evil encoder may define a CTU size of 128, a minimum block size of 128 and qtbtt_dual_tree_intra_flag equal to 1: in this case all Cb would be implicitly split into 64x64 blocks in the intra slices.

# SPS and Slice Header syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_ctu_size_minus5 | u(2) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slices != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| … |  |
| } |  |

sps_max_mtt_hierarchy_depth_inter_slice specifies the default maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS. When partition_constraints_override_ flag is equal to 1, the default maximum hierarchy depth can be overridden by slice_max_mtt_hierarchy_depth_luma present in the slice header of the slices referring to the SPS. The value of sps_max_mtt_hierarchy_depth_inter_slice shall be in the range of 0 to 2*(CtbLog2SizeY − MinCbLog2SizeY), inclusive.

sps_max_mtt_hierarchy_depth_intra_slice_luma specifies the default maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_ flag is equal to 1, the default maximum hierarchy depth can be overridden by slice_max_mtt_hierarchy_depth_luma present in the slice header of the slices referring to the SPS. The value of sps_max_mtt_hierarchy_depth_intra_slice_luma shall be in the range of 0 to 2*(CtbLog2SizeY − MinCbLog2SizeY), inclusive.

sps_max_mtt_hierarchy_depth_intra_slice_chroma specifies the default maximum hierarchy depth for chroma coding units resulting from multi-type tree splitting of a chroma quadtree leaf with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_ flag is equal to 1, the default maximum hierarchy depth can be overridden by slice_max_mtt_hierarchy_depth_chroma present in the slice header of the slices referring to the SPS.  The value of sps_max_mtt_hierarchy_depth_intra_slice_chroma shall be in the range of 0 to 2*(CtbLog2SizeY − MinCbLog2SizeY), inclusive. When not present, the value of sps_max_mtt_hierarchy_depth_intra_slice_chroma is inferred to be equal to 0.

slice_max_mtt_hierarchy_depth_luma specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in the current slice. The value of slice_max_mtt_hierarchy_depth_luma shall be in the range of 0 to 2*(CtbLog2SizeY − MinCbLog2SizeY), inclusive. When not present, the value of slice_max_mtt_hierarchy_depth_luma is inferred as follows:

- If slice_type equal to 2 (I), the value of slice_max_mtt_hierarchy_depth_luma is inferred to be equal to sps_max_mtt_hierarchy_depth_intra_slice_luma

- Otherwise (slice_type equal to 0 (B) or 1 (P)), the value of slice_max_mtt_hierarchy_depth_luma is inferred to be equal to sps_max_mtt_hierarchy_depth_inter_slice.

slice_max_mtt_hierarchy_depth_chroma specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf with treeType equal to DUAL_TREE_CHROMA in the current slice. The value of slice_max_mtt_hierarchy_depth_chroma shall be in the range of 0 to 2*(CtbLog2SizeY − MinCbLog2SizeY), inclusive. When not present, the values of slice_max_mtt_hierarchy_depth_chroma is inferred to be equal to sps_max_mtt_hierarchy_depth_intra_slices_chroma.

log2_min_luma_coding_block_size_minus2 plus 2 specifies the minimum luma coding block size.
The value of log2_min_luma_coding_block_size_minus2 shall be in the range of 0 to sps_max_luma_transform_size_64_flag  ?  4  :  3, inclusive.

# Conclusion

This contribution proposes to increase the range and the maximum value for max_mtt_hierarchy_depth and to define a range for log2_min_luma_coding_block_size_minus2 syntax elements. It is shown that the actual syntax disallows to reach the minimum coded block size in some configurations or may lead to undefine behaviour for some value of minimum coded block size.

# References

[ 1] B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).