| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0348 |
| --- | --- |

| Title: | On BT/TT flag signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Poirier, F. Galpin, F. Le Léannec, F. Urban, | Tel: Email: | tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VVC Draft 6, Binary Tree and Ternary Tree splits are always enabled. This contribution proposes to add high-level syntax element to control BT and TT splits enabling at SPS and slice level. The proposal claims that these new syntax elements allow more flexibility at the encoder.

# Introduction

In VTM-6, Binary Tree and Ternary Tree splits are always enabled, to be more flexible up to 6 flags are defined to control BT and TT splits enabling for intra luma, inter and intra chroma slices.

# Proposed Modification

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| …… |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_ctu_size_minus5 | u(2) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_bt_enabled_flag _inter_slice | u(1) |
| sps_tt_enabled_flag_inter_slice | u(1) |
| if( sps_bt_enabled_flag_inter_slice ¦¦ sps_tt_enabled_flag_inter_slice) { |  |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| } |  |
| sps_bt_enabled_flag_intra_slice_luma | u(1) |
| sps_tt_enabled_flag_intra_slice_luma | u(1) |
| if( sps_bt_enabled_flag_intra_slice_luma ¦¦ sps_tt_enabled_flag_intra_slice_luma) { |  |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slices != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_bt_enabled_flag_intra_slice_chroma | u(1) |
| sps_tt_enabled_flag_intra_slice_chroma | u(1) |
| if(sps_bt_enabled_flag_intra_slice_chroma ¦¦ sps_tt_enabled_flag_intra_slice_chroma){ |  |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| } |  |
| if ( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_max_luma_transform_size_64_flag | u(1) |

Same modifications are proposed for slice header control.

# Conclusion

This contribution proposes to add control over the enabling of BT and TT splits. It is asserted that these new controls allow a higher flexibility at the encoder.

# References

[ 1] B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).