| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0350-r1 |
| --- | --- |

| Title: | CE6 Related: Modified LFNST Index Coding For Fast Encoder Implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Franck GALPIN Tangi POIRIER Ya CHEN | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In CE6-1.1 tests of JVET-P0041, fast LFNST implementations are proposed by allowing only one LFNST kernel per intra-prediction mode, and alternate LFNST kernels among the intra modes. It is reported that these methods reduce the encoder time to 90% with AI luma loss of 0.11%. The tests purely target encoder time reduction and therefore the decoding time is unchanged. This contribution proposes to code the second bin of LFNST index using CABAC engine and map its value according to the intra prediction mode or MIP mode (even or odd) during the decoding process. A fast encoding method is achieved by restricting the RD search to the first LFNST kernels, where the second bin is coded as 0. Two tests are carried out to study the fast method (second bin always 0) and regular method (second bin 0 or 1)

- The fast method achieves also 90% encoding time and 0.11% luma loss in AI

- The regular method achieves 100% encoding time and 0.01% luma loss in AI

Further tests show that if MIP is not considered for LFNST index mapping and default context initialization is used, the method achieves similar reduction in coding time while luma loss is increased to 0.14 in AI.

# Introduction

CE6-1.1a and CE6-1.1d propose signalling only one flag (lfnst_idx) that enables LFNST for a CU. If lfnst_idx is true, LFNST is applied. LFNST transform matrix selection is derived from the intra prediction mode as follows:

CE6-1.1a:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel index lfnstIdx, where lfnstIdx is 1 or 2, is calculated using the variable predModeIntra specifying the intra prediction mode for LFNST set selection as follows:

If predModeIntra is smaller than or equal to 34, lfnst_idx is set to be “predModeIntra % 2 == 0 ? 1 : 2.”

Otherwise, lfnst_idx is set to be “predModeIntra % 2 == 0 ? 2 : 1.”

Method described in JVET-O0350:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel index lfnstIdx, where lfnstIdx is 1 or 2, is calculated using the variable predModeIntra specifying the intra prediction mode for LFNST set selection as follows:

lfnstIdx = ( predModeIntra % 2 ) + 1

lfnstIdx = ( predModeIntra % 2 ) + 1

Effectivly, CE6-1.1a and CE6-1.1d distribute the two LFNST kernels per intra mode between the neighboring modes, such that even modes use the first kernel and odd modes use the second kernel (and vice versa). This essentially reduce the encoding time since the number of RD is reduced since the second option of LFNST is removed.

Both CE6-1.1a and CE6-1.1d are purely targeting encoding time improvement. That is, the decoding is not time improved. Furthermore, the methods leads to a non-trivial coding loss because of imposing encoder constraints. Therefore, this contribution proposes an alternative LFSNT coding method that enables fast encoding method as well as regular coding. The motivation behind it is to provide a flexible encoder design that can use fast method with 90% Enc time and 0.11% luma AI loss, or 100% Enc time with 0.01% luma AI loss.

# Proposal

LFNST index is a two bins index that is binarized as:

| lfnst_idx | Code | Meaning |
| --- | --- | --- |
| 0 | 0 | no lfnst |
| 1 | 10 | lfnst kernel 1 |
| 2 | 11 | lfnst kernel 2 |

The second lfnst bin is selecting the LFNST index. It is bypass coded in VTM6.0. This contribution proposes to code this bin with CABAC engine. When LFNST kernel process is invoked, the lfnst index is mapped as:

| lfnst_idx | Mapped value |
| --- | --- |
| 0 | 0 |
| 1 | predModeIntra %2 ? 1 : 2 |
| 2 | predModeIntra %2 ? 2 : 1 |

By doing so, a fast encoder method can be achieved by restricting the RD search to not use lfnst_idx equal to 2. In this case, lfnst_idx is interpreted the same way as CE6-1.1d. Otherwise, the regular RD search allows both lfnst_idx 1 and 2.

That advantage of the proposed lfnst_idx coding is that the fast method becomes an encoder method and the regular RD search can be retained to achieve the maximum coding gain.

As described in JVET-P0349, MIP modes can be considered when computing the odd and even indexes. Therefore, this contribution has two proposals: With and without MIP adaptation. Their corresponding specification changes are:

# Proposal 1: Index mapping with MIP adaptation

### 8.7.4 Transformation process for scaled transform coefficients

#### 8.7.4.2 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-965)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-966)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-967)

nLfnstSize = 1  <<  log2LfnstSize		(8-968)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-969)

- When intra_mip_flag[ xTbComp ][ yTbComp ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

- When predModeIntra is equal to either INTRA_LT_CCLM, INTRA_L_CCLM, or INTRA_T_CCLM, predModeIntra is set equal to IntraPredModeY[ xTbY + nTbW / 2 ][ yTbY + nTbH / 2 ].

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

…

#### 8.7.4.3 Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

lfnstIdx is mapped using the table below:

| lfnst_idx | Mapped value |
| --- | --- |
| 1 | predModeIntra %2 ? 1 : 2 |
| 2 | predModeIntra %2 ? 2 : 1 |

- When intra_mip_flag[ xTbComp ][ yTbComp ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

…

# Proposal 2: Index mapping without MIP adaptation

In method 1, MIP intra mode is taken into when LFNST index mapping is performed. This is because MIP consideration come with small RD gain, as presented in JVET-P0349. Method 2 does not consider MIP so that the change to VTM specification is minimized.  The corresponding specification change is given below:

#### 8.7.4.3 Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

lfnstIdx is mapped using the table below:

| lfnst_idx | Mapped value |
| --- | --- |
| 1 | predModeIntra %2 ? 1 : 2 |
| 2 | predModeIntra %2 ? 2 : 1 |

# Experimental Results

The two proposals were implemented on top of VTM6.0. The tests were conducted under the CTC. The first test is the fast method, in which the encoder is restricted to the first kernels of LFNST, and the second test is the regular method where the encoder can select between the two kernels of LFNST. The results of the each test are given below:

## Proposal 1: Index mapping with MIP adaptation

Proposal 1 was tested with two options: with context optimization and without context optimization. In the first option, the context for coding second bin of LFNST index is optimized to for a fast encoder that constantly encodes it as zero, while in the second option, the context is initializes with default context initialization (CNU).

## Results with context optimization

In this test, the CABAC context for coding the second bin on LFNST is initialized with values that are optimized for mostly being coded as zero. The results provided below:

Table 1 Results of first test (Fast Encoder) of Proposal 1 with context optimization

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.76% | 1.07% | 89% | 98% |
| Class A2 | 0.04% | 0.64% | 0.72% | 89% | 100% |
| Class B | 0.08% | 0.68% | 0.81% | 90% | 102% |
| Class C | 0.11% | 0.44% | 0.74% | 90% | 100% |
| Class E | 0.13% | 0.67% | 0.73% | 91% | 100% |
| Overall | 0.11% | 0.63% | 0.81% | 90% | 100% |
| Class D | 0.05% | 0.45% | 0.63% | 88% | 101% |
| Class F | 0.12% | 0.33% | 0.40% | 90% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.50% | 0.64% | 98% | 106% |
| Class A2 | 0.06% | 0.35% | 0.43% | 99% | 110% |
| Class B | 0.05% | 0.60% | 0.51% | 94% | 99% |
| Class C | 0.05% | 0.32% | 0.33% | 96% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.46% | 0.47% | 96% | 101% |
| Class D | 0.02% | 0.49% | 0.57% | 97% | 90% |
| Class F | 0.05% | 0.05% | 0.20% | 98% | 94% |

The results are in line with the results reported in CE6-1.1a and CE6-1.1d, that is, the same reduction in the encoding time and same coding loss.

The results of the second test are shown below:

Table 2 Results of second test (regular Encoder) of Proposal 1 with context optimization

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.01% | 100% | 99% |
| Class A2 | 0.02% | 0.06% | 0.05% | 101% | 102% |
| Class B | 0.01% | 0.11% | -0.01% | 99% | 100% |
| Class C | 0.02% | 0.03% | 0.16% | 100% | 100% |
| Class E | 0.01% | 0.06% | 0.07% | 101% | 101% |
| Overall | 0.01% | 0.07% | 0.06% | 100% | 100% |
| Class D | 0.01% | 0.18% | 0.31% | 99% | 101% |
| Class F | 0.03% | 0.11% | 0.02% | 96% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.01% | 102% | 100% |
| Class A2 | 0.03% | 0.03% | 0.13% | 103% | 102% |
| Class B | -0.01% | 0.14% | -0.09% | 97% | 99% |
| Class C | 0.00% | 0.14% | 0.02% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.00% | 100% | 99% |
| Class D | -0.03% | 0.08% | 0.16% | 101% | 88% |
| Class F | -0.02% | -0.08% | 0.05% | 100% | 96% |

The results show minor impact on the CTC coding conditions. Therefore, the loss of the fast method can be compensated with regular RD search, unlike the methods proposed in CE6-1.1.

### Results without context optimization

The same experiments were repeated without context optimization of the second LFNST bin, where the default context initialization (CNU) is used. The following results are obtained:

Table 3 Results of first test (Fast Encoder) of Proposal 1 without context optimization

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.81% | 1.03% | 89% | 99% |
| Class A2 | 0.04% | 0.65% | 0.76% | 89% | 100% |
| Class B | 0.10% | 0.74% | 0.85% | 91% | 100% |
| Class C | 0.14% | 0.45% | 0.73% | 91% | 102% |
| Class E | 0.17% | 0.61% | 0.69% | 91% | 99% |
| Overall | 0.13% | 0.65% | 0.81% | 90% | 100% |
| Class D | 0.11% | 0.54% | 0.62% | 92% | 104% |
| Class F | 0.17% | 0.28% | 0.27% | 91% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.37% | 0.55% | 99% | 100% |
| Class A2 | 0.08% | 0.37% | 0.45% | 100% | 100% |
| Class B | 0.09% | 0.59% | 0.56% | 94% | 99% |
| Class C | 0.13% | 0.33% | 0.49% | 97% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.43% | 0.52% | 97% | 98% |
| Class D | 0.00% | 0.15% | 0.27% | 98% | 89% |
| Class F | 0.09% | 0.22% | 0.29% | 97% | 95% |

Table 4 Results of second test (regular Encoder) of Proposal 1 without context optimization

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.16% | 0.05% | 98% | 98% |
| Class A2 | 0.00% | 0.06% | 0.04% | 99% | 101% |
| Class B | 0.00% | 0.03% | 0.03% | 102% | 102% |
| Class C | 0.01% | -0.15% | 0.08% | 101% | 103% |
| Class E | -0.01% | 0.04% | 0.04% | 99% | 99% |
| Overall | 0.00% | -0.04% | 0.05% | 100% | 101% |
| Class D | 0.00% | -0.02% | -0.10% | 99% | 103% |
| Class F | -0.01% | -0.02% | 0.02% | 97% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.02% | 103% | 106% |
| Class A2 | 0.04% | -0.11% | -0.05% | 105% | 109% |
| Class B | -0.01% | 0.09% | 0.18% | 98% | 98% |
| Class C | -0.01% | 0.03% | -0.01% | 101% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.05% | 101% | 100% |
| Class D | -0.07% | 0.04% | 0.14% | 101% | 88% |
| Class F | -0.05% | -0.12% | 0.00% | 100% | 91% |

The results show that the context optimization brings slight benefit for the fast encoder, while adds 0.01 loss to the regular encoder.

## Proposal 2: Index mapping without MIP adaptation

This is proposed to have less change to the current specification of VVC. The same testing conditions are used to test the fast encoder and regular encoder. The results are given in the tables below:

Table 5 Results of first test (Fast Encoder) of Proposal 2 without context optimization

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22% | 0.83% | 1.06% | 89% | 103% |
| Class A2 | 0.05% | 0.64% | 0.76% | 91% | 104% |
| Class B | 0.10% | 0.75% | 0.83% | 90% | 101% |
| Class C | 0.16% | 0.40% | 0.64% | 89% | 98% |
| Class E | 0.18% | 0.70% | 0.60% | 91% | 101% |
| Overall | 0.14% | 0.66% | 0.78% | 90% | 101% |
| Class D | 0.10% | 0.52% | 0.65% | 90% | 99% |
| Class F | 0.17% | 0.38% | 0.27% | 86% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.36% | 0.58% | 99% | 99% |
| Class A2 | 0.08% | 0.40% | 0.46% | 100% | 101% |
| Class B | 0.09% | 0.56% | 0.59% | 94% | 99% |
| Class C | 0.11% | 0.24% | 0.30% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.41% | 0.48% | 97% | 99% |
| Class D | 0.05% | 0.34% | 0.28% | 98% | 88% |
| Class F | 0.07% | 0.22% | 0.32% | 98% | 96% |

Comparing this results with the one obtained with MIP adaptation, we can see that 0.01% luma AI gain is achieved with the fast encoder. Furthermore, class A1 loss can be reduced from 0.19% to 0.14% in RA.

# Conclusion

The contribution proposes coding the second bin of lfnst_idx using CABAC, while alternatively interpreting the index during the transform kernel selection stage, such that the kernels are distributed between even and odd intra prediction modes. The experimental results show that the proposed coding allows fast encoding method with similar performance to CE6-1.1 as well as regular coding with small loss.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).