| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 01–11 October 2019 | Document: JVET-P0351-v2 |
| --- | --- |

| Title: | [AHG12] On Subpictures merging for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael Ouedraogo Franck Denoual Frédéric Mazé | Email: | nael.ouedraogo@crf.canon.fr franck.denoual@crf.canon.fr frederic.maze@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution addresses a technical issue encountered with VVC subpicture merging coming from pictures with size that is not multiple of CTB size. The problem occurs when slices of a subpicture initially located on a right or bottom boundary of the picture are decoded at a new location in a merged bitstream. The CTB split inference of the CTBs on the last CTB row or column of the merged subpictures is different in function of the new location of the slice. This split process constraints the new decoding location of subpicture and thus limits the merging possibilities of subpictures.

This contribution proposes to define “split inference boundaries” for subpicture. These boundaries are signaled in the SPS and modify the split inference of the CTBs that they cross. In addition, it is proposed to signal in the PPS, bands of pixels which are excluded from the conformance window to remove the band of non conformant pixels in CTBs crossed by the “split inference boundaries”. It is asserted that this would remove the limitation on the merging possibilities of subpictures.

The modifications of this third version of this proposal are the following:

- Revise the split inference condition of split_cu_flag

- Fix ranges of syntax elements related to the position of the exclusion bands

- Add proposal option 1 to signal one vertical and one horizontal exclusion band instead of multiple (option 2)

- Attach the full specification text for option 1

# Introduction

During the creation of a new video stream with subpictures taken from different video streams , rightmost or bottom subpictures may be located elsewhere in a merged bitstream as represented in the Figure 1. In this example, the rightmost subpictures 2 and 4 of the HQ bitstream are placed at leftmost position in the merged stream.

Figure 1: Subpicture and bitstream merging

When the picture width (respectively the height) of the bitstream (for example the HQ bitstream) is not a multiple of CTB width (resp. height), the last column (resp. row) of CTB are automatically split and skipped to avoid coding the coding blocks outside of the picture. In VVC, this splitting inference relies on the comparison of the coding block location and size with the actual width and height of the picture (pic_width_in_luma_samples and pic_height_in_luma_sample).

As a result, when the slices initially located at a rightmost or bottom subpicture are moved elsewhere, their coding block locations change and the splitting inference process may fail to determine the skipped coded blocks.

As a result, the picture size of the video sequence may limit the merging operations that can be done without breaking the decoding of subpictures.

# Proposal

## Principle

We propose additional signaling to solve the problem of split inference when merging subpictures coming from right or bottom border of a picture. We define vertical and horizontal boundaries that infer the split of the coding tree blocks that they cross similarly to the split inference on picture boundaries done during encoding of these right or bottom subpictures. We call these new boundaries “split inference boundaries”.

To keep complexity reasonable, we define these boundaries for the whole picture (and not per subpicture) in the SPS.

Since this mechanism mainly addresses the bitstream merging of independent subpictures, we propose the following constraints:

First, it is a requirement that the subpictures crossed by split inference boundaries have a subpic_treated_as_pic_flag equal to 1 and loop_filter_across_subpic_enabled_flag equal to 0.

Second, it is a requirement that the vertical split inference boundary crosses CTBs in the last CTB column of subpictures. Similarly, the horizontal split inference boundary should cross only CTBs of the last CTB row of subpictures.

## SPS syntax

The syntax of the SPS is changed to specify location of vertical and horizontal split inference boundaries that span across the whole picture (yellow highlights).

| sequence_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| subpic_split_inference_flag | u(1) |
| if (subpic_split_inference_flag ) { |  |
| split_inference_boundary_pos_x | u(13) |
| split_inference_boundary_pos_y | u(13) |
| } |  |
| } |  |
| […] |  |

subpic_split_inference_flag is a flag indicating the use of a subpicture splitting inference. subpic_split_inference_flag equal to 1 indicates the presence of split_inference_boundary_pos_x and split_inference_boundary_pos_y. subpic_split_inference_flag equal to 0 indicates the absence of split_inference_boundary_pos_x and split_inference_boundary_pos_y.

split_inference_boundary_pos_x is used to compute the value of SplitInferenceBoundaryPosX, which specifies the location of the vertical split inference boundary in luma samples. split_inference_boundary_pos_x shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 4 ) − 1, inclusive.

The location of the vertical split inference boundary SplitInferenceBoundaryPosX is derived as follows:

SplitInferenceBoundaryPosX = split_inference_boundary_pos_x * 4

split_inference_boundary_pos_y is used to compute the value of SplitInferenceBoundaryPosY, which specifies the location of the horizontal split inference boundary in luma samples. split_inference_boundary_pos_y shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 4 ) − 1, inclusive.

The location of the horizontal split inference boundary SplitInferenceBoundaryPosY is derived as follows:

SplitInferenceBoundaryPosY = split_inference_boundary_pos_y * 4;

## Coding tree block splitting inference

The inference of the split of the coding tree block is modified to take into account the split inference boundaries. The semantic of split_cu_flag is modified as follows:

split_cu_flag equal to 0 specifies that a coding unit is not split. split_cu_flag equal to 1 specifies that a coding unit is split into four coding blocks using a quad split as indicated by the syntax element split_qt_flag, or into two coding blocks using a binary split or into three coding blocks using a ternary split as indicated by the syntax element mtt_split_cu_binary_flag. The binary or ternary split can be either vertical or horizontal as indicated by the syntax element mtt_split_cu_vertical_flag.

When split_cu_flag is not present, the value of split_cu_flag is inferred as follows:

- If one or more of the following conditions are true, the value of split_cu_flag is inferred to be equal to 1:

- SubPicInferenceSplitFlag equal to 0 and x0 + cbWidth is greater than pic_width_in_luma_samples.

- SubPicInferenceSplitFlag equal to 0 and y0 + cbHeight is greater than pic_height_in_luma_samples.

- SubPicInferenceSplitFlag equal to 1 and x0 + cbWidth is greater than SubPicInferenceBoundaryPosX

- SubPicInferenceSplitFlag equal to 1 and y0 + cbHeight is greater than SubPicInferenceBoundaryPosY

- Otherwise, the value of split_cu_flag is inferred to be equal to 0.

The variable SubPicInferenceSplitFlag is equal to 1 when the subpicture of the slice is crossed by a split inference boundary. The SubPicInferenceBoundaryPosX (resp. SubPicInferenceBoundaryPosY) is the horizontal (resp. vertical) coordinate of the vertical (resp. horizontal) split inference boundary when present.

## Signaling of exclusion bands in the conformance window

We propose to define band of pixels excluded from the conformance window to allow removing the subpicture areas with non conformant pixels represented by hatched area in the figure below. These areas correspond to the pixels between the vertical split inference boundary and the right of a subpicture or to the pixels between the horizontal split inference boundary and the bottom of a subpicture.

Figure 2: Conformance window exclusion bands.

### Option 1: vertical and horizontal exclusion bands

| picture_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| conformance_exclusion_ver_band_flag | u(1) |
| if (conformance_exclusion_ver_band_flag) { |  |
| conformance_exclusion_ver_band_width_minus1 | ue(v) |
| conformance_exclusion_ver_band_pos_x | ue(v) |
| } |  |
| conformance_exclusion_hor_band_flag | u(1) |
| if (conformance_exclusion_hor_band_flag) { |  |
| conformance_exclusion_hor_band_height_minus1 | ue(v) |
| conformance_exclusion_hor_band_pos_y | ue(v) |
| } |  |

conformance_exclusion_ver_band_flag equal 1 indicates the presence of conformance_exclusion_ver_band_width_minus1 and conformance_exclusion_ver_band_pos_x in the PPS, which specify the vertical conformance exclusion band. conformance_exclusion_band_flag equal 0 indicates the absence of conformance_exclusion_ver_band_width_minus1 and conformance_exclusion_ver_band_pos_x in the PPS.

conformance_exclusion_ver_band_width_minus1 plus 1 specifies the width of the vertical conformance exclusion band in luma samples and is used to compute PpsConformanceVerBandPosX. PpsConformanceVerBandPosX specifies the location of the left boundary  of the vertical conformance exclusion band. conformance_exclusion_ver_band_width_minus1 shall be in the range 0..CtbSizeY − 2.

conformance_exclusion_ver_band_pos_x is used to compute the value of PpsConformanceVerBandPosX. conformance_exclusion_ver_band_pos_x  shall be in the range of 1.. PicWidthInCtbsY-1, inclusive.

The location of left boundary of the vertical exclusion band PpsConformanceVerBandPosX is derived as follows:

PpsConformanceVerBandPosX = conformance_exclusion_ver_band_pos_x* CtbSizeY – (conformance_exclusion_ver_band_width_minus1 + 1)

conformance_exclusion_hor_band_flag equal 1 indicates the presence of conformance_exclusion_hor_band_height_minus1 and conformance_exclusion_hor_band_pos_y in the PPS, whichorizontal conformance exclusion band. conformance_exclusion_band_flag equal 0 indicates the absence of conformance_exclusion_hor_band_height_minus1 and conformance_exclusion_hor_band_pos_y in the PPS.

conformance_exclusion_hor_band_height_minus1 plus 1 specifies the height of the horizontal conformance exclusion band in luma samples and is used to compute PpsConformanceHorBandPosY. PpsConformanceHorBandPosY specifies the location of the top boundary of the horizontal conformance exclusion band. conformance_exclusion_hor_band_height_minus1 shall be in the range 0..CtbSizeY − 2.

conformance_exclusion_hor_band_pos_y is used to compute the value of PpsConformanceHorBandPosY. conformance_exclusion_hor_band_pos_y  shall be in the range of 1.. PicHeightInCtbsY-1, inclusive.

The location of the top boundary of the horizontal exclusion band PpsConformanceHorBandPosY is derived as follows:

PpsConformanceHorBandPosY = conformance_exclusion_hor_band_pos_y * CtbSizeY – (conformance_exclusion_hor_band_height_minus1 + 1)

### Option 2: Multiple vertical and horizontal exclusion bands

| picture_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| conformance_exclusion_band_flag | u(1) |
| if (conformance_exclusion_band_flag) { |  |
| num_ver_conformance_exclusion_bands | ue(v) |
| for( i = 0; i < pps_num_ver_conformance_exclusion_bands; i++ ) { |  |
| conformance_exclusion_ver_band_width_minus1 [ i ] | ue(v) |
| conformance_exclusion_ver_band_pos_x[ i ] | ue(v) |
| } |  |
| num_hor_conformance_exclusion_bands | ue(v) |
| for( i = 0; i < pps_num_hor_conformance_exclusion_bands; i++ ) |  |
| conformance_exclusion_hor_band_height_minus1 [ i ] | ue(v) |
| conformance_exclusion_hor_band_pos_y[ i ] | ue(v) |
| } |  |

With the following semantics:

conformance_exclusion_band_flag equal 1 indicates the presence of conformance exclusion bands in the PPS. conformance_exclusion_band_flag equal 0 indicates the absence of conformance exclusion bands in the PPS.

num_ver_conformance_exclusion_bands specifies the number of conformance_exclusion_ver_band_pos_x[ i ] and conformance_exclusion_ver_band_width[ i ] syntax elements that are present in the PPS. When num_ver_conformance_exclusion_bands is not present, it is inferred to be equal to 0.

conformance_exclusion_ver_band_width_minus1[ i ] specifies the width of the i-th vertical conformance exclusion band in luma samples and is used to compute PpsConformanceVerBandPosX[ i ] , which specifies the location of the i-th vertical conformance exclusion band boundary. conformance_exclusion_ver_band_width_minus1[ i ] shall be in the range 0..CtbSizeY − 2.

conformance_exclusion_ver_band_pos_x[ i ] is used to compute the value of PpsConformanceVerBandPosX[ i ], which specifies the location of the i-th vertical conformance exclusion band. conformance_exclusion_ver_band_pos_x[ i ]  shall be in the range of 1.. PicWidthInCtbsY-1, inclusive.

The location of the i-th vertical exclusion band PpsConformanceVerBandPosX [ i ] is derived as follows:

PpsConformanceVerBandPosX [ i ] = conformance_exclusion_ver_band_pos_x [ i ] * CtbSizeY – (conformance_exclusion_ver_band_width_minus1[ i ] + 1)

The distance between any two vertical conformance exclusion band boundaries shall be greater than or equal to CtbSizeY in luma samples.

num_hor_conformance_exclusion_bands specifies the number of conformance_exclusion_hor_band_pos_y[ i ] and conformance_exclusion_hor_band_height[ i ] syntax elements that are present in the PPS. When num_hor_conformance_exclusion_bands is not present, it is inferred to be equal to 0.

cconformance_exclusion_hor_band_height_minus1[ i ] specifies the height of the i-th horizontal conformance exclusion band in luma samples and is used to compute PpsConformanceHorBandPosY[ i ] , which specifies the location of the i-th horizontal conformance exclusion band boundary. conformance_exclusion_hor_band_height_minus1[ i ] shall be in the range 0..CtbSizeY − 2.

conformance_exclusion_hor_band_pos_y[ i ] is used to compute the value of PpsConformanceHorBandPosY[ i ], which specifies the location of the i-th horizontal conformance exclusion band boundary in units of luma samples. conformance_exclusion_hor_band_pos_y [ i ] shall be in the range of 1.. PicHeightInCtbsY-1, inclusive.

The location of the i-th horizontal exclusion band PpsConformanceHorBandPosY [ i ] is derived as follows:

PpsConformanceHorBandPosY [ i ] = conformance_exclusion_hor_band_pos_y [ i ] * CtbSizeY – (conformance_exclusion_hor_band_height_minus1[ i ] + 1)

The distance between any two horizontal conformance exclusion band boundaries shall be greater than or equal to CtbSizeY luma samples which is the size of the CTB in luma samples.

# Conclusion

To sum up, we think that current CTB split inference process limits the usage of subpictures (even for subpicture with boundaries treated as picture boundaries) for bitstream merging. The proposed syntax elements allow reducing these limitations.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).